STANDARD MICROSYSTEMS CORPORATION

Packet Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1994-1995 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  Basic packet driver use.

o  Software interrupts.

o  How to use the packet driver to connect to a NetWare 2.2 server

using the BYU packet driver shell.

o  How to use the packet driver to connect to a NetWare server

using the Intel packet driver shell.

o  Using the packet driver with SMC/WD adapters.

Note: This driver supports all 8416, 8216 (Ultra), 80xx (Elite),

80xx (PLUS), and 8416 (EtherEZ) Series ISA, EISA, and Micro

Channel Ethernet adapters.

BASIC PACKET DRIVER USE

The following section describes basic packet driver use. Since

different networks have different requirements, you can include the

following switches in your command lines as needed.

-d    This switch delays the adapter's initialization until the first

time the packet driver is accessed. Use this switch if your

workstation loads the operating system over the network using a

Boot ROM on the SMC adapter.

-n    This switch configures the driver to change 8137-formatted

packets seen by the client into IEEE802.3 packets.  Unless you

use the BYU's NetWare packet driver shell, you can omit this

switch. If you want to use 8137 packets, use "econfig" to

configure Intel's PDIPX103 packet driver shell.

-p    This switch disables promiscuous mode. A small level of security

can be achieved by disabling promiscuous mode; however,

disabling promiscuous mode should not be mistaken for real

security.

-w    This switch is used with Microsoft Windows for nonresident DOS

TCP stacks, such as NCSA Telnet and PC-Gopher. Alternatively,

you can use the Windows public domain utility, winpkt.

-i    The SMC packet driver can be used with both Ethernet_II and

IEEE 802.3 frame types. The packet driver specification, however,

only allows a driver to report one class which, by default, is

Ethernet_II. If you use the packet driver with client software

that expects to find an IEEE 802.3 packet driver instead of an

Ethernet_II packet driver, use this switch to configure the

driver to use IEEE 802.3-formatted packets.

SOFTWARE INTERRUPTS

A packet driver provides services for client programs. These programs

access the packet driver by issuing software interrupts.

Any software interrupt between the hexadecimal addresses of 0x60

through 0x80 can be used with the packet driver, with the following

exceptions:

o  0x67 is reserved for the EMS specification.

o  0x70 through 0x77 are reserved for hardware IRQ lines 8 through 15.

After you choose a software interrupt, assign that interrupt to the

packet driver, and install the packet driver. The client programs

will then automatically locate the packet driver by searching for

the packet driver's signature at software interrupts from 0x60

through 0x80.

CONNECTING TO A NetWare 2.2 SERVER USING THE BYU PACKET DRIVER SHELL

If you want to connect to a NetWare 2.2 server that, by default, is

configured to use IEEE 802.3-type packets, use the following command

line examples.

PKT8000 -n 0x7e

BYUIPX

NETX

In this command line:

o  PKT8000 denotes the SMC/Western Digital adapter.

o  -n configures the driver to change 8137-formatted packets seen by

the client into IEEE802.3 packets.

o  Software interrupt setting 0x7e is the hexadecimal address 7E,

which allows packet driver clients to use interrupt 7E to

communicate with the packet driver.

CONNECTING TO A NetWare SERVER USING THE INTEL PACKET DRIVER SHELL

If you want to connect to a NetWare server using the Intel packet

driver shell, pdipx103, use the following command line:

PKT8000 0x7e

PDIPX

NETX

In this line:

o  PKT8000 denotes the SMC/Western Digital adapter.

o  0x7e is the hexadecimal address 7E, which allows packet driver

clients to use interrupt 7E to communicate with the packet driver.

USING THE PACKET DRIVER WITH WD JUMPER-CONFIGURED ADAPTERS

You can use the packet driver with Western Digital (WD) adapters. If

you use the packet driver with jumper-configured WD adapters, you

must include the following adapter information in a batch file or on

a command line:

o  Software interrupt setting

o  Input/output (I/O) address

o  Interrupt (IRQ) setting

o  RAM address

If you are using a jumper-configured adapter and want to connect to

a NetWare 2.2 server using the BYU packet driver shell, refer to the

next section. If you are using a jumper-configured adapter and the

Intel PDIPX driver and want to connect to a NetWare 3.11 server,

refer to "Using Older Adapters with the Intel Packet Driver NetWare

Shell."

Using SMC/WD Adapters with NetWare 2.2

and the BYU Packet Driver Shell

The following command line can be used to connect SMC and Western

Digital adapters to a NetWare 2.2 server when the server has not

been configured using "econfig" and you are using the BYU packet

driver shell.

PKT8000 -n <0xSoftware Interrupt> <0xI/O> <IRQ> <0xRAM Address>

BYUIPX

NETX

In this command line:

o  PKT8000 denotes the SMC/Western Digital adapter.

o  -n configures the driver to use 8137-formatted packets. This

switch is necessary to connect to a Novell NetWare 2.2 server.

However, if you are using Ethernet II frame types, you must

omit this switch.

o  <0xSoftware Interrupt> is the adapter's software interrupt setting,

and is prefaced by "0x" to indicate a hexadecimal number.

o  <0xI/O> is the I/O base address of the adapter, and is prefaced

by "0x" to indicate a hexadecimal number.

o  <IRQ> is the adapter's interrupt setting.

o  <0xRAM Address> is the RAM base address of the adapter, and is

prefaced by "0x" to indicate a hexadecimal number.

A typical command line might resemble:

PKT8000 -n 0x7e  0x280 10 0xCC00

BYUIPX

NETX

Using SMC/WD Adapters with the Intel Packet Driver NetWare Shell

The following command line can be used to connect SMC and Western

Digital adapters to a NetWare NetWare 3.11, 3.12, 4.0, 4.01, or

4.10 server:

PKT8000 -i <0xSoftware Interrupt> <0xI/O> <IRQ> <0xRAM Address>

PDIPX

NETX

In this line:

In this command line:

o  PKT8000 denotes the denotes the SMC/Western Digital adapter.

o  -i configures the driver to use IEEE 802.3-formatted packets,

which the PDIPX protocol expects to receive.

o  <0xSoftware Interrupt> is the adapter's software interrupt setting,

and is prefaced by "0x" to indicate a hexadecimal number.

o  <0xI/O> is the adapter's I/O base address, and is prefaced

by "0x" to indicate a hexadecimal number.

o  <IRQ> is the adapter's interrupt setting.

o  <0xRAM Address> is the RAM base address of the adapter, and is

prefaced by "0x" to indicate a hexadecimal number.

A typical command line might resemble:

PKT8000 -i 0x7e 0x280 10 0xCC00

PDIPX

NETX

Note: If you are using Ethernet II frame types, you must use

"econfig" to configure the PDIPX driver before loading it.

"econfig" is a step you use to configure a shell. For more

information, refer to your Novell NetWare documentation.

This commercial computer software and documentation is provided with

RESTRICTED RIGHTS.  Use, duplication, or disclosure by the

Government is subject to restrictions set forth in subparagraph

(c) (1) (ii) of the Rights in Technical Data and Computer Software

clause at DFARS 252.227-7013, or subparagraphs (c) (1) and (2) of the

Commercial Computer Software -- Restricted Rights clause at

FAR 52.227-19, or Alternate III of the Rights in Data -- General

clause at FAR 52.227-14, as applicable.  The manufacturer /

subcontractor is Standard Microsystems Corporation,   80 Arkay Drive,

Hauppauge,  NY    11788.

SMC and Standard Microsystems are registered trademarks; and EtherEZ,

EtherCard Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite

16C Ultra, EliteSeries, EZStart, and SuperDisk are trademarks of

Standard Microsystems Corporation.  Other product and company names

are registered trademarks or trademarks of their respective holders.