short explanation of ZuTSR's features and installation

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

1. ZuTSR is not free. It's shareware.

This means, you are free to use it for a trial period of 21 days.

After this time, you have to stop using it or to send

the equivalent of 20.- DM to the author.

If you are able to get your currency converted into DM, it would

be greatly appreciated.

You can send it via postal money order to

Kolja Waschk,

Oevelgoenne 23,

W-2000 Hamburg 52, Germany (up to the last day of june'93)

22605 Hamburg (after 1st july'93)

For other questions regarding ZuTSR and ZUtil, you can

reach me on FidoNet : 2:241/2101.27 or, as soon as it

appears in the nodelist, on 2:241/2114.

I can also receive UseNet e-mail :

koljawk@zerebrm.isnet.sub.org

Or contact me by phone : 49-40-8804056

(sorry for my english in advance...)

Please keep your receipt, it's the proof you paid the money.

If you send cash, please include your full international

address, I will return a notice to you immediately.

Paying the 20.-DM allows you to use this program and all other

versions which differ only in the last digit of the version

number (e.g. 1.00, 1.01, 1.02, 1.03...).

Every increased digit means at least one more available feature...

I can't guarantee that ZuTSR will work on your PC. So, test it

out before you send the money.

I've got it working here with a ZyXEL U1496E+, Rom Rev RM5.05EP,

a 286/16 PC and FrontDoor 2.02nc.

2. How ZuTSR works

Hope this makes you understand how the things will happen :

Modem sends RING

to the Computer   -> FrontDoor issues 'Force Answer Command'.

Instead of using "ATA|" as the answer command, you use "ZUTSR|".

This will call ZuTSR from memory.

(ZuTSR is loaded as a 'Fossil driver add-on' and watches all

Characters which are sent to the Modem via the Fossil driver)

ZuTSR takes the call, plays the first message, and an optional

beep. After that, it switches the modem into recording mode,

and tries to detect if the caller says something or keeps quiet.

If the caller keeps quiet, the call is assumed to be a data

call. ZuTSR issues "ATA|" to the modem to start the connection

handshake. ZuTSR exits after this, so messages from the modem

like 'CONNECT FAX' or 'CONNECT 14400' are sent to FrontDoor

directly. FrontDoor does all the rest to answer the call.

If the caller said something, a second message will be played

to inform that his call is recognized as a voice call and that

he has to wait a few more seconds.

Then, two possibilities : If you own a PBX system, you could

let ZuTSR send a command to the ZyXEL to transfer the call to

your voice phone. Again, ZuTSR exits and FrontDoor has the

control over everything again (except the call)

If you don't own a PBX system, you could let ZuTSR simulate

the press of a function key and exit. FrontDoor will recognize

this 'function request' and execute an external program or

exit with an errorlevel, defined in Mailer>Function Keys.

As an external program, you could use any software you find

that is capable of handling the ZyXEL voice mode and taking

control over a call that is already on the line. (Off-Hook).

For this, I suggest you use ZUtil, another program by the

author. It's also fine for recording of greeting messages

etc., although you might use ZFAX 2.x for this...

3. How to configure ZuTSR

I didn't have the time to translate the ZuTSRCFG program

messages into english. So I include the german screen output

and the english translation here. It's not soo much to

become %/&@ about it... In later releases, I may include

a translated version.

Execute ZuTSRCFG by simply typing ZuTSRCFG at the command line

prompt. The file ZuTSR.COM has to be in the same directory where

ZuTSRCFG.EXE is. As an alternative, you can give the path to

ZuTSR.COM or copies of it as a command line parameter to ZuTSRCFG.

Sample: ZUTSRCFG D:\ETC\ZUTSR.COM

Changes, made with ZuTSRCFG, will only affect the copy of ZuTSR.COM

on your disk. An already loaded ZuTSR will not be affected !

You have to unload ZuTSR and load again to make the changes be

effective (this is done by executing ZuTSR.COM twice)

3.1 Configuration items

A) Modemtyp (type of modem)

You can choose between normal and plus model.

B) Antwortmodus (Answer mode)

- nur Ansage "Hallo", auflegen :

only play the first ('hello') message, then hang up.

- "Hallo", "Ich leite weiter", weiterleiten

play the first message, the second message ('please wait

a few seconds more') and then pass the call to another

phone or an external program.

- Unterscheidung Sprache/Daten

Distinguish between Voice and Data, as described in section 2

- Unterscheidung Sprache/Daten/Fax

Distinguish between Voice, Data and Fax, as described in section 2

The difference to 'distinguish between voice and data' is,

that the modem is set into data mode FCLASS=6, not FCLASS=0.

C) Sprachdatei "Hallo"

The path to the ZyXEL voice file, containing the first message.

Your first message should make the caller saying something.

A short "Hellooh ?" works fine in Germany...

D) "Ich leite weiter"

The path to the second message file. Some words about

'transferring your call... please wait'

E) Piepton nach "Hallo"

Beep tone after first message On (An) / Off (Aus)

F) Frequenz 1:

the main frequency of the beep tone.

G) Frequenz 2:

a second frequency to be generated.

H) Dauer:

the duration of the beep tone.

I) Weiterbehandlung des Anrufs ueber :

- Nebenstellenanlage

(PBX) send commands J) and K) to the modem to transfer the

call to another phone.

- Simulation einer Funktionstaste

simulate press of function key.

See also section 2 to learn what I mean.

J) Modembefehl zum Weiterleiten :

Enter the command to make your modem transferring the call

to another phone here. (if you have a PBX with this feature)

If I) is set to "simulate press of function key", this

command is ignored.

K) Modembefehl zum Auflegen danach :

Enter the modem command to hang up the line after transferring

here. Should be something like 'ATZ' or 'AT H0'

L) Verzoegerung zwischen diesen beiden Modemkommandos:

You can specify a delay between J) and K) here.

Useful if your PBX system reacts very slow...

M) Zu simulierende Funktionstaste :

(function key to be simulated)

You can specify the function key which should be simulated

to pass the call to an external program (only needed if

I) is set to 'simulate press of function key')

S) Konfiguration beenden, Aenderungen speichern

end configuration program, save changes in ZUTSR.COM

Q) Konfiguration beenden, Aenderungen nicht speichern

end configuration program, do not save changes

4. How to configure FrontDoor for use with ZuTSR

Set Config>Modem>Answer control>Force answer to "ZUTSR|" instead of "ATA|"

If you want to use the 'simulation of function key' method,

configure Config>Mailer>Function Keys for your needs.

Add "AT+VSD=x,30|" to your init strings in Config>Modem>Command strings>Init

This specifies the silence sensitivity treshold level.

x = 0 means 'never recognize a signal as silent', try up to

x = 31, which means 'recognize only *very* loud signals as _not_ silent'.

The x value is different for each modem, firmware and telco equipment/

location/company, so you have to find out what fits your needs best

by yourself.

5. How to load/unload ZuTSR

ZuTSR acts as an 'add-on' to the Fossil driver.

It _must_ be loaded _after_ the Fossil driver is loaded.

So, if you load your Fossil driver in your CONFIG.SYS,

you should add a line '<path\>ZUTSR' in your AUTOEXEC.BAT.

It takes only about 2k of memory when loaded.

Sample: C:\COMMPROG\ZUTSR

You can unload ZuTSR from memory by simply executing

ZUTSR.COM again. If you want to unload your Fossil driver

also, you have to unload ZuTSR _first_.

You can't load or unload ZuTSR when you are in a different

DOS shell than the one from which you are going to use it.

Loading ZuTSR from within batch files could be risky.

6. Errors

If you encounter bugs in my program, of which you are sure

that they are not bugs in the modem (the ZyXELs behave quite

strange under some conditions), please report.