PRO-CR<tm>  -  PROFESSIONAL OPTICAL CHARACTER RECOGNITION

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

Copyright 1989, David P. Gray, Gray Design Associates

All Rights Reserved

Member, Association of Shareware Professionals

------------------------------[ SPECIFICATION ]------------------------------

*       Reads 8 to 30 point mono and proportional fonts.

*       Up to 200 words per minute.

*       Requires IBM or compatible XT/AT (AT recommended).

*       Requires 80Kb memory (512Kb recommended).

*       Supports HP ScanJet or any scanner that supports TIFF files.

*       Training and font editing supported with EGA or VGA adapter.

*       Real-time viewing of text during normal and training scan.

*       Continuous scanning if auto document feeder attached.

*       Upgrade to Version 2 when available.

---------------------------[ U S E R   G U I D E ]---------------------------

1.      General.

2.      Font Selection.

3.      Non HP ScanJet Users.

4.      Output Text File.

5.      Performance.

6.      Theory of Operation.

7.      Menus, Learn and Edit Mode.

8.      Revision History and Future Releases.

9.      Site Licenses.

10.     Comments to the Author.

11.     Association of Shareware Professionals.

12.     Miscellaneous.

-------------------------------[ 1.  GENERAL ]-------------------------------

PRO-CR<tm> performs Optical Character Recognition on an IBM PC or compatible.

The program will run on an XT or AT, however an AT is strongly recommended

due to the highly cpu-intensive nature of the program.

A graphics adapter is not required for basic character recognition, but is

needed for the training and font-edit functions.  If a graphics adapter is

used, it should be EGA or VGA.  (CGA does not have the required resolution).

The minimum memory requirement is about 80K, although the program adapts

itself to use as much conventional memory as available.  Version 1 does not

support expanded or enhanced memory.  A temporary disk-file is used for any

parts of the scanned image that will not fit into memory at once.

There are 2 methods of supplying input to PRO-CR<tm>.

1.      Direct scanning from an HP ScanJet.

2.      Reading from a TIFF file produced by any other scanner.

Direct scanning allows you to scan a single page if you have a flat-bed

scanner only or optionally scan multiple pages if you have an automatic

document feeder attached.  Version 1 always scans entire pages.

PRO-CR<tm> recognizes both mono-spaced and proportionally spaced fonts.  It

adjusts automatically to character size and will automatically switch fonts

when more than one is selected.

PRO-CR<tm> is trainable.  A learning mode is provided to learn unrecognized

shapes or new fonts.

FILES DISTRIBUTED:

OCR.EXE                 -       The PRO-CR<tm> program

README.DOC              -       Important information

HELP1.DOC               -       Text file used for online help

HELP2.DOC               -       Text file used for online help

HELP3.DOC               -       Text file used for online help

MANUAL.DOC              -       This file

COURIER.OCR             -       Font file

ROMAN.OCR               -       Font file

HELV.OCR                -       Font file

IMAGE.TIF               -       Example TIFF file for processing

NOTE: The text in the IMAGE.TIF file is in Courier.

----------------------------[ 2.  FONT SELECTION ]---------------------------

PRO-CR<tm> provides a number of standard fonts for selection.  More than one

font may be selected in cases where you are not sure what font is on the page

to be processed, or if there is more than one font on the page.  For cases

where only one font appears on the page to be scanned, selecting this font

will generally give more accurate results and faster times than selecting all

the fonts.  However, the penalty of selecting all fonts is not great and is

probably the best thing to do if you are in any doubt.

If you are not sure what a particular font looks like, use the font editing

feature to see the shapes of the default supplied fonts.  (See the chapter on

the edit mode).

-------------------------[ 3.  NON HP SCANJET USERS ]------------------------

Compatibility with non-HP scanners is made possible through the use of TIFF

(tag image file format) files.  Many scanners and desktop publishing programs

use this standard file format.   A resolution of 300 dots per inch gives a

good compromise between accuracy and processing time.  If the text you are

scanning is large, over 12 points, you may wish to scan at a lower resolution,

say 240 or 200 dpi to speed processing in PRO-CR<tm>. In general, though, the

higher the resolution the better the accuracy.

When reading from a TIFF file, PRO-CR<tm> looks for the file IMAGE.TIF

Even if you do have an HP ScanJet you can still use it for cases when you do

not wish to scan the whole page.  Use the scanning program that came with the

scanner to scan the part of the page containing the text you wish to process.

Version 1 of PRO-CR<tm> does not read compressed TIFF files.

---------------------------[ 4.  OUTPUT TEXT FILE ]--------------------------

Whether scanning direct or reading from the TIFF file, all processed output

is directed to a plain ASCII text file, default name TEXT.SAV.  Version 1

does not support word processor attributes or file formats.  You may change

the the name of the output file in the 'run' menu.  Text is always appended

to the file for each page scanned until you choose a new file name.

-----------------------------[ 5.  PERFORMANCE ]-----------------------------

5.1.  Font size

---------------

PRO-CR<tm> automatically adjusts itself to a range of point sizes within any

document.  The range is approximately 8 to 30 points.  The low end depends

on the quality of the document and the typeface used.  These figures assume

the image was scanned at 300 dots per inch.  (The resolution used for the

direct scanning mode).

Learning mode allows a total of 12 fonts, with up to 90 shapes in each font

and up to 3 fonts selectable simultaneously in learning or non-learning mode.

5.2.  Processing speed

----------------------

With one font selected, PRO-CR<tm> will process text at approximately 200

words per minute on a 20MHz 386 PC.

5.3.  Error rate

----------------

The error rate is dependent on the quality of the text being processed and

on the number of characters that 'run together'.  In general the mono-spaced

fonts such as Courier are easiest and the Roman font is the hardest to

accurately recognize.  For cases where characters run together, the learning

mode can be used to help recognition.

With good quality type the error rate is approximately 95% to 99% for Courier

and 90% to 95% for Roman and Helvetica.

-------------------------[ 6.  THEORY OF OPERATION ]-------------------------

(Non technically-minded people please skip)

6.1.  PRO-CR<tm> Uses Feature Extraction

----------------------------------------

With one proprietry global (topological) feature and two local features.  The

local features are optimized for the three supplied fonts.  With all three

fonts selected, good recognition is achieved on other non-stylized fonts via

this combination of features.  PRO-CR<tm> also includes a large number of

ad-hoc positional and context sensitive tests.

6.2.  Single character errors, why they occur

---------------------------------------------

PRO-CR<tm> will not correctly recognize single characters 100% of the time.

For every character guessed wrong, the reason is usually to be found on the

document.  Broken characters, skewed lines, misplaced text, smudges, to name

a few.  Sometimes it is just 'bad luck' (for the technically minded, every

signal processing system involves some noise.  In this case the noise is in

the scanning conversion and is a function of the scan resolution.  Some

characters look very much alike  (S vs 5, b vs h) and one pixel dropped from

the wrong place and appearing in another place can cause mis-recognition.)

6.3.  Run-ons

-------------

One of the biggest problems faced by Optical Character Recognizers are run-

ons.  The ultimate 'run-on' is human hand writing in which all the characters

are joined together.  This kind of recognition is beyond the scope of most

PC-based OCRs (including this one, except as follows:).

PRO-CR<tm> recognizes mono-spaced fonts such as Courier and proportional fonts

such as Helvetica and Roman.  In tightly spaced proportional fonts many of the

characters run into each other.  (This can also happen in badly spaced mono-

fonts.)  The software only recognizes single objects and so gets very confused

by characters joined together in this way.  It attempts to 'split up' such

run-ons in an attempt to recognize them as two characters but will many times

still fail.  For cases where there are three characters it is almost certain

to fail.  The run-ons are rather dependant on the particular printer which

printed the page and for this reason a 'learning' mode is provided.  This

allows for learning unique shapes applicable to a particular document.  It also

provides a mechanism to learn a completely new font.  Bear in mind that when

learning a new font, best results are obtained with good clean type, 10 points

or larger.  Don't try working with any kind of script font where all

characters flow together, you won't get very far !

-------------------[ 7.  MENUS, LEARN MODE AND EDIT MODE ]-------------------

The operator interface is implemented as a series of menu levels.  Completion

of one takes you to the next, selecting 'Quit' takes you to the previous (or

back to DOS if at the first menu).  The following is a list of menus and

selections.

MENU 1.  (Select mode:)

Select 'Scan_mode' if you will be scanning direct or

Select 'File_mode' if you will be reading from your image.tif file.

MENU 2.  (Select font(s):)

Select one or more fonts to be used when performing the ocr.  The

selected fonts are indicated by a check mark.  Select the 'OK' option

to get to the run menu, menu 3.

Menus 2a and 2b are for use when learning or editing a font.  You can

skip these menus during normal use and select 'OK' to proceed to menu 3.

MENU 2a. (Select font for learning:)

Select the 'Learn' option to select a font for learning or to add a brand

new font for learning.  Only one font can be learnt and is indicated by

an 'L' instead of a check mark.  During the ocr you will be prompted for

up to 3 characters for any unrecognized characters.  If you are not sure

what the unrecognized text is, press return.  It will be ignored.

Some points to note about learning mode:

None of the 3 fonts include 'run-ons' or, in other words, combinations of

characters which are joined together.  The reason for this is that the

shapes of the joined together characters are largely printer dependent

and so, what might work well for one document, would not work for another.

In addition, the more shapes that are added to the font library, the more

chance there is of choosing the wrong shape.

There are 2 uses for learning (training) mode:

1.      When there is a large amount of scanning to be done, for example

a book, and it is worthwhile creating a special font just for this one

document.  Do not try to learn to the 3 fonts supplied, they are write

protected.  Instead, add a new font and learn to this.

2.      Another use for the learning mode is to learn a new font from

scratch.  In this case best results will be obtained if you supply the

font in the form of an alphabet, characters spaced well apart and in a

large point size, say 14 or more.  If necessary you can learn a completely

new font just from the final copy to be processed but will not give the

best results.  The program will prompt less and less as it proceeds to

learn the alphabet.  It will often prompt for a character more than once.

This is an indication of the variability of the characters scanned.

Hints for learning:

The learning mode uses any characters you give it to try to match new

characters.  In this way it should prompt you less and less as it learns

the complete alphabet, eventually prompting only for joined or broken

characters.  However, you will find that on occasion it will prompt you

for characters you have already entered.  This is due to the fact that

there is a recognition threshold set which is a compromise between

recognizing a character that has not been learnt yet and prompting too

often for characters already learnt.

Use the edit mode to consolidate your learnt font, removing unwanted

duplicate characters or runs and correcting any mistakes made when enter-

ing the string representation for the character shape.  Do not try to

enter a string for shapes that you do not recognize yourself, just hit

return to skip to the next character during a learning session.  Do not

enter punctuation marks especially, these are handled with special

algorithms.  Some characters, such as o, u, v, x etc. are ambiguous with

regard to case when viewed out of context.  If you are unsure as to the

case of a shape that the program prompts you with, either skip the

character by entering return or simply enter the lower case version.  The

program has special algorithms for adjusting the case of such ambiguous

characters.

After a learning session, always run the program in non-learn mode using

the new font to determine the results.

You may use one font, for example one of the supplied fonts, while

learning to a new font.

MENU 2b. (Select font to edit:)

Select the 'Edit' option to select a font for editing.  Editing consists

of deleting unwanted shapes from the font or changing the text which they

represent.  Follow the directions on the edit screen.  Note that the

default fonts supplied with PRO-CR<tm> are write protected so any attempt

to learn to them or edit them will fail.

Menu 3.  (run)

Select 'File name' if you wish to change the name of the file which will

be written to with the processed text.  The default file is 'text.sav'.

This is a plain ASCII text file which may be imported into any word-

processor or desktop publishing.  If more than one scan is done, new

information is appended to this file until you select a new file name.

Select 'Start' for a single page scan or

Select 'Auto feed' for a multi page scan.  This is only available when

scanning directly, file mode will process everything in the file.  Also,

an automatic document feeder must be present and ready for use.

-----------------[ 8.  REVISION HISTORY AND FUTURE RELEASES ]----------------

Revision History

----------------

1.0     5/16/89:        Baseline version.

1.01    5/18/89:        Fixed character editing in font edit function, caused

by bug in compiler's loop optimizer.

1.02    5/31/89:        Don't reject TIFFs with no bits_per_sample tag.

Assume a value of 1.

1.03    6/19/89:        Don't reject TIFFs with no resolution tags.

1.04    8/28/89:        Fixed bug in learn-mode.

Future Versions

---------------

Version 2 is currently in progress.  Estimated shipping date is last quarter

of 1989.

The following is a list of features expected to be included:

*       Enhanced speed and recognition rate.

*       Font independance (just hit the start button).

*       Mouse support.  Selection of areas to be scanned.

*       Mixed text and graphics blocks for desktop publishers.

*       Direct support of Logitech hand scanner.

*       Ability to handle compressed TIFFs, PCX and MSP formats.

----------------------------[ 9.  SITE LICENSES ]----------------------------

COMPANIES please note that only ONE USER at ONE LOCATION may use and operate

TOUCH TYPE TUTOR.

Additional computers, users and locations should be registered separately,

by volume, or by obtaining a site license.

DISCOUNT RATES are offered to companies registering for a site license or by

volume.   Please write to Gray Design Associates, P.O. Box 333, Northboro,

MA 01532 for a rate schedule.

-----------------------[ 10.  COMMENTS TO THE AUTHOR ]-----------------------

Any feedback would be greatly appreciated.  Please direct any comments to

author personally via CompuServe (73607,2105) or via mail to David P. Gray,

Gray Design Associates, P.O. Box 333, Northboro, MA 01532.

---------------[ 11.  ASSOCIATION OF SHAREWARE PROFESSIONALS ]---------------

This software is produced by David P. Gray who is a member of the Association

of Shareware Professionals (ASP).  ASP wants to make sure that the shareware

principle works for you.   If you are unable to resolve a shareware-related

problem with an ASP member by contacting the member directly, ASP may be able

to help.

The ASP Ombudsman can help you resolve a dispute or problem with an ASP member,

but does not provide technical support for members' products.  Please write to

the ASP Ombudsman at P.O. Box 5786, Bellevue, WA 98006 or send a CompuServe

message via easyplex to ASP Ombudsman 70007,3536.

---------------------------[ 12.  MISCELLANEOUS ]----------------------------

HP and ScanJet are registered trade marks of Hewlett Packard.

----------------------------[ END OF MANUAL.DOC ]----------------------------