______________________________________________________________________________

DELP MENU

DELP is an algorithm for obtaining a  very  fast  fit  of  a  sum  of

exponentials.  It  works on the principle that a sum of exponentials is

a solution of a linear homogeneous differential equation with  constant

coefficients.  The  integral form of the equation is used.  "Int.level"

is the degree of the integral equation,  and so is equal to the  number

of exponentials being fitted.

On running the fitting procedure,  the first thing you will notice is

a red line going more or less through the data.  It is  the  polynomial

representation  of  the  data.  Do  not  worry if it does not appear to

follow the fine features too well,  the important thing is that  it  is

the  "best  least-squares" fit of the Legendre polynomial series to the

data.

The next  few  events  happen  nearly  simultaneously:  the  data  is

overlaid  with  the exponential fit (in deep blue),  the results of the

fit appear in a rectangular window, and the residual (in blue) with the

integrated residual appear (suitably scaled) in a further window.

A good fit  can  be  identified  by  the  following:  the  blue  line

overlying the data is close to the original data, the residual is flat,

the  integrated residual is evenly distributed about zero,  and the sum

of square deviations is small.

The window used to display the residual is  the  last  window  to  be

defined  by  Graph-H window and V window.  For example if "H window" is

set to 2 and "V window" is set to 3,  then the last window is window 6.

This is where the residual will appear.

DELP-Int.Level- The number of exponentials to be fitted.  The maximum

here is 8, minimum 1.

DELP-Poly.degree-  As  an  intermediate  step,   the  data  is  first

represented  by a finite Legendre polynomial series.  This has the twin

benefits of reducing  the  data  set  to  a  manageable  size,  and  of

filtering  the  data  in  a  rational  manner.  The maximum is 50,  the

minimum is 2x(Int.level+1).

DELP-Run fit- Start the DELP fitting procedure.

DELP-Baseline-  The  baseline   may   be   calculated   automatically

("Baseline  AUTO"),  or  an  estimate may be made in advance ("Baseline

SET").  Use this function to toggle between the two.  What comes out of

the "least squares" fit of an integral equation to the  data,  are  the

exponential  rate  constants (inverse time constants).  If the baseline

is unknown,  then one coefficient of the polynomial series (that  might

have  been  used  to calculate the rate constants) has to be discarded.

If the baseline is known,  then a more accurate estimate  of  the  rate

constants may be made.

On  selecting  "Baseline  SET",  a  line  and  a box should appear on

screen.  If they do not,  then press "Z"  to  zero  the  baseline.  The

horizontal cursor may be moved up and down using the arrow keys,  or by

typing in a number.  Note that the steps widen with frequently repeated

key strokes.  Press <Esc> when you have positioned the cursor  to  your

satisfaction.

Note  that  the exponential amplitudes and offset are calculated as a

second  and separate procedure,  so if the calculated offset differs by

more than 1 % from your estimate,  then this suggests that the original

estimate was incorrect.

DELP-Split run- There is the option of running the fit simultaneously

on  two separate sections of data.  This is useful for example if there

is a glitch in the middle of an otherwise good trace.  It is  advisable

to avoid this function where possible.

DELP-Append- Write the results of the last fit as an ASCII string, to

the end of the ASCII output file.

DELP-Overwrite-  Replace  the  contents  of  the output file with the

results of the last fit.

DELP-Delp file name- The name of the text file to which  the  results

of the last fit will be written.

DELP-Clear- Clear the screen.

DELP-Skip points- There is the option to decimate the data.  This may

be  useful  if  you  have  32,000 data points to be fitted (the maximum

allowed),  and you have only a 8086 processor running at  4  MHz!  Note

that  entering 10 here means that only every tenth point is used in the

fit, so if there is a large component of random noise,  then filter the

data first.

DELP-Mode for output- There are a number of  ways  of  recording  the

results of the DELP fit in an ASCII  file.  The  exponential  rates  or

time   constants   may   be  written,   or  the  differential  equation

coefficients.  Also further info about each individual fit, such as the

filename,  sweep number,  baseline estimate etc may be  recorded.  This

also  affects the form of the number appearing on screen:  whether they

are rate constants, time constants or Q matrix roots.

DELP-Exponent.(ial) weighting.  This option may be toggled On or Off.

When it is on, the section of trace selected for fitting is weighted by

the function exp(Lt),  where L is the Weight rate selected  below.  The

advantage  of  this  is  that  some  data  may  contain  widely  spaced

exponentials with a lot  of  noise,  so  that  the  faster  exponential

contributes  little power to the total signal.  By selecting a narrower

fitting window,  this problem may be overcome.  An altern-ative  is  to

weight   the  points  containing  the  faster  exponential.   The  most

appropriate way to do this within DELP is to multiply the whole section

of data by a single (usually decaying) exponential.  The  effect  on  a

signal containing two exponentials with rates L1 and L2 is to produce a

signal  with  exponential  rates  L1+LW  and  L2+LW,  and if there is a

constant offset (or Baseline is set to AUTO) there is an additional  LW

exponential component.  If the Baseline Set option is being used, there

is no problem, LW is automatically subtracted from L1+LW and L2+LW.  If

Baseline Auto option is used,  then the matrix being reduced to find L1

and  L2 has to be deflated by the known root (LW) before solving for L1

and L2.  It is strongly recommended that the  Baseline  SET  option  is

used when trying exponential weighting.

This option is particularly useful for fitting noisy data with widely

spaced exponential components (>>10), with relatively little amplitude

in the faster component.

DELP-Weight  rate:  this  is used in conjunction with the Exponential

weighting option described above.  An exponential  weight  rate  of  -2

means that the section of data to be fitted is multiplied by a function

that  decays  by  2  e's  worth over the selected range between the two

cursors.

DELP-Fix rates- When this is toggled to On, you are first prompted to

change each of the proffered rate constants from the last fit (if any).

Further  fits  will  be  performed,  minimizing  with  respect  to  the

exponential amplitudes only.

DELP-Limit plot of fit: By default, the exponential fit is calculated

for the entire portion of the trace appearing on screen.  If "Limit .."

is On,  the fit is calculated (and displayed and plotted) only over the

region  defined  by  StartFit  and  EndFit  (set by cursors from Graph-

Display, or from Graph-Set cursors-1: StartFit1 etc).

_____________________________________________________________________________

PATTERN SEARCH (PtnSch)

This  is  an  iterative  (read  VERY SLOW) procedure to find a set of

coefficients that in this application finds a pattern that leads  to  a

reduction  in  the  "sum  of squares",  and repeats it or changes it to

improve the rate of convergence.

This fitting procedure is not recommended  for  fitting  exponentials

for  a number of reasons.  Quite apart from the fact that it frequently

converges to false minima  close  to  the  true  minimum,  it  is  time

consuming to set up and painfully slow compared to DELP.

Its advantages are that any function (in general,  although not here)

may be fitted,  and any variable may be  selectively  tweaked  or  held

constant.

1) Select the required equation, and make a note of the number of

variables.

2) Go to the Parameter menu.

3) Enter the number of variables.

4) Set the "maximum number of iterations to 100"

5) Set "skip points" to the maximum indicated

6)  Enter  your first guess at the parameters in the left column only.

The other two columns will be updated automatically.

7) Leave the Parameter menu <Esc>.

8) Use the function Keep to preserve the values you have  just  typed.

(You can restore them later with Reset values.)

9) Use the function Search to begin the search for  a  solution.  This

is  just  a  trial run to see if your estimates are anywhere near.  The

screen is only updated every tenth time that an improved fit  is  found

(hence the maximum iterations needs to be quite high).

10)  If the fit is converging,  then go back to the Parameter menu and

increase the number of iterations, and decrease the number of points to

skip.

For the meaning of Append,  Overwite and Clear,  see DELP MENU

_____________________________________________________________________________

TRANSFORM MENU

Transform-Re-Scale-  A  menu  for  adjusting  such  parameters as the

digitization frequency,  scaling and number of points per  sweep.  This

is useful if your data does not have a recognizable header.

Transform-Filter- Digital implementation  of  analogue  (causal)  and

non-causal  filter  characteristics is available for filtering a single

sweep or part of a sweep.  Note  that  this  menu  can  be  reached  by

<Alt>+"Q".

Transform-FFT-   Creation   of   power   spectra,    averaging,   and

control/signal subtraction.

Transform-Compensate Rs- This is a routine  solely  for  the  use  of

electrophysiologists  who do not have adequate single electrode voltage

clamp.  It is only useful if the  current  you  are  recording  is  not

strongly  voltage  sensitive.  A purely Ohmic correction is made so the

the current becomes what it would have been had there  been  no  series

resistance  voltage error.  The correction is made by assuming an Ohmic

difference between the assumed electrochemical gradient (the difference

between the holding potential and the reversal potential) and the  true

electrochemical gradient when the error voltage across the electrode is

added.

Transform-Histogram- This is not written yet.

Transform-Multiply by:- This function changes  the  absolute  digital

representation of the data. The scaling is also adjusted so that a data

point  at 1V for example will still be displayed as a data point at 1V.

First you are prompted to enter or edit a gain value:  "2" will  double

the apparent size of the data trace, "0.5" will halve it. {t

Transform-Y  power  N-  When this function is selected,  all the data

will be raised to whatever power is specified.  Note  that  if  a  non-

integer  power  is  specified,  then  the data must all be greater than

zero.  This function is useful for example in fitting a "n" independent

subunit  model to data,  so that the data is expected to take the form:

I*(1-exp(L1t))^n * exp(L2t).  Taking the 1/n'th power will reduce  this

to  a  simple  double  exponential with rates L1 and L2/n.  You will be

prompted to enter two numbers.  The first concerns the scaling  of  the

output:  you enter the Y value of the data that will appear in the same

place on the screen after the transformation (if in doubt,  leave  this

at  zero  for  automatic scaling).  The second is the power by which to

raise the data.

TRANSFORM-RE-SCALE MENU

Transform-Re-scale-Full  scale  Y-  This  should be the value of your

measured variable (eg voltage,  current,  temperature etc) at the  full

range  of  a  16-bit ADC.  For example,  suppose you have a 16 bit ADC,

with a +/- 10 V input range,  and  you  want  to  record  the  absolute

voltage,  then you should enter "Full scale Y" as "10".  Many ADC's are

only 12 bit, moreover some devices write to the highest 12 bits of a 16

bit integer (as read by the computer) an example is the CED  1401,  and

others  (AXON)  write to the lowest 12 bits.  The latter case is pretty

obvious,   because  your  data  appears  very  small.   There  are  two

approaches to dealing with data that is in the lower 12 bits: the first

is  to  have  the  "File-12  to  16  bit"  option set to ON so that all

incoming data is multiplied by 2^4,  the second option is to leave  the

data  as  it is and set the "Full scale Y" range to 10 V (for a +/- 10V

ADC input) x 2^4 ie 160V.

Transform- Re-scale- Zero scale Y- This is the value of the  analogue

signal,  when  the  digital  representation  is zero.  Suppose that the

input signal to the ADC was offset by 1 V (ie you added 1 V DC  to  the

input  signal)  then the value you should type for "Zero scale Y" is "-

1".  Alternatively, suppose that your ADC input range is set to zero to

10 V (rather than -10 to +10V),  then "Zero scale Y" will be  "5",  and

"Full  scale  Y" will be "10".  I never have this set to anything other

than zero,  so I cannot guarantee  that  in  all  the  algorithms  deal

correctly with a number other than zero.

Transform- Re-scale- Dig.�freq.- The digitization frequency should be

entered here in Hz.  For example 1 kHz should be entered as "1000".

Transform-  Re-scale-  Points  per  sweep- If your data consists of a

string of repeated traces,  each of the same length,  then  you  should

give  the  length  of  a  single  trace  or sweep as the number of data

points.

Transform- Re-scale- Time zero/ End of trace- "Time zero" is the time

you wish to use to refer to the beginning of a trace. "End of trace" is

the time for the end of the trace.  If you edit either of  these,  then

"Dig.�freq" will be re-adjusted automatically.

Transform-  Re-scale-  Header  size- If your data has a header,  ie a

string of bytes that refer to the digitization frequency, the number of

sweeps etc,  that cannot be read by this program,  and you do not  wish

the header bytes to be displayed as data points,  then you can give the

size of the header here in words (a word is two bytes).  You can  guess

at  the  header size by setting Header size to zero and looking at your

data displayed. The first "N" points will appear odd looking or random.

These first "N" points are your header.  Use  a  cursor  to  count  the

number of points, and set Header size to N.

Transform- Re-scale- Y�scale�label- The label out on the  scale  bar,

is  either  the  label  read from the header file,  or a default label.

This is an option to change it to something else.

TRANSFORM-FILTER MENU

Transform-Filter-Filter�data- This function starts the  filtering  of

the  current  buffer.  The filter function is first generated using the

last parameters selected.  The number of points filtered is the "Points

per sweep" (see above Re-scale menu).

Transform-Filter-Type�of�filter-  There  is  a  choice   of   digital

approximations   to   common  analogue  filters,   two  non-causal  (or

symmetrical) filters (Gaussian and sinc),  two differentiating filters,

and a bandpass filter.  In addition there are Savitsky-Golay and Wiener

filters.

The Sinc, Gaussian, Dsinc, 3ptdiff, Bandpass filters, Savitsky-Golay,

and  Wiener filters are non-causal,  and the Butterworth,  Bessel,  and

Tchebyshev filters are causal.  (By non-causal, we mean that the output

of the filter depends on both the preceding  and  the  succeeding  data

points.)

To  filter  without  a  time  shift,  use the non-causal filters.  To

obtain a minimal  relative  phase  shift  (or  "ringing")  uses  either

Gaussian  or  Bessel.  To  obtain  maximal  attenuation  at the cut-off

frequency use the Sinc or Tchebyshev filter.

To differentiate a trace,  use either the 3ptdiff  for  minimal  high

frequency  attenuation,  or  Dsinc  (differentiated  Sinc function) for

selecting a specific attenuation of the differentiated signal.

To get the very best signal to noise ration  with  known  signal  and

noise spectral densities, use the Wiener type filters (if you are brave

enough!).

The analogue filters are specified by their name, their roll-off, and

the number of poles.

The  Gaussian  filter  needs only a roll-off.  The Sinc,  Dsinc,  and

bandpass filters need a roll-off and a  window  width  of  the  impulse

response.  The  window width is the number of cycles to one side of the

central maximum that  are  passed  unchanged  before  a  modified  Hann

(raised cosine) window truncates the response.

The  Bandpass  filter also requires a bandwidth,  which should not be

greater than twice the roll-off.

There are also Savitsky-Golay filters,  which is specified again by a

number  of  points  (you  just  specify an approximate roll-off) and an

"order" which is taken from the number of poles.

Lastly  there  are  Wiener  and  Wiener/scc  filters  which   require

absolutely   a   noise   spectrum   (in  the  Transform-Transform  FFT-

Noise/signal-noise file),  and in the case of the  Wiener  filter  also

require a signal spectrum (in the Transform-Transform FFT-Noise/signal-

signal  file).  The use of a system response spectrum is optional since

in some circumstances it may be legitimately assumed to be flat in  the

frequency  range  of  interest.  Without  a  system  response  function

specified, the system response is assumed to be flat out to a frequency

that you enter as the "Roll off -3 dB".  The Wiener/scc filter  assumes

that you have square events (single channel currents) as the underlying

signal, and prompts you at some point for the size of the square event.

It  is  advisable not to use these filters unless you really understand

what  is  going  on.   (see  Press  et   al,   Numerical   Recipes   in

C/Fortran/Pascal,  Cambridge.)  Note  that  there must be a valid power

spectrum in the file that you are  prompted  for,  and  it  must  be  a

spectrum  of  the  same  number  of  points  that  you  specify  in the

Transform-Transform FFT-Ranges menu-2 power N.

Transform-Filter-Poles- The number of poles (analogue  filters  only)

determines  how  steep is the attenuation above the roll-off frequency.

The more poles then the steeper the attenuation.  The minimum is 2  and

the maximum 8.

Transform-Filter-Roll off -3dB- The frequency  at  which  the  signal

will be attenuated by 3 decibels should be given here.

Transform-Filter-Local filter- Sometimes you  may  want  to  see  the

result  of  filtering  just  the  part  of  a  data sweep that you have

displayed rather than the whole sweep.  This function  does  that,  and

automatically returns you to the Graph-Display mode.

Transform-Filter-Window width- The half-width of the Hann window used

to truncate the Sinc,  Dsinc,  Bandpass  and  Bandstop  filter  impulse

responses.  Widths  from  1 to 5 will work well with the Sinc and Dsinc

filters,  but  the  Bandpass  and  especially  the Bandstop filters may

require larger windows. It  is  strongly recommended that you check the

"before" and "after" power spectra for the shape of  response  you  had

envisioned.

NB  all  the  filtered traces are preceded by an undefined section of

data equal to the length (or half the length) of the  impulse  response

function.  The preceding section is "5-point averaged" only.

NB  Bandstop:  this  is  a function better performed in the frequency

domain,  however with a large value for the "Window width"  between  20

and 30 dB can be achieved for a quite narrow bandwidth (10% of Fc). For

wider bandwidths, filtering several times (up to three) can improve the

result.

TRANSFORM-TRANSFORM FFT MENU

This  menu  is  used to perform power spectrum analysis.  The maximum

number of points that may be analyzed is  4096.  The  newly  calculated

spectrum may be saved/averaged to one of two files, labelled "Noise" or

"Signal".  The difference spectrum may be calculated.  Any of the "Last

FFT",  the  "Noise",  the  "Signal"  or the "Difference" spectra may be

displayed,  and dumped to an ASCII file,  or plotted to a printer using

the PCL or HPG instruction set.  In addition, some manipulations of any

of the spectra may be made:  smoothing,  integration,  differentiation,

convolution, deconvolution.

Transform-Transform FFT-Ranges- This brings up a menu,  from which it

is possible to select a range of data to transform,  either using "Mark

data range" (allows you to move  two  screen  cursors  around),  or  by

typing in the start and end of the data range,  and/or the power of two

required.  If the selected range is not a power of two,  then the  data

is "zero-padded" to the next higher power of two. Normally, the spectra

are scaled automatically for screen display, but it is also possible to

choose the Y scale range, and set the scaling to "Manual scaling ON".

Transform-Transform  FFT-Go FFT- Using this command produces a FFT of

the selected data range, and displays it as a power spectrum,  with the

phase of each component.

Transform-Transform  FFT-Noise/Signal:-  This  brings  up a menu from

which the following can be selected:  the  "Noise  file",  the  "Signal

file",  the  "Difference  file",  or  the "Last FFT".  Whichever FFT is

selected here is used when the "Plot",  the "Average" or  the  "Output"

functions are used.  You are prompted for a file name,  but if the file

name is correct,  then just press [Enter].  Pressing "=" gets  you  the

path and name (no extension) of the data file loaded.

Transform-Transform  FFT-Average  FFT- The last transform is averaged

(or rather the power spectrum is averaged) with  the  contents  of  the

currently selected file (Signal, Noise or Difference but not Last FFT).

Note  that  you  are  not  allowed  to  average transforms of different

numbers of points.

Transform-Transform  FFT-Clear average- Resets the currently selected

file,  so that no transform is represented by that file,  and sets  the

number averaged to zero.

Transform-Transform FFT- Subtract menu- Do subtraction- Provided that

a) the Noise and Signal files contain FFTs and b) the FFTs are the same

size,  a scalar difference of power spectra is calculated,  and written

to  the  file  with  the  file  name  selected  under  Transform   FFT-

Noise/Signal-Difference   file   name.   This  difference  spectrum  is

displayed on screen.  Note the option to "Smooth first".  The smoothing

parameters are set in the "Functions" menu.

Transform-Transform  FFT-Plot  the transform- Whichever file has been

selected is plotted as a power spectrum on screen.

Transform-Transform FFT-Display window:- Select which plotting window

(of those set under the Graph Menu) to use for the plot  of  the  power

spectrum.

Transform-Transform   FFT-Taper   edges-   To  avoid  spurious  power

resulting from the discontinuity of data at the limits,  it is  usually

necessary to do two things. The first is to subtract any offset so that

the signal is as close as possible to zero at the limits. The second is

to  fade  out the data gradually towards the limits.  There are several

windowing functions offered,  but recommended are the  Hann  and  Welch

windows.  A  "square" window is basically no windowing function at all,

unmodified data out to the limits.

Transform-Transform FFT-Baseline subtract- This option may be toggled

ON or OFF.  If it is on,  then on running the FFT (Go  FFT),  the  mean

value  of  the  selected data range is calculated and subtracted before

the FFT is calculated.  See above for when to use this.

Transform-Transform FFT-Multiple FFTs- This  calls  up  a  menu  from

which it is possible to set up repeated FFT calculations, and averaging

to  the  selected  file.  Either  the  same  data  range may be used in

successive sweeps (Consecutive sweeps),  or successive blocks  of  data

from  the  same  data sweep are used,  starting with the block selected

under the Ranges menu.  If the end of the sweep, or the end of the data

file is reached, the calculation of FFTs ends gracefully!

Transform-Transform FFT-Output- The options are to  either  create  a

file  of  ASCII  X,Y  values,  which are the frequency (linear) and the

power at that frequency,  or to plot the data to a plot-file.  The plot

file  name  is  the same as the one specified under Copy menu,  and the

plot is appended to this file.  Either the PCL language or HPG language

is used,  depending again on the settings under the Copy menu.  The dot

size and symbol used are those specified under the Laser menu,  and the

axis labels are written using the scale bar  font  and  size  specified

under the Laser menu-Change font menu.

Transform-Transform-FFT-Functions-  A  menu of manipulations that can

be applied to a spectrum.

First,  note that all manipulations will be applied to  whichever  of

the   Noise/signal/difference/Last   spectra   is   selected   under  -

Noise/Signal:. Also note that no changes are made to the file until you

request the file to be saved.

_______________________________________________________________________

TRANSFORM-TRANSFORM FFT-FUNCTIONS

-Mult.  by f.  The power spectrum displayed on screen is tilted up to

the  R  by  an  additional  slope  of  2.  This  is  the  equivalent of

integrating the time series data from which the spectrum came.

-Divide by f.  This is the equivalent  of  differentiating  the  time

series  data.  The  power  spectrum displayed is tilted down to R by an

additional slope of 2.

-Convolute with:- You are first prompted for a file  name.  You  must

give  the name of a file that contains a spectrum of the same number of

points as the spectrum that you already have loaded.  The  spectra  are

then multiplied together point by point.

-Unconvolute  with:-  Sorry  about  this  ugly  neologism.  This is a

"deconvolution". As above, you are prompted for the name of a file that

contains a power spectrum.  The previously loaded spectrum  is  divided

point by point by the deconvolution spectrum.

-FFT smooth. There are two ways given of smoothing the spectra. Which

is  used  depends  on  the  settings of the parameters below.  Either a

triangular filter (with a constant log-width) is used,  or an arbitrary

polynomial.

-Length:. The number of points to be left unaltered by the triangular

filter, and also the number of points to move R before incrementing the

scalar width of the filter. A value of 10 is a good starting point.

-Order:.  The  degree or order of the polynomial to be used to smooth

the spectrum.  If this is set to zero, then a triangular filter is used

instead.  This  means  of  smoothing is not very effective.  Even large

values (20-30) for the order do not always  result  in  a  satisfactory

representation of the spectrum. The maximum value allowed is 50.

-Start FFT fit:. Which data point to begin the polynomial fit.

-End FFT fit:. Which data points to end the polynomial fit.

-@Save FFT.  If you have,  for example,  the NOISE spectrum selected,

and the spectrum is stored in a file "noise.fft",  then when you select

@Save  FFT,  the  results  of  the  last  manipulation will be saved to

"noise.fft".

NB Normalization.  Two points need to be borne in mind.  First  that

what  you  see displayed is a normalized spectrum.  If you add together

the power at each frequency,  you will get power in the  original  time

series data, ie the vertical scale is not is power/Hz but power per FFT

interval.  The  second point to be remembered is that the normalization

is calculated on plotting, displaying or manipulating the FFT,  so that

is you change the Fullscale Y value in Transform-Rescale, then you will

change what you see when you re-plot the transform. (The transform does

not   have   to   be   re-calculated  for  a  change  to  take  effect.

_______________________________________________________________________

MACRO MENU

Macro-Go- Start  running  a  macro.  Characters  are  read  from  the

current  macro  file  whose  file name is the last one to be entered as

"Macro  file  name".   These  characters  are   then   interpreted   as

keystrokes.

Macro-Macro  file  name-  Enter the name of the file that you wish to

record to or run.

Macro-Record- This function toggles  "ON/OFF".  When  you  turn  this

function on, each subsequent key stroke is written to the file given by

"Macro  file  name",  starting  at  the beginning of the file.  To stop

recording a macro, then use this function again, ie toggle it "OFF".

Macro-Delete- At the moment this function is  unnecessary,  since  an

empty file is created on starting to record.

Macro-0..9  preset-  Up  to  ten  macro  file  names may be saved and

recalled.

Macro-Number of runs- This enables the macro to be repeated.  Enter a

number here,  and the macro is repeated that many times on pressing "G"

(for  Go).  Note  that  the  number  is reset to zero by the process of

running the macro: this is to avoid inadvertent use of the feature.

NB Editing a macro file using a text editor.  Ideally you should use  a

text  editor  that  allows  you to enter any character from #0 to #255.

Note  that  for  example  the  keystroke  <Right  Arrow>  generate  two

characters,  a  null  character (#0) plus another character ('M').  For

convenience,  the  character  '^'  is  interpreted  to  mean  the  null

character  (#0),  so  instead of using the null character followed by a

capital 'M' to mean <Right Arrow>,  the symbols ^M may be used instead.

-----------------------------------------------------------------------

#