|---------------------------------|

|  PCI Bus Sniffer version 0.65�  |

|---------------------------------|

Created by Marco Koetsveld,

on the 4th of august 1999

About this PCI Bus Sniffer

----------------------------

This is a very easy to use program that allows you to inspect the

PCI configuration of your computer. When you start the program

the first check will be if you have a PCI-bus in your computer.

If you don't have PCI, don't try it, just a waste of time.

But when you do have PCI, you can use this program to get maybe

just the info you need.

I have created this program using the routines provided by Craig

Hart and PCI information from Ralf Brown's files. The reason I

made it is quite simple: I was working on unattended Windows 95

installations, but I wanted to have PCI specific information of

the pc's I was working on. So I needed some background info on

PCI and created this program. Hope you like it.

By the way, it is compiled using Turbo Pascal 6.0

How it works

--------------

Before anything of PCI is scanned, the list with all the PCI Vendors

and PCI Devices is loaded into memory, this could cause a short delay

when running the program from floppy disk. This list is called

VENDORS.TXT and has to be in the same directory as the PCISNIF.EXE

Then the first step is to get the PCI configuration of your computer.

This is done by direct hardware access instead of Bios-calls, the

reason is simply because getting PCI information using a Bios-call

fails occasionally and gives you wrong information. If this direct

hardware access is not supported by your pc, the bios-method is used.

Next the information is stored in memory and displayed on your monitor.

The PCI devices that are detected in your pc are on the MainScreen,

scroll through the list and press Enter to get specs on the highlighted

PCI device (and Escape to return to MainScreen).

When you are in the MainScreen, press F5 to get PCI information such

as PCI version, PCI Characteristics, number of buses etc. When you

press F5 a screen appears which displays a summary of IRQ's assigned

to PCI devices, possible sharing of IRQs is also shown.

When you want to save all the information to a file, press F2 when

you are in the mainscreen. The information is saved to a logfile.

Try it out, it gives you a complete overview of all the info that is

shown in the program. The name of this file is created using the

biosdate of your computer. I used this feature to inspect various

Bios-versions on some pc's and possible differences in PCI info.

Usage of the keys

-------------------

Cursor Up        : Up......

Cursor Down      : Down.....

Home             : Sweet home, go to top of list

End              : Goto end of list

PageUp           : Page Up.....

PageDown         : Page Down.... (this is getting stupid)

Cursor Left      : Jump to top of list on screen

Cursor Right     : Jump to bottom of list on screen

Enter            : Display Specs of selected PCI device

Escape           : When Specs of PCI device are shown --> return to MainScr.

When on MainScreen --> The End (of this program)

F1               : Help

F2               : Save PCI information to LogFile

F3               : View Vendors.Txt

F5               : Show PCI version, buses etc

F6               : Show PCI IRQ Summary

F7               : Show PCI IRQ Routing Table

Updates of Vendor & Device information

----------------------------------------

All the PCI Vendors & Devices (and subsystems) are listed in the list

VENDORS.TXT. I have supplied this program with version 79 of this file.

VENDORS.TXT is maintained by Craig Hart, the latest version of this

list is always available from: http://members.hyperlink.net.au/~chart

Check this site out for all the FREE information on PCI, AGP, etc etc!!

If you have additions/corrections to this list, please send you Email

to Craig Hart: chart@hyperlink.net.au

Notice

--------

PCISNIF is a program that is distributed for free. You can use it as much

as you want, but use it at your own risk. I am NOT RESPONSIBLE for any

incorrect information provided by PCISNIF.

Updates of this program & contacting me

-----------------------------------------

If you have any comments on this program please feel free to mail them

to me. This is my mail-address:  Marco.Koetsveld@Wang.Com

Succes with this PCI Bus Sniffer,

greetz............... Marco Koetsveld

Breukelen, Holland

August 4, 1999

Marco.Koetsveld@Wang.Com

ps. It will not work under Windows NT

And Craig's new vendors format is not supported yet