Chapter   3

Award BIOS Utility

This chapter explains the Setup Utility for the Award BIOS and tells how to configure the system by setting the BIOS parameters.

# 3.1	Entering the Award BIOS Setup Menu

The Award BIOS Utility allows you to reconfigure your system.  To enter the system menu, turn on the system.  During POST, a message telling you how to enter Setup appears at the bottom of the screen.  Follow this instruction by simply pressing c.

ROM PCI/ISA BIOS (XXXXXXXX)
CMOS SETUP UTILITY
AWARD SOFTWARE, INC.

| STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP PCI CONFIGURATION SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS |  |  | I/O CONFIGURATION SETUP PASSWORD SETTING IDE HDD AUTO DETECTION HDD LOW LEVEL FORMAT SAVE & EXIT SETUP EXIT WITHOUT SAVING |  |
| --- | --- | --- | --- | --- |
| ESC : Quit ¯ ® ¬ : Select Item F10 : Save & Exit Setup (Shift) F2 : Change Color |  |  |  |  |
| Description of each function |  |  |  |  |
|  |  | Choosing Load BIOS Defaults or Load Setup Defaults at this point modifies all applicable settings. |  |  |

The section at the bottom of the screen tells how to control the screen.  Use the arrow keys to move between items, jm to color scheme of the display, ^ to exit, and u to save the changes before exit.  Another section at the bottom of the screen displays a brief description of the highlighted item.

After selecting an item, press e to select or enter a submenu.

# 3.2	Standard CMOS Setup

The Standard CMOS Setup sets the basic system parameters such as the date, time, and the hard disk type.  Use the arrow keys to highlight an item and { or } to select the value for each item.

ROM PCI/ISA BIOS (XXXXXXXX)
STANDARD CMOS SETUP
AWARD SOFTWARE, INC.

Base Memory : 640 K Extended Memory:15360 K Other Memory : 384 K Total Memory : 16384 K Date (mm:dd:yy) : Wed. Mar 6 1996 Time (hh:mm:ss) : 00:00:00 HARD DISK TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTORS MODE Primary Master : Auto 0 0 0 0 0 0 AUTO Primary Slave : None 0 0 0 0 0 0 --- Secondary Master: None 0 0 0 0 0 0 --- Secondary Slave : None 0 0 0 0 0 0 --- Drive A : 1.44M, 3.5 in Drive B : None Video : EGA/VGA Halt On : All Errors

ESC : Quit ¯ ® ¬ : Select Item PU/PD/+/- : Modify F1 : Help (Shift) F2 : Change Color

## 3.2.1	Date

To set the date, highlight the Date parameter.  Press { or } to set the current date.  The date format is month, date, and year.

## 3.2.2	Time

To set the time, highlight the Time parameter.  Press { or } to set the current time in hour, minute, and second format.  The time is based on the 24-hour military clock.

## 3.2.3	Hard Disks

The four hard disk parameters listed in the menu are Primary Master, Primary Slave, Secondary Master and Secondary Slave.  These parameters allow you to configure the drives that you connect into your IDE connectors.

To configure, press { or } to select a hard disk drive type, or type the number and press e.  Select User to define your own hard disk type manually.  You can use the IDE HDD Auto Detection function in the Main Menu to automatically configure your hard disk drive parameters.  If your system does not have a hard disk drive, select None and press e.

|  | For an IDE hard disk, we recommend that you use the Auto Detection utility to enter the drive specifications automatically. See the section IDE HDD Auto Detection. Select the User option if you want to enter the specifications manually. |
| --- | --- |

## 3.2.4	Floppy Drive Types

Press { or } to select a floppy disk drive type.  The settings are:

- 360 K, 5.25-in.

- 1.2 M, 5.25-in.

- 720 K, 3.5-in.

- 1.44 M, 3.5-in.

- 2.88 M, 3.5-in.

Choose None if you have no floppy drive.

## 3.2.5	Video

This parameter allows you to configure the video display card present in your system.  The video settings are:

- EGA/VGA

- CGA 40

- CGA 80

- MONO

Select the adapter type setting that matches your video display card and monitor.

## 3.2.6	Error Halt

This parameter enables you to control the system stops in case of error.  The available settings are:

- No Errors

- All Errors

- All But Keyboard

- All But Diskette

- All But Disk/Key

The default setting is All Errors.

## 3.2.7	Memory

The Memory parameters are for display only.  These are determined by the BIOS power-on self-test.

### Base Memory

The POST detects the amount of base (or conventional) memory installed in the system.  The value of the base memory is 640 KB for systems with 640 KB or more memory installed on the mainboard.

### Extended Memory

The BIOS detects the amount of extended memory present during the POST.  This is the amount of memory located above 1 MB in the CPU memory address map.

### Other Memory

This refers to the memory located in the 640 KB to 1024 KB address space.  This is the memory that can be used for different applications.  DOS uses this area to load device drivers to keep as much base memory free for application programs.

# 3.3	BIOS Features Setup

This screen appears when you select the option BIOS Features Setup from the main menu.

ROM PCI/ISA BIOS (XXXXXXXX)
BIOS FEATURES SETUP
AWARD SOFTWARE, INC.

| Virus Warning : Disabled CPU Internal Cache : Enabled External Cache : Enabled Quick Power On Self Test : Enabled Boot Sequence : A, C Swap Floppy Drive : Disabled Boot Up Floppy Seek : Enabled Boot Up NumLock Status : On Boot Up System Speed : High Gate A20 Option : Fast Memory Parity Check : Disabled Typematic Rate Setting : Disabled | Video BIOS Shadow : Enabled C8000-CBFFF Shadow : Disabled CC000-CFFFF Shadow : Disabled D0000-D3FFF Shadow : Disabled D4000-D7FFF Shadow : Disabled D8000-DBFFF Shadow : Disabled DC000-DFFFF Shadow : Disabled |
| --- | --- |
| Typematic Rate (Chars/Sec) : 6 Typematic Delay (Msec) : 250 Security Option : Setup PS/2 mouse function control : Disabled PCI/VGA Palette Snoop : Disabled | ESC: Quit ¯®¬: Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |

## 3.3.1	Virus Warning

Set this parameter to Enabled to activate the warning message.  This feature protects the boot sector and partition table of your hard disk from virus intrusion.  Set it to Disabled to disregard the message.

Any attempt during boot-up to write to the boot sector of the hard disk drive stops the system and the following warning message appears on the screen.  Run an anti-virus program to locate the problem.

! WARNING !
Disk Boot Sector is to be modified
Type "Y" to accept write, or "N" to abort write
Award Software, Inc.

## 3.3.2	CPU Internal Cache

The default setting for this parameter is Enabled.  This enables the CPU’s built-in cache.  Disabling the parameter slows down the system.  Therefore, we recommend that you leave it enabled unless you are troubleshooting a problem.

## 3.3.3	External Cache

Similar to the CPU internal cache, the default setting for this parameter is Enabled.  This enables the secondary cache.  Disabling the parameter slows down the system.  Therefore, we recommend that you leave it enabled unless you are troubleshooting a problem.

## 3.3.4	Quick Power-on Self-test

This parameter speeds up POST by skipping some items that are normally checked.  The default setting is Enabled.

## 3.3.5	Boot Sequence

The settings are C,A and A,C to specify the system search sequence.  The default setting is A,C.

## 3.3.6	Swap Floppy Drive

Enabling this parameter tells your system to read the current floppy disk drive in use regardless of the drive designation.  The system may read drive A as drive B or vice-versa.  The default setting is Disabled.

## 3.3.7	Boot-up Floppy Seek

When this parameter is set to Enabled, BIOS detects whether there is a floppy disk drive installed in your system.  Set it to Disabled to disregard the function.  The default setting is Enabled.

## 3.3.8	Boot-up NumLock Status

Setting this parameter to On enables the numeric function of the numeric keypad.  Set this parameter to Off to disregard the function.  Disabling the numeric function allows you to use the numeric keypad for cursor control.  The default setting is On.

## 3.3.9	Boot-up System Speed

Set the system speed to High or Low with this parameter.  High is the default setting.

## 3.3.10	  Gate A20 Option

The settings for this parameter are Normal and Fast.  If the data transfer is controlled by the 8042 chip, set the parameter to Normal.  The Fast setting transfers the control to the ASICs.  The default setting is Fast.

## 3.3.11	  Memory Parity Check

This parameter lets you enable or disable the optional parity DRAM.  The default setting is Disabled.  If you install SIMMs with parity bit chips, set this parameter to Enabled.

## 3.3.12	  Typematic Rate Setting

Set this parameter to Enabled to determine the typematic rate.  Set it to Disabled to disregard the rate setting.  The default setting is Disabled.

## 3.3.13	  Typematic Rate

This parameter allows you to control the speed at which the system registers repeated keystrokes.  The typematic rate settings are 6, 8, 10, 12, 15, 20, 24, and 30 chars/sec.  The default setting is 6 chars/sec.

## 3.3.14	  Typematic Delay

This parameter allows you to control the display time between the first and the second characters.  The typematic delay settings are 250, 500, 750, and 1000 msec.  The default setting is 250 msec.

## 3.3.15	  Security Option

The security option settings are System and Setup.  The System option limits access to both the System and Setup.  A prompt asking you to enter your password appears on the screen everytime you boot the system.  The Setup option limits access only to setup.  The default setting for this parameter is Setup.

To disable the security option, select Password Setting from the main menu and press e.

## 3.3.16	  PS/2 Mouse Function Control

Setting the parameter to Enabled and connecting a mouse device to your system automatically assigns IRQ12 to mouse function.  Select Disabled to re-assign IRQ12 to other functions.  The default setting is Disabled.

## 3.3.17	  PCI/VGA Palette Snoop

This function lets you enable or disable the palette snooping function in the PCI/VGA devices installed in the system.  Palette snooping is a technique that enables the a device to transfer the control access to the palette registers.  Before you set this parameter to Enabled, check your PCI and VGA device manuals for more information about this function.  The default setting is Disabled.

## 3.3.18	  Video BIOS Shadow

Enabling the parameter allows you to change the video BIOS from ROM to RAM.  When the system boots, the video display card BIOS routines are copied into the RAM area.  This enhances system performance as information access is faster in RAM than in ROM.  The default setting for this parameter is Enabled.

## 3.3.19	  C8000-CBFFF Shadow to DC000-DFFFF Shadow

These six lines are for shadowing other expansion cards with ROMs.  The default setting for these areas is Disabled.  You need to know the specific addresses that ROMs use to shadow the expansion cards.  If you do not know this information, enable all the ROM shadow settings.  This ensures shadowing of any present ROMs.  Also, enabling all the settings reduces the available memory by 640 KB to 1024 KB.

|  | The F000 and E000 addresses are exclusively shadowed for BIOS. |
| --- | --- |

# 3.4	Chipset Features Setup

The following screen appears when you select the option Chipset Features Setup from the main menu.

ROM PCI/ISA BIOS (XXXXXXXX)
CHIPSET FEATURES SETUP
AWARD SOFTWARE, INC.

| Auto Configuration : Enabled L2 Cache Update Mode : WB L2 (WB) Tag Bit Length : 7 bits Sync SRAM Leadoff Timing : 3T DRAM RAS to CAS Delay : 4T RAS Active When Refresh : 5T CAS Delay In Posted-WR : 1T FP CAS Precharge Time : 1T FP RAS Precharge Time : 4T EDO CAS Pulse Width : R1 W2 T EDO CAS Precharge Time : 1T EDO RAS Precharge Time : 4T | System BIOS Cacheable : Disabled Video BIOS Cacheable : Enabled Memory Hole At 15M-16M : Disabled 16 bit I/O wait state : 1 wait 16 bit I/O Recovery time : 5 BusClk 8 bit I/O Recovery time : 8 BusClk |
| --- | --- |
| Slow Refresh (1:4) : Disabled ISA Bus Clock Frequency : PCICLK/3 | ESC: Quit ¯®¬: Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |

Do not change the values of the Chipset parameters unless you are a qualified technician.  To configure, simply set the Auto Configuration parameter to Enabled.  This automatically enters and locks the optimum settings for the chipset.  Disable the parameter to unlock the settings without changing them.  The default setting is Enabled.

# 3.5	Power Management Setup

The Power Management Setup screen enables you to control the mainboard’s green features.  See the following screen.

ROM PCI/ISA BIOS (XXXXXXXX)
POWER MANAGEMENT SETUP
AWARD SOFTWARE, INC.

| Power Management : Disabled PM Control by APM : Yes Video Off Option : Susp, Stby®Off Video Off Method : V/H SYNC+Blank Suspend Switch : Enabled Doze Speed (div by) : 2 Stby Speed (div by) : 3 ** PM Timers** HDD Off After : Disabled Doze Mode : Disabled Standby Mode : Disabled Suspend Mode : Disabled ** PM Events** COM Ports Activity : Enabled | VGA Activity : Disabled IRQ3 (COM2) : Enabled IRQ4 (COM1) : Enabled IRQ5 (LPT2) : Enabled IRQ6 (Floppy Disk) : Enabled IRQ7 (LPT1) : Enabled IRQ8 (RTC Alarm) : Disabled IRQ9 (IRQ2 Redir) : Enabled IRQ10 (Reserved) : Enabled IRQ11 (Reserved) : Enabled IRQ12 (PS/2 Mouse) : Enabled IRQ13 (Coprocessor) : Enabled IRQ14 (Hard Disk) : Enabled IRQ15 (Reserved) : Enabled |
| --- | --- |
| LPT Ports Activity : Enabled HDD Ports Activity : Enabled PCI/ISA Master Act. : Enabled IRQ-15 Activity : Enabled | ESC: Quit ¯®¬: Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |

## 3.5.1	Power Management

This function enables you to control the power-saving modes, display turn off and HDD power down.  The four settings are as follows:

| Setting | Description |
| --- | --- |
| Max Saving | Maximize energy conservation by putting the system into power-saving mode after a brief period of system inactivity. The maximum saving default is 20 sec. |
| Min Saving | A power-saving assignment which activate after a moderate period of system inactivity. The minimum saving default is 40 Min. |
| Disabled | Turns off the power-saving function. |
| User Defined | Allows you to set the power-saving options according to your requirements. |

## 3.5.2	PM Control by APM

If your system supports the Advanced Power Management (APM) feature, set this parameter to Yes.

## 3.5.3	Video Off Option

This parameter determines when the video off feature activates.  The default is Susp, Stby®Off.

## 3.5.4	Video Off Method

If your video display adapter supports the green features, set this parameter to its default setting which is V/H SYNC+Blank.  If not, then set this to Blank Setting.

The default setting saves more power since it turns off the CRT vertical and horizontal scanning once the monitor screen blanks.  However, with non-green monitors, CRT scanning does not stop even if the monitor screen blanks.

|  | Screen saver software does not work with this feature. It is used only to prevent burning in a static image on the CRT while the monitor is on. A screen saver cannot display while the monitor is shut down to save both energy and the screen. |
| --- | --- |

## 3.5.5	Suspend Switch

Enabling this parameter also enables the SMI connector on the mainboard which connects to the lead from a Suspend switch mounted on the system case.  The default setting is Enabled.

## 3.5.6	Doze & Standby Speeds

These lines allow you to set the speed at which the CPU will operate during each mode.  The number indicates the normal CPU speed divisor.

## 3.5.7	PM Timers

These parameters enable you to control the time-out settings for the Power Management Scheme.

### HDD Off After

This time-out setting indicates when to shut down the IDE hard disk.  The settings are from 1 to 15 minutes or Disabled.

|  | HDD Power Down does not affect SCSI hard disks. |
| --- | --- |

### Doze Mode, Standby Mode, Suspend Mode

These modes set the period of time after which each of these modes activates.

## 3.5.8	PM Events

This function allows you to monitor system activities.  Enabling the items under this function resumes system to full power.

|  | Normally, a Microsoft serial mouse or compatible uses either COM1 (IRQ4) or COM2 (IRQ3) and a PS/2-type mouse uses IRQ12. If you know the IRQ of your mouse, you can make sure that the wake-up event for that IRQ is enabled by moving the mouse or clicking a button. |
| --- | --- |

## 3.5.9	IRQ3 to IRQ15

These lines allow you to set the IRQs individually.  An activity from any enabled IRQ resumes the system to normal mode if the IRQ 15 Activity item in the PM Events is enabled.

# 3.6	PCI Configuration Setup

The PCI Configuration Setup allows you to configure the PCI devices installed in your system.  The following screen appears if you select the option PCI Configuration Setup from the main menu.

ROM PCI/ISA BIOS (XXXXXXXX)
PCI CONFIGURATION SETUP
AWARD SOFTWARE, INC.

| PnP BIOS Auto Config : Disabled 1st Available IRQ : 10 * 2nd Available IRQ : 11 * 3rd Available IRQ : 9 * 4th Available IRQ : 5 * PCI IRQ Activated By : Level PCI IDE 2nd Channel : Enabled PCI IDE IRQ Map To: : PCI-AUTO Primary IDE INT# : A Secondary IDE INT# : B CPU-PCI Post Write Rate : 4T Latency for CPU-PCI : 1T CPU-PCI Burst Mem Write : Enabled |  |
| --- | --- |
| CPU-PCI Post Mem Write : Enabled PCI Master Burst Range : 2KB | ESC: Quit ¯®¬: Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |

## 3.6.1	PnP BIOS Auto Configuration

When enabled, the BIOS automatically sets an IRQ for each PCI/PnP device.  The Disabled setting allows you to manually set the IRQ assignments.  The default setting is Disabled.

|  | Disable the parameter if you have a non-PnP device installed in your system. |
| --- | --- |

## 3.6.2	Available IRQs

These parameters appear only if the PnP BIOS Auto Configuration parameter is set to Disabled.  These let you to sequentially assign the IRQ to the PCI slots.  The available settings are 3, 4, 5, 7, 9, 10, 11, 12, 14, 15 and NA. When assigning, make sure that the IRQs do not conflict.

Enabling the PnP BIOS Auto Configuration to Enabled automatically sets the IRQ for each slot.  Therefore, these parameters do not appear on screen.

## 3.6.3	PCI IRQ Activated By

This option lets the user set when to activate the PCI IRQ.  The available settings are Edge and Level.  The Edge is the start of the signal and Level is the point at which the signal is high.  The default setting for this parameter is Level.

## 3.6.4	PCI IDE 2nd Channel

This parameter lets you enable or disable the second channel of the PCI IDE card, including the I/O port and the assigned IRQ to it.  The default setting is Enabled.

## 3.6.5	PCI IDE IRQ Map To

This feature enables you to control the PCI IDE IRQ mapping.  The available options are ISA, PCI-Slot 1, PCI-Slot 2, PCI-Slot 3, PCI-Slot 4, and PCI-Auto.  The default is PCI-Auto.

### Primary and Secondary IDE INT#

These options let you set the interrupt pins assigned for the primary and secondary channels of the PCI IDE card installed in your system.

## 3.6.6	CPU-PCI Post Write Rate

This parameter lets you to set the posting rate of the CPU to the PCI write data in the posting buffer.  The selections are 3T and 4T.  The default setting is 4T.

## 3.6.7	Latency for CPU-PCI

This parameter lets you to set the CPU to PCI latency time during write cycle.  The latency time refers to the length of time that the bus master occupies the PCI bus.  The selections are 1T and 2T.  The default is 1T.

## 3.6.8	CPU-PCI Burst Mem Write

Enabling this parameter allows the translation of the host cycles into memory-burst cycles.  After translation, this controls the memory burst-write cycles.

## 3.6.9	CPU-PCI Post Mem Write

This parameter lets you control the CPU posting to PCI write data in the posting buffers.  The default setting is Enabled.

## 3.6.10	  PCI Master Burst Range

Increasing the burst range improves system performance.  However, the burst range varies depending on the SIMM configuration that your system supports.  The available options are 256 B, 512 B, 1 KB, 2 KB, and 4 KB.  Select 4 KB if you want to install SIMMs in pairs.

# 3.7	Load BIOS Defaults

This option loads the default values permanently stored in the BIOS ROM.  If the values specified in the Setup utility become corrupted, these defaults load automatically when you turn on the system power.  The defaults loaded affect all the Setup items except the Standard CMOS Setup.  These settings are non-optimal and bypass all high-performance features.  This is useful if you are having problems with your current system configuration and need to determine the cause.

To use this feature, highlight this option on the main menu and press e.  A line appears on the screen asking if you want to load the BIOS default values.  To load, press Y then e.  Press N to disregard.  See the following screen.

ROM PCI/ISA BIOS (XXXXXXXX)
CMOS SETUP UTILITY
AWARD SOFTWARE, INC.

| Load BIOS Defaults (Y/N)? STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP PCI CONFIGURATION SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | I/O CONFIGURATION SETUP PASSWORD SETTING IDE HDD AUTO DETECTION HDD LOW LEVEL FORMAT SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| ESC : Quit ¯ ® ¬ : Select Item F10 : Save & Exit Setup (Shift) F2 : Change Color |  |
| Load BIOS Defaults except Standard CMOS Setup |  |

# 3.8	Load Setup Defaults

The Load Setup Defaults option loads optimized settings from the BIOS ROM.  Use this option to load the default settings for normal use. The defaults loaded affect all the Setup items except the Standard CMOS Setup.

To use this feature, highlight this option on the main screen and press e.  A line appears on the screen asking if you want to load the Setup default values.  To load, press Y then e.  Press N to disregard.  See the screen below.

ROM PCI/ISA BIOS (XXXXXXXX)
CMOS SETUP UTILITY
AWARD SOFTWARE, INC.

| Load Setup Defaults (Y/N)? STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP PCI CONFIGURATION SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | I/O CONFIGURATION SETUP PASSWORD SETTING IDE HDD AUTO DETECTION HDD LOW LEVEL FORMAT SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| ESC : Quit ¯ ® ¬ : Select Item F10 : Save & Exit Setup (Shift) F2 : Change Color |  |
| Load SETUP Defaults except Standard CMOS Setup |  |

# 3.9	I/O Configuration Setup

The I/O Configuration Setup allows you to configure the I/O features.  The following screen appears if you select the option I/O Configuration Setup from the main menu.

ROM PCI/ISA BIOS (XXXXXXXX)
I/O CONFIGURATION SETUP
AWARD SOFTWARE, INC.

| Onboard FDC Controller : Enabled Onboard Serial Port 1 : COM1/3F8 Onboard Serial Port 2 : COM2/2F8 Onboard Parallel Port : 378/IRQ7 Parallel Port Mode : Normal IDE HDD Block Mode : Enabled Internal PCI/IDE : Both IDE Primary Master PIO : Auto IDE Primary Slave PIO : Auto IDE Secondary Master PIO: Auto IDE Secondary Slave PIO : Auto Primary IDE Prefetch : Disabled Secondary IDE Prefetch : Disabled |  |
| --- | --- |
| IDE Burst Mode : Disabled IDE Post Write : Disabled | ESC: Quit ¯®¬: Select Item F1 : Help PU/PD/+/- : Modify F5 : Old Values (Shift) F2 : Color F6 : Load BIOS Defaults F7 : Load Setup Defaults |

## 3.9.1	Onboard FDC Controller

Setting this parameter to Enabled allows you to connect your floppy disk drives to the onboard floppy disk connector instead of a separate controller card.  Change the setting to Disabled if you want to use a separate controller card.  The default setting is Enabled.

## 3.9.2	Onboard Serial Ports 1 and 2

These lines allow you to assign addresses for the board’s serial connectors.  The serial ports can be assigned as follows:

| Port |  |  | Address |  |
| --- | --- | --- | --- | --- |
| COM1 |  |  | 3F8H (onboard serial port 1 default) |  |
| COM2 |  |  | 2F8H (onboard serial port 2 default) |  |
| COM3 |  |  | 3E8H |  |
| COM4 |  |  | 2E8H |  |
| Disabled |  |  | Disables the onboard port |  |
|  |  | Make sure that the ports have different assignments. |  |  |

## 3.9.3	Parallel Port

This parameter controls the onboard parallel port connector.  The options for this parameter are:

- 378H/IRQ7

- 3BCH/IRQ7

- 278H/IRQ5

- Disabled

The default setting is 378H/IRQ7.

|  | If you are using an I/O card with a parallel port, make sure that the addresses do not conflict. |
| --- | --- |

## 3.9.4	Parallel Port Mode

The default setting for this function is Normal.  If your system has a parallel interface peripheral device, set this function according to the enhanced mode that your device supports.  The available options are:

- Normal

- EPP (Enhanced Parallel Port)

- ECP (Extended Capabilities Port)

- ECP/EPP

## 3.9.5	IDE HDD Block Mode

This feature enhances disk performance by allowing multisector data transfers.  Most IDE drives, except with old designs, can support this feature.  The default setting is Enabled.

## 3.9.6	Internal PCI/IDE

Enabling this function activates the onboard IDE channels and connectors.  Set this to Disabled to deactivate the onboard PCI and IDE controllers.  The default setting is Both.

## 3.9.7	IDE Primary Master PIO

This parameter allows you to set the PIO mode supported by the master drive connected to your primary IDE connector.  The options are Auto, Mode 1, Mode 2, Mode 3, and Mode 4.  Select Auto to automatically detect the HDD PIO mode of your hard disk.  The default setting is Auto.

|  | This parameter does not appear if the Internal PCI/IDE is disabled. |
| --- | --- |

## 3.9.8	IDE Primary Slave PIO

This parameter allows you to set the PIO mode supported by the slave drive connected to your primary IDE connector.  The options are Auto, Mode 1, Mode 2, Mode 3, and Mode 4.  Select Auto to automatically detect the HDD PIO mode of your hard disk.  The default setting is Auto.

|  | This parameter does not appear if the Internal PCI/IDE is disabled. |
| --- | --- |

## 3.9.9	IDE Secondary Master PIO

This parameter allows you to set the PIO mode supported by the master drive connected to your secondary IDE connector.  The options are Auto, Mode 1, Mode 2, Mode 3, and Mode 4.  Select Auto to automatically detect the HDD PIO mode of your hard disk.  The default setting is Auto.

|  | This parameter does not appear if the Internal PCI/IDE is disabled. |
| --- | --- |

## 3.9.10	  IDE Secondary Slave PIO

This parameter allows you to set the PIO mode supported by the slave drive connected to your secondary IDE connector.  The options are Auto, Mode 1, Mode 2, Mode 3, and Mode 4.  Select Auto to automatically detect the HDD PIO mode of your hard disk.  The default setting is Auto.

|  | This parameter does not appear if the Internal PCI/IDE is disabled. |
| --- | --- |

## 3.9.11	  Primary IDE Prefetch/
  Secondary IDE Prefetch/
  IDE Burst Mode/
  IDE Post Write

Enabling these parameters improve system performance.  However, not all hard disks support these features.  We recommend that you consult your hard disk manual before you enable one of these parameters.

|  | These parameters do not appear if the Internal PCI/IDE is disabled. |
| --- | --- |

# 3.10	Password Setting

This option lets you set a password.  Password prevents unauthorized use of your computer.

ROM PCI/ISA BIOS (XXXXXXXX)
CMOS SETUP UTILITY
AWARD SOFTWARE, INC.

| Enter Password: ****** STANDARD CMOS SETUP BIOS FEATURES SETUP CHIPSET FEATURES SETUP POWER MANAGEMENT SETUP PCI CONFIGURATION SETUP LOAD BIOS DEFAULTS LOAD SETUP DEFAULTS | I/O CONFIGURATION SETUP PASSWORD SETTING IDE HDD AUTO DETECTION HDD LOW LEVEL FORMAT SAVE & EXIT SETUP EXIT WITHOUT SAVING |
| --- | --- |
| ESC : Quit ¯ ® ¬ : Select Item F10 : Save & Exit Setup (Shift) F2 : Change Color |  |
| Change/Set/Disable Password |  |

To set a password, highlight the password type that you want and press e.  At the prompt, type your password.  Your password can be up to 8 alphanumeric characters.  Press e to enter your password.  At the next prompt, re-type your password and press e again to confirm the new password.  After the password entry, the screen automatically reverts to the main screen.

To disable the password, press e when prompted to enter the password.  The screen displays a message confirming that the password has been disabled.

|  | If you use the Password features, the Security Option line in the BIOS Features Setup determines when to enter the password. |
| --- | --- |

# 3.11	IDE HDD Auto Detection

If your system has an IDE hard drive, you can use this utility to detect its parameters and enter them into the Standard CMOS Setup automatically.

ROM PCI/ISA BIOS (XXXXXXXX)
CMOS SETUP UTILITY
AWARD SOFTWARE, INC.

Select Primary Master Option (N=Skip) : N OPTIONSSIZECYLSHEADPRECOMPLANDZSECTORMODE 1 (Y)426826166553582563NORMAL HARD DISKS TYPE SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE Primary Master:

This utility only detects one set of parameters for your IDE hard drive.  Some IDE drives can use more than one set of parameters.  If your hard disk is formatted using different parameters than those detected, you have to enter the parameters manually.  If the parameters listed don't match the ones used to format the disk, the information on that disk won't be accessible.  If the auto-detected parameters displayed do not match those that used for your drive, ignore them.  Press N to reject the values and enter the correct ones manually from the Standard CMOS Setup screen.

# 3.12	HDD Low-level Format

The HDD Low-level Format utility is designed as a tool to save your time formatting your hard disk.  This automatically sets the necessary information of the selected drive.  It also searches for the bad tracks and lists them for your reference.

This screen appears if you select HDD Low-level Format from the main menu.

NO. CYLS HEAD

| Hard disk Low-level Format Utility SELECT DRIVE BAD TRACK LIST PREFORMAT Current selected drive is: C DRIVE: C CYLINDER: 0 HEAD: 0 |  |  |
| --- | --- | --- |
| SIZE CYLS HEAD PRECOMP LANDZ SECTOR MODE Primary Master : None 0 0 0 0 0 0 0 Primary Slave : None 0 0 0 0 0 0 0 Secondary Master: None 0 0 0 0 0 0 0 Secondary Slave : None 0 0 0 0 0 0 0 |  |  |
| Up/Down - Select item Enter - Accept ESC - Exit/Abort Copyright (c) Award Software, Inc. 1994 All Rights Reserved |  |  |

Press w and y to move around the selections.  Press e to accept the selection.  Press ^ to abort the selection or exit the utility.

## 3.12.1	  Select Drive

Select an installed hard disk drive (either C, D, E, or F).  The bottom of the screen displays the drive automatically detected by the utility.

## 3.12.2	  Bad Track List

The following options are available for this function:

- Auto scan bad track - Automatically scans and lists the bad tracks in the window located at the right side of the screen.

- Add bad track - Allows you to manually enter the information of the known bad track in the window located at the right side of the screen.

- Modify bad track - Allows you to modify the information of the added bad track in the window located at the right side of the screen.

- Delete bad track - Deletes the added bad tracks.

- Clear bad track table - Clears the whole bad track list in the window located at the right side of the screen.

## 3.12.3	  Preformat

The following options are available for this function:

- Interleave - Allows you to manually set the interleave number of the hard disk drive selected for low-level formatting.  You may enter any number from 1 to 8.  Before setting, check the documentation that comes with the drive for the correct interleave number, or select 0 for automatic detection.

- Auto scan bad track - Allows the utility to scan first then format.  Formatting is done by tracks.

- Start - Allows you to begin the low-level format by pressing Y.

# 3.13	Save & Exit Setup

This function automatically saves all CMOS values before leaving Setup.

# 3.14	Exit without Saving

Use this function to exit Setup without saving the CMOS value changes.  Do not use this option if you want to save the new configuration.

# 3.15	NCR SCSI BIOS and Drivers

The NCR 53C810 SCSI BIOS resides on the same flash memory chip as the system BIOS.  To use the onboard NCR BIOS, you need to install an NCR 53C810 SCSI controller card in your system.

All SCSI devices that you install in your system require software drivers.  The NCR SCSI BIOS directly supports SCSI hard disks under DOS, Windows and OS/2.  It also uses DOS-format and SCO UNIX-format support floppy disk device drivers that come with the NCR 53C810 SCSI controller card.  The DOS-format device drivers are for SCSI devices used with DOS, Windows NT, Novell NetWare and OS/2.  The SCO UNIX-format device drivers are for SCSI devices used with SCO UNIX.  These drivers offer higher performance than the direct BIOS support.

To use the device drivers, you must install them in your system hard disk drive and add them to your system configuration files.  For detailed installation instructions, see the README files that come with the drivers.

|  | The system board also supports the Award Flash Memory Writer Utility which allows you to upgrade the system BIOS. For more information on this utility, contact your local distributor. |
| --- | --- |