May 1993

3Com 3+ Share Driver Installation

for

D-Link DE-650 PCMCIA Ethernet Adapter

=======================================

The D-Link 3COM 3+ Share Driver Program lets you run 3Com 3+ Share

on your PC with the D-Link DE-650 PCMCIA Ethernet Adapter.

The following files will be used in this installation:

Files		 Contents

-----------	 -------------------------

DE650PD.SYS	 DE-650 Driver Program

ETH.SYS 	 3Com 3+ Share Driver

1. Follow the installation procedure for 3Com EtherLink board.

2. Copy the program ETH.SYS from the Installation Diskette for

the D-Link DE-650 PCMCIA Ethernet Adapter to the boot disk/diskette.

This program will replace the program of the same file name

(ETH.SYS) when you perform the installation procedure in step 1

above.

3. In the CONFIG.SYS file on the boot disk/diskette, make sure

you have the following statements.  The order of these

statements must be as shown below.

*** PC installed Socket and Card services ***

DEVICE = DE650PD.SYS <-d> <-w> <sw_int> <irq> <io_addr> <m_base>

DEVICE = ETH.SYS

OR

*** PC installed Socket service or Databook Cardtalk***

DEVICE = AE.EXE [/slt:P0] [/iop:P1] [/mem:P2] [/irq:P3]

DEVICE = DE650PD.SYS <-d> <-w> <sw_int> <irq> <io_addr> <m_base>

DEVICE = ETH.SYS

OR

*** PC's PCMCIA controller is Intel 82365SL ***

DEVICE = PCIC.EXE [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3] [/grp:P4]

DEVICE = DE650PD.SYS <-d> <-w> <sw_int> <irq> <io_addr> <m_base>

DEVICE = ETH.SYS

OR

*** PC is Toshiba T3300SL ***

DEVICE = ICCNT.EXE [/iop:P1] [/mem:P2] [/irq:P3]

DEVICE = DE650PD.SYS <-d> <-w> <sw_int> <irq> <io_addr> <m_base>

DEVICE = ETH.SYS

*** For Phoenix PCM ***

DEVICE= PCMSS.EXE

DEVICE= PCMCCU.EXE

DEVICE= PCMDLINK.COM (irq:5, mem:d600, io base:320, only needed PCMCCU

in v1.3)

Please refer to \OPEN_ADP\README.DOC or \install.doc for detail information.

You may now boot and start your 3COM workstation.

Notes:

1. Parameters for DE650PD.SYS

<-d>		Delay Initial - The driver program will not initialize

the network interface card until the packet driver program

is accessed by its upper layer program first time.

<-w>		the -w parameter is for use with the Windows environment only.

<sw_int>	is the software interrupt vector number used by the packet

driver program, which can be from 96 to 128.

<irq>		is the hardware interrupt (IRQ) line set on your network

interface card. The default is 5 if this parameter is not

specified.

<io_addr>	is base I/O address set on your network interface card.

The default address of 0x300 is used, if this parameter

is not specified.

<m_base>	is attribute memory base. The default value is 0xd000.

2. Parameters for OPEN-ADP utility:

PCIC.EXE

PCIC.EXE is used to enable Intel 82365SL PC Card Interface Controller

(PCIC) chipset.

ICCNT.EXE

ICCNET.EXE is used to enable ICCNT (I/O Card Controller) chipset in

TOSHIBA T3300SL Model Notebook.

AE.EXE

AE.EXE is used to enable PCMCIA Ethernet Card on which Socket Service has

been installed.

PCMDLINK.COM

PCMDLINK.COM is PCM client driver only for Phoenix PCMCCU v1.3 needed.

device=ae.exe [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3]

or

device=pcic.exe [/iop:P0] [/mem:P1] [/irq:P2] [/slt:P3] [/grp:P4]

or

device=iccnt.exe [/iop:P0] [/mem:P1] [/irq:P2]

device=pcmdlink.com (resources is fixed on irq:5, mem:d600, io base:320)

Parameter description:

/iop   :I/O base

/mem   :Attribute memory base

/irq   :IRQ value

/slt   :Slot type

/grp   :Group value

P0 :The selectable window I/O base with 0x20 range.

The default value is 0x300.

P1 :The selectable window memory base.

The default value is 0xd000.

P2 :The selectable irq value.

The default value is 0x5.

P3 :The selectable slot number.

The default number is 1.

P4 :The selectable grp value.

The default value is 0.

Group 0 means Index IO is 3E0h, Data IO is 3E1h, Base is 00h

Group 1 means Index IO is 3E0h, Data IO is 3E1h, Base is 80h

Group 2 means Index IO is 3E2h, Data IO is 3E3h, Base is 00h

Group 3 means Index IO is 3E2h, Data IO is 3E3h, Base is 80h