STANDARD MICROSYSTEMS CORPORATION

Novell NetWare 386 v4.01 Server Driver

For Ethernet LAN Adapters

INSTALLATION GUIDE

This software is licensed by SMC for use by its customers only.

Copyright (c) 1993 Standard Microsystems Corporation.

All rights reserved.

This installation guide describes:

o  How to use EZStart to copy the Novell NetWare 386 version 4.01

server driver from the SuperDisk to your server.

o  How to load the LAN driver and establish network connection.

o  Novell NetWare 386 v4.01 server driver error messages.

o  Novell NetWare 386 v4.01 server driver warning messages.

o  Novell NetWare 386 v4.01 server driver custom counters.

Note: This driver supports all 8216 (Ultra) and 80xx (PLUS) Series

ISA and Micro Channel Ethernet adapters. This driver also

supports the Novell 32-bit client specification, which allows

the driver to work with Windows NT, UnixWare, and other 32-bit

Intel-based operating systems. Contact Novell for more

information.

COPYING THE DRIVER FILE

From the EZStart Driver Installation screen, select 'Copy Files'

to copy all the files used by this driver from the SuperDisk to the

Destination Directory recommended by EZStart, or to a different

directory that you specify.

LOADING THE DRIVER

Installing the LAN driver is part of the NetWare installation

process. Use the following instructions with those

in the NetWare 386 installation manual to install the driver.

You will need the SuperDisk during this procedure.

1. Refer to the "Load LAN driver module" section in the Novell

NetWare 386 v4.01 installation manual for instructions on

installing NetWare.

2. Follow the instructions on starting NetWare and configuring

the server. Use the Load command to load the LAN driver.

If the LAN driver is not located in the default directory,

specify the path to the file on the command line. The

Load command will prompt you for configuration parameters

if they are not supplied on the command line.

The Load command line parameters may be saved in an

AUTOEXEC.NCF file, so they execute automatically every time

the server starts. Refer to the NetWare 386 installation

manual on how to create an AUTOEXEC.NCF file.

Driver Command Line Parameters

The following examples show various command line parameters:

SMC ISA adapters: The only required parameter is PORT.

Syntax: load C:SMC8000.LAN PORT=XXX <Enter>

(where XXX is the I/O address of the adapter, in

hexadecimal)

Example: load C:SMC8000.LAN PORT=280 <Enter>

All Micro Channel adapters: The only required parameter is SLOT.

Syntax: load C:SMC8000.LAN SLOT=X <Enter>

(where X is the PS/2 slot where the adapter is installed)

Example: load C:SMC8000.LAN SLOT=4 <Enter>

Western Digital adapters, which are not software configurable:

The required parameters are PORT, INT, and MEM.

Syntax: load C:SMC8000.LAN PORT=XXX INT=YY MEM=ZZZZZ:200 <Enter>

(where XXX is the I/O address of the adapter, in

hexadecimal; YY is the IRQ setting of the adapter, in

hexadecimal; and ZZZZZ is the RAM address of the

adapter, in hexadecimal)

Example: load C:SMC8000.LAN PORT=280 INT=3 MEM=D0000:200 <Enter>

Note: In the above example, the MEM=ZZZZZ: setting for a 16-bit

adapter would be 400 instead of 200.

3. Repeat step 2 if you are loading a driver for another LAN adapter

or an additional frame type.

Note: The driver has to be bound to start working (refer to the

section "Bind the protocol to the LAN driver" in the NetWare

386 installation manual).

NOVELL NetWare 386 v4.01 SERVER DRIVER ERROR MESSAGES

"Could not find an SMC Ethernet adapter at I/O base

address XXX"

The driver was unable to find an SMC Ethernet LAN adapter

at the specified base I/O address. Check for adapter

conflicts and retry the installation. Check that the adapter

is seated properly in adapter slot.

"Could not find an adapter at slot X (Micro Channel only)"

The driver was unable to find an SMC Ethernet LAN adapter

at the specified slot number. Check for adapter conflicts

and retry the installation. Check that the adapter is

seated properly in adapter slot.

"Adapter hardware failure detected (initialization)"

The adapter did not pass the initialization process.

Check the adapter for configuration conflicts.

"Adapter hardware failure detected (self-test)"

The adapter did not pass the initialization self-test

process due to a hardware problem. Check the adapter

for configuration conflicts.

"Cannot access adapter RAM properly"

The adapter could not read or write the adapter RAM. Verify

that another adapter does not share the same memory space as

the LAN adapter.

"Other system RAM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

other RAM in the system. Reconfigure the adapter to use

a different RAM address and retry the installation.

"Other system ROM already occupies adapter RAM"

The driver has detected that the adapter RAM conflicts with

ROM in the system. Reconfigure the adapter to use a

different RAM address and retry the installation.

"Adapter IRQ higher than 7 not valid in 8-bit slot"

The IRQ number specified cannot be used if the adapter

is inserted into an 8-bit slot. Select a different IRQ

number.

"Adapter unable to process a multicast address"

A node address was given on the command line or the board is

configured for a node address that is reserved for multicast

address. Broadcast and multicast addresses are not allowed

as valid node addresses. Use the node address over-ride

with caution as this can cause problems on your network.

"Configuration corrupted; run software setup"

The driver failed to initialize the adapter due to invalid

data in configuration EEROM. Run the software setup utility

and reconfigure the adapter.

NOVELL NetWare 386 v4.01 SERVER DRIVER WARNING MESSAGES

"Unneeded Memory Address ignored"

You specified an unnecessary memory address parameter when

loading the driver. A software-specified memory address will

be used instead.

"Unneeded Memory Length ignored"

You specified an unnecessary memory length parameter when

loading the driver. A software-specified memory address will

be used instead.

"Unneeded Interrupt number ignored"

You specified an unnecessary interrupt number parameter when

loading the driver. A software-specified interrupt number will

be used instead.

NOVELL NetWare 386 v4.01 SERVER DRIVER CUSTOM COUNTERS

You can use the MONITOR.NLM server utility to view these counters.

TxOKSingleCollision

The number of packets transmitted successfully that en-

countered only one collision during transmission. It is

normal for this counter to increment on busy networks.

TxOKMultipleCollisions

The number of packets transmitted successfully that en-

countered more than one collision during transmission.

It is normal for this counter to increment on busy networks.

TxOKButDeferred

The number of successfully transmitted packets that were

delayed because the medium was busy. This counter may

increment on a busy network.

TxAbortLateCollision

The number of collisions detected after the packet started

transmitting. The packet was not sent successfully due to

a collision during transmit. It is normal for this counter

to increment occasionally due to packet collisions. If

this counter increments frequently, it may indicate a

problem with another NIC adapter on the network.

TxAbortExcessCollisions

The number of packets that were not transmitted because the

maximum collision limit was exceeded (16 collisions). This

counter may occur occasionally if there is high network

traffic.

TxAbortCarrierSense

This counter indicates the number of times the carrier-sense

signal was not-asserted or was deasserted during transmission

of a packet without a collision. This counter may increment

occasionally during periods of high network traffic. If this

counter increments frequently, it may indicate that there is

a cabling problem or a problem with another LAN adapter on

the network.

RxAbortFrameAlignment

The counter indicates the number of packets that were rejected

to an alignment error. The length of the packet did not pass

the Frame Check Sequence (FCS) check. This counter may incre-

ment occasionally due to collisions occurring on a busy net-

work. If the counter increments frequently it may indicate a

problem with another LAN adapter on the network.

AdapterResetCount

The number of times the NIC was reset due to a transmit

timeout error or an unrecoverable error. This counter may

increment occasionally during normal network operation.

If this counter increments frequently, it may indicate that

there is a problem with the LAN adapter.

ReceiveLostPacketCount

This count contains the total number of packets that were

lost due to an overrun of receive buffers. This counter

may increment occasionally during periods of high network

traffic. If this counter increments frequently, it may

indicate a problem with the LAN adapter.

TransmitFIFOUnderrunCount

This counter indicates the total number of times that the

NIC could not access the bus before the FIFO buffer emptied.

Transmission of the packet is aborted when this event

happens. This counter may occasionally increment on a busy

network. If this counter increments frequently, it may

indicate a problem with the LAN adapter.

TransmitTimeoutCount

The number of times a transmit complete interrupt did not

occur within 2 seconds of starting a packet transmission.

When this occurs, the NIC is reset and an attempt is made

to re-transmit the packet. This counter may ocassionally

increment on a busy network. If this counter increments

frequently, it may indicate a problem with the network

cabling or the LAN adapter.

AdapterMiscErrorCount

This counter indicates a non-fatal error occurred. This

counter may increment occasionally during normal operation.

If this counter increments frequently, it may indicate a

problem with the LAN adapter.

SMC and Standard Microsystems are registered trademarks; and EtherCard

Elite 16 Ultra, EtherCard Elite 16T Ultra, EtherCard Elite 16C Ultra,

Elite Series, EliteView, EZStart, SuperDisk, and UltraChip are

trademarks of Standard Microsystems Corporation.  Other products

and company names are registered trademarks or trademarks of their

respective holders.