#

File: philips.info,  Node: Top,  Prev: (panasoni.info),  Next: (pioneer.info),  Up: (remotes.info)

The RC5 code:

-------------

The RC5 code is an 13-bit code, it used to have a 5-bits group and a

6-bits command, and 2 stop-bits, but it was changed to 5-bits group,

7-bits command and 1 stop-bit.

Old RC5:

data length: 14bit (2sync bits, 1 control bit, 11 data bits) 24,889ms

Carrier: 36kHz

No header.

+----+

1 is coded:      |    |

+----+    +

T    T

+----+

0 is coded: |    |

+    +----+

T    T

T = 889us

******************************************************************************

------------------------------------------------------------------------------

Commnd: Video mode: TV1 & TV2   Video mode: TXT         Video mode: VD

Group 0 (TV1) 1 (TV2)   Group 2                 Group 3

------------------------------------------------------------------------------

0     0                       0                       0

1     1                       1                       1

2     2                       2                       2

3     3                       3                       3

4     4                       4                       4

5     5                       5                       5

6     6                       6                       6

7     7                       7                       7

8     8                       8                       8

9     9                       9                       9

10     1/2/3 digs (japan:10)   step page +             =/= =/=

11     channel/program(jap:11) step page -             *

12     standby                 standby                 standby

13     mute/demute             mute/demute             mute/demute

14     P.P. (personal pref.)   P.P.                    P.P.

15     display                 display                 display

16     volume +                volume +                volume +

17     volume -                volume -                volume -

18     brightness +            brightness +            brightness +

19     brightness -            brightness -            brightness -

20     colour saturation +     colour saturation +     colour saturation +

21     colour saturation -     colour saturation -     colour saturation -

22     bass +                  bass +                  bass +

23     bass -                  bass -                  bass -

24     treble +                treble +                treble +

25     treble -                treble -                treble -

26     balance right           balance right           balance right

27     balance left            balance left            balance left

28     contrast +              enter/validation (Frnc)

29     contrast -              memory out

30     search +                sequence out            attribute on/off

31     tint/hue +

32     channel/program +       exchange

33     channel/program -       index/summary (France)  index

34     alt/channel             row zero (France)

35     1st/2nd language        1st/2nd language        1st/2nd language

36     spacial stereo on/off   spacial stereo on/off   spacial stereo on/off

37     mono/stereo (USA)

38     sleeptimer              hardcopy printer        hardcopy printer

39     tint/hue -              antiope TV mixed (Frnc)

40     RF switch/PIP select    PIP select              terminal initiation

41     vote                    page hold on/off

42     timed page/channel clck timed page (F:magazine)

43     increment (USA)         large top/lrge bot/norm large top/lrge bot/norm

44     decrement (USA)         reveal/conceal/canc pic

45                             cancel teletext picture

46     2nd controls (USA:menu) TV/Text mixed/Subtitle  TV/Text mixed

47     PIP shift               PIP shift               view data line off

48     pause                   pause                   pause

49     erase                   erase                   erase

50     rewind                  rewind                  rewind

51     go to                   go to                   go to

52     wind                    wind                    wind

53     play                    play                    play

54     stop                    stop                    stop

55     recording               recording               recording

56     external 1              external 1              external 1

57     external 2              external 2              external 2

58     PIP display mode        PIP display mode

59     view data mode/advance                          view data line on

60     teletext submode (J:12)

61     system standby          system standby          system standby

62     crispener on/off        news flash (France)

63     system select           system select           system select

64     surround sound          surround sound          surround sound

65     balance front           balance front           balance front

66     balance rear            balance rear            balance rear

67     sound effects           sound effects           sound effects

68     sound effects           sound effects           sound effects

69     sound effects           sound effects           sound effects

70     sound effects           sound effects           sound effects

71     sound effects           sound effects           sound effects

72     sound or menu func.     sound or menu func.     sound or menu func.

73     sound or menu func.     sound or menu func.     sound or menu func.

74     sound or menu func.     sound or menu func.     sound or menu func.

75

76

77     linear function up      linear function up      linear function up

78     linear function down    linear function down    linear function down

79     sound or menu func.     sound or menu func.     sound or menu func.

80     cursor step up          cursor step up          cursor step up

81     cursor step down        cursor step down        cursor step down

82     menu on                 menu on                 menu on

83     menu off                menu off                menu off

84     display A/V status      display A/V status      display A/V status

85     cursor step left        cursor step left        cursor step left

86     cursor step right       cursor step right       cursor step right

87     menu function           menu function           menu function

88     PIP on/off              PIP on/off              PIP on/off

89     PIP shift               PIP shift               PIP shift

90     PIP main swap           PIP main swap           PIP main swap

91     strobe on/off           strobe on/off           strobe on/off

92     multi strobe            multi strobe            multi strobe

93     main freezed            main freezed            main freezed

94     3/9 multi scan          3/9 multi scan          3/9 multi scan

95     PIP mode select         PIP mode select         PIP mode select

96     mosaic                  mosaic                  mosaic

97     solarization            solarization            solarization

98     main stored             main stored             main stored

99     PIP strobe              PIP strobe              PIP strobe

100     recall main             recall main             recall main

101     PIP freeze              PIP freeze              PIP freeze

102     PIP step up             PIP step up             PIP step up

103     PIP step down           PIP step down           PIP step down

104

105

106

107                             red

108                             green

109                             yellow

110                             cyan

111                             index

112

113

114

115

116

117

118     sub mode                sub mode                sub mode

119     sub mode                sub mode                sub mode

120

121

122     cleaning up             cleaning up             cleaning up

123     connect                 connect                 connect

124     disconnect              disconnect              disconnect

125     cleaning up             cleaning up             cleaning up

126     cleaning up             cleaning up             cleaning up

127     cleaning up             cleaning up             cleaning up

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Commnd: Video mode: LV1         Video mode: VCR1 & VCR2 Video mode: Satellite

Group 4                 Group 5 (VCR1) 6 (VCR2) Group 8

------------------------------------------------------------------------------

0     0                       0                       0

1     1                       1                       1

2     2                       2                       2

3     3                       3                       3

4     4                       4                       4

5     5                       5                       5

6     6                       6                       6

7     7                       7                       7

8     8                       8                       8

9     9                       9                       9

10     picture number/time     1/2 digits/AM/PM        1/2 digits

11     channel number          channel/program mode    channel/program mode

12     standby                 standby                 standby

13     mute/demute             mute/demute             mute/demute

14     P.P. (personal pref.)   P.P.                    P.P.

15     display                 display                 display

16     volume +                volume +                volume +

17     volume -                volume -                volume -

18     brightness +            brightness +            brightness +

19     brightness -            brightness -            brightness -

20     colour saturation +     colour saturation +     colour saturation +

21     colour saturation -     colour saturation -     colour saturation -

22     bass +                  bass +                  bass +

23     bass -                  bass -                  bass -

24     treble +                treble +                treble +

25     treble -                treble -                treble -

26     balance right           balance right           balance right

27     balance left            balance left            balance left

28     repeat once/TSPo        ITR (OTR) delay

29     repeat cont/TSPc        timer programming

30     next                    record select

31     fast run reverse        fast run reverse

32     entry                   step +                  channel/program +

33     auto stop/memory        step -                  channel/program -

34     slow run reverse        slow run reverse

35     audio 1                 audio output select     language select/mode 1

36     audio 2                 clock/channel           special stereo on/off

37     still picture           speed -                 audio mixed / mode 2

38     speed +                 speed +

39     speed -                 Instant Time Recording

40     slow run forward        slow run forward

41     still forward           still forward

42     fast run forward        fast run forward

43     search automatic        index scan

44     scan reverse            scan reverse

45     open/close (eject)      eject

46     scan forward            scan forward            subtitle

47     play reverse            play reverse

48     pause                   pause                   pause

49     erase                   erase                   erase

50     rewind                  rewind                  rewind

51     go to                   go to                   go to

52     wind                    wind                    wind

53     play                    play                    play

54     stop                    stop                    stop

55     recording               recording               recording

56     external 1              external 1              external 1

57     external 2              external 2              external 2

58     clear all memory        short/long play

59     freeze segment          counter memory

60     TXT (pal)/ant.sw (ntsc) scroll local display    TXT submode

61     system standby          system standby          system standby

62     CX on/off               RF switch

63     system select           system select           system select

64     surround sound          surround sound          surround sound

65     balance front           balance front           balance front

66     balance rear            balance rear            balance rear

67     sound effects           sound effects           sound effects

68     sound effects           sound effects           sound effects

69     sound effects           sound effects           sound effects

70     sound effects           sound effects           sound effects

71     sound effects           sound effects           sound effects

72     sound or menu func.     sound or menu func.     sound or menu func.

73     sound or menu func.     sound or menu func.     sound or menu func.

74     sound or menu func.     sound or menu func.     sound or menu func.

75                             data stream start

76                             data stream end

77     linear function up      linear function up      linear function up

78     linear function down    linear function down    linear function down

79     sound or menu func.     catalogue               sound or menu func.

80     cursor step up          cursor step up          cursor step up

81     cursor step down        cursor step down        cursor step down

82     menu on                 menu on                 menu on

83     menu off                menu off                menu off

84     display A/V status      display A/V status      display A/V status

85     cursor step left        cursor step left        cursor step left

86     cursor step right       cursor step right       cursor step right

87     menu function           acknowledge             menu function

88     PIP on/off              PIP on/off              PIP on/off

89     PIP shift               PIP shift               PIP shift

90     PIP main swap           PIP main swap           PIP main swap

91     strobe on/off           strobe on/off           strobe on/off

92     multi strobe            multi strobe            multi strobe

93     main freezed            main freezed            main freezed

94     3/9 multi scan          3/9 multi scan          3/9 multi scan

95     PIP mode select         PIP mode select         PIP mode select

96     mosaic                  mosaic                  mosaic

97     solarization            solarization            solarization

98     main stored             main stored             main stored

99     PIP strobe              PIP strobe              PIP strobe

100     recall main             recall main             recall main

101     PIP freeze              PIP freeze              PIP freeze

102     PIP step up             PIP step up             PIP step up

103     PIP step down           PIP step down           PIP step down

104                             automatic start ID

105                             write skip ID

106                             skip on/off

107                             write end ID

108                             write FTS

109                             write TOC

110

111

112                             viss index next

113                             viss index previous

114                             mark

115                             erase

116                             renumber

117                             blank search

118     sub mode                sub mode                TV1

119     sub mode                sub mode                sub mode

120                             auto repeat

121                             insert temp

122                             scr loc disp

123     connect                 connect                 connect

124     disconnect

125

126

127

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Commnd: Audio mode: LV2/CDLV    Audio mode: Pre-Amp     Audio mode: Tuner

Group 12                Group 16 (#1) 19 (#2)   Group 17

------------------------------------------------------------------------------

0     0                       0                       0

1     1                       1                       1

2     2                       2                       2

3     3                       3                       3

4     4                       4                       4

5     5                       5                       5

6     6                       6                       6

7     7                       7                       7

8     8                       8                       8

9     9                       9                       9

10     picture number/time     graphic equalizer left  1/2 digits

11     chapter number          graphic equalizer right

12     standby                 standby                 standby

13     mute/demute             mute/demute             mute/demute

14     P.P. (personal pref.)   P.P.                    P.P.

15     display                 display                 display

16     volume +                volume +                volume +

17     volume -                volume -                volume -

18     brightness +            brightness +            brightness +

19     brightness -            brightness -            brightness -

20     colour saturation +     colour saturation +     colour saturation +

21     colour saturation -     colour saturation -     colour saturation -

22     bass +                  bass +                  bass +

23     bass -                  bass -                  bass -

24     treble +                treble +                treble +

25     treble -                treble -                treble -

26     balance right           balance right           balance right

27     balance left            balance left            balance left

28     repeat once/TSPo        graph equal left&right

29     repeat cont/TSPc        loudness

30     next                    high tone filter        search +

31     programming             low tone filter         search -

32                             step +                  preset +

33     auto stop/memory        step -                  preset -

34     slow run reverse        signal path scroll

35     stereo left/right

36     previous

37     still reverse           surround mode           mono/stereo

38     speed +                 sleeptimer

39     speed -                 balance rear

40     slow run forward        balance front           RF switch

41     still forward           TPS                     store execute

42     digital multi screen    timer set

43     strobe                  timer up                scan forward

44     scan reverse            timer down

45     open/close (eject)      timer memo              FM

46     scan forward            acoustic memory         MW (USA: MW/FM)

47                             acoustic memory select  LW

48     pause                   pause                   pause

49     erase                   erase                   erase

50     rewind                  rewind                  rewind

51     go to                   go to                   go to

52     wind                    wind                    wind

53     play                    play                    play

54     stop                    stop                    stop

55     recording               recording               recording

56     external 1              external 1              external 1

57     external 2              external 2              external 2

58     clear all memory        clear all memory        clear all memory

59     repeat A/B program

60     TXT (pal)/ant.sw (ntsc) dyn range expand/compr

61     system standby          system standby          system standby

62     CX on/off

63     system select           system select           system select

64     surround sound          surround sound          surround sound

65     balance front           balance front           balance front

66     balance rear            balance rear            balance rear

67     sound effects           sound effects           sound effects

68     sound effects           sound effects           sound effects

69     sound effects           sound effects           sound effects

70     sound effects           sound effects           sound effects

71     sound effects           sound effects           sound effects

72     sound or menu func.     sound or menu func.     sound or menu func.

73     sound or menu func.     sound or menu func.     sound or menu func.

74     sound or menu func.     sound or menu func.     sound or menu func.

75

76

77     linear function up      linear function up      linear function up

78     linear function down    linear function down    linear function down

79     sound or menu func.     catalogue               sound or menu func.

80     cursor step up          cursor step up          cursor step up

81     cursor step down        cursor step down        cursor step down

82     menu on                 menu on                 menu on

83     menu off                menu off                menu off

84     display A/V status      display A/V status      display A/V status

85     cursor step left        cursor step left        cursor step left

86     cursor step right       cursor step right       cursor step right

87     menu function           menu function           menu function

88     PIP on/off              PIP on/off              PIP on/off

89     PIP shift               PIP shift               PIP shift

90     PIP main swap           PIP main swap           PIP main swap

91     strobe on/off           strobe on/off           strobe on/off

92     multi strobe            multi strobe            multi strobe

93     main freezed            main freezed            main freezed

94     3/9 multi scan          3/9 multi scan          3/9 multi scan

95     PIP mode select         PIP mode select         PIP mode select

96     mosaic                  mosaic                  mosaic

97     solarization            solarization            solarization

98     main stored             main stored             main stored

99     PIP strobe              PIP strobe              PIP strobe

100     recall main             recall main             recall main

101     PIP freeze              PIP freeze              PIP freeze

102     PIP step up             PIP step up             PIP step up

103     PIP step down           PIP step down           PIP step down

104

105

106

107

108     FTS

109

110

111

112     index next

113     index previous

114

115

116

117

118     sub mode

119     sub mode

120

121

122

123     connect                 connect                 connect

124     disconnect              disconnect              disconnect

125

126

127

------------------------------------------------------------------------------

------------------------------------------------------------------------------

Commnd: Audio mode: CD          Audio mode: Phono       Audio mode: Recorder

Group 20                Group 21                Group 18 (#1) 23 (#2)

------------------------------------------------------------------------------

0     0                       0                       0

1     1                       1                       1

2     2                       2                       2

3     3                       3                       3

4     4                       4                       4

5     5                       5                       5

6     6                       6                       6

7     7                       7                       7

8     8                       8                       8

9     9                       9                       9

10     cursor scroll

11     display scroll

12     standby                 standby                 standby

13     mute/demute             mute/demute             mute/demute

14     P.P. (personal pref.)   P.P.                    P.P.

15     display                 display                 display

16     volume +                volume +                volume +

17     volume -                volume -                volume -

18     brightness +            brightness +            brightness +

19     brightness -            brightness -            brightness -

20     colour saturation +     colour saturation +     colour saturation +

21     colour saturation -     colour saturation -     colour saturation -

22     bass +                  bass +                  bass +

23     bass -                  bass -                  bass -

24     treble +                treble +                treble +

25     treble -                treble -                treble -

26     balance right           balance right           balance right

27     balance left            balance left            balance left

28     rept once/TSPo/shuffle  repeat once             repeat once

29     repeat cont/TSPc        repeat continuously     repeat continuously

30     select +                select +                select +

31     select -                select -                select -

32     next                    next                    next

33     previous                previous                previous

34     index next              index next

35     index previous          index previous          last record cancel

36     play/program            stereo/mono             sequential

37     speed nominal                                   blank search

38     speed +

39     speed -

40                                                     recording pause

41     store execute           store execute           store execute

42                                                     recording blank

43     scan forward            scan forward            scan forward

44                                                     mechanism A

45     open/close              open/close              eject

46                             wave scroll             mechanism B

47                                                     play reverse

48     pause                   pause                   pause

49     erase                   erase                   erase

50     rewind                  rewind                  rewind

51     go to                   go to                   go to

52     wind                    wind                    wind

53     play                    play                    play

54     stop                    stop                    stop

55     recording               recording               recording

56     external 1              external 1              external 1

57     external 2              external 2              external 2

58     clear all memory        clear all memory        clear all memory

59     repeat A/B program

60     dynamic range expand

61     system standby          system standby          system standby

62     dynamic range compress                          counter memory

63     system select           system select           system select

64     surround sound          surround sound          surround sound

65     balance front           balance front           balance front

66     balance rear            balance rear            balance rear

67     sound effects           sound effects           sound effects

68     sound effects           sound effects           sound effects

69     sound effects           sound effects           sound effects

70     sound effects           sound effects           sound effects

71     sound effects           sound effects           sound effects

72     sound or menu func.     sound or menu func.     sound or menu func.

73     sound or menu func.     sound or menu func.     sound or menu func.

74     sound or menu func.     sound or menu func.     sound or menu func.

75

76

77     linear function up      linear function up      linear function up

78     linear function down    linear function down    linear function down

79     sound or menu func.     sound or menu func.     sound or menu func.

80     cursor step up          cursor step up          cursor step up

81     cursor step down        cursor step down        cursor step down

82     menu on                 menu on                 menu on

83     menu off                menu off                menu off

84     display A/V status      display A/V status      display A/V status

85     cursor step left        cursor step left        cursor step left

86     cursor step right       cursor step right       cursor step right

87     menu function           menu function           menu function

88     PIP on/off              PIP on/off              PIP on/off

89     PIP shift               PIP shift               PIP shift

90     PIP main swap           PIP main swap           PIP main swap

91     strobe on/off           strobe on/off           strobe on/off

92     multi strobe            multi strobe            multi strobe

93     main freezed            main freezed            main freezed

94     3/9 multi scan          3/9 multi scan          3/9 multi scan

95     PIP mode select         PIP mode select         PIP mode select

96     mosaic                  mosaic                  mosaic

97     solarization            solarization            solarization

98     main stored             main stored             main stored

99     PIP strobe              PIP strobe              PIP strobe

100     recall main             recall main             recall main

101     PIP freeze              PIP freeze              PIP freeze

102     PIP step up             PIP step up             PIP step up

103     PIP step down           PIP step down           PIP step down

104                                                     automatic start ID

105                                                     write skip ID

106                                                     skip on/off

107                                                     write end ID

108     FTS                                             write FTS

109                                                     write TOC

110

111

112

113

114                                                     write start ID

115                                                     Ca/Er/SK ID

116                                                     reformat

117                                                     blank search

118     sub mode                sub mode                sub mode

119     sub mode                sub mode                sub mode

120

121

122

123     connect                 connect                 connect

124     disconnect              disconnect              disconnect

125

126

127

------------------------------------------------------------------------------

The codes and parts of the RC5REMOTE program are copied from

the article "Remote Control for HP28S" written by

Michiel Niemeijer & Eric Toonen.

Bj�rn Gahm   -- Email: d89-bga@nada.kth.se

Snail: �ppelstigen 2

181 62 LIDING�

SWEDEN