--------- XYSRVR Version 0.91 Beta - (C) Double Delta Industries ---------

This is a beta release update of XYSrvr for MSDOS. I would appreciate any

problem reports or suggestions that may arise from the use of this

software. This software has only been proven reliable when using the HP48

internal XSEND/XRECV (GX rev. R) commands, the HP FXRECV program from

Goodies Disk #9, the XMODEM2.GX Library 995 and X-Ymodem 2.2 Library 1275

(SX only) on Goodies Disk #9 (Not yet tested for PC->HP48 Ymodem transfers

with the new Ymodem-128 option). Maybe someone with an SX can test out the

X-YMODEM.LIB Ymodem functionality and email the results to me. If you have

or know of any other HP48 file transfer programs, e-mail them to me at

diamond@eskimo.com or post it and let me know by e-mail where to look for

it (Let me know if it is GX hostile if you can). I will most likely see it

if you just post to comp.sys.hp48. I will test any GX compatible programs

and post the results to comp.sys.hp48. If you are using an SX only program

please test it and e-mail the results along with your hardware

configuration. Please review the installation information in the

documentation. If there is anyone out in MAC or Unix land that would like

to take a crack at porting XYsrvr to your platforms, I can support you with

pascal source code for the X/Ymodem and remote command processing routines

(as these are probably the only parts that would be useful to you).

The MSDOS XYSRVR (XModem/YModem server) was developed to provide faster

file transfers between the PC and HP48 calculators. The program can also

serve as a dumb terminal to a serial device. In Server mode it provides

remote file transfer functions similar to Kermit. The program is released

as SHAREWARE so please register it if you use it, hey it's only $20 US. So

if you regularly use this program and you want to get rid of that annoying

little reminder message then fill out the order form (ORDER.FRM) and send

the bucks to the address listed below. I will send you a non-annoying

latest revision on the media of your choice (Please specify). Updates are

$5 US, outside US add $2 and Washington State residents add 8.6% sales tax.

XYSRVR was developed with the following commercial products:

Turbo Pascal 5.5         ==> by Borland International

Technojock Turbo Toolkit ==> by The Technojock, Bob Ainsbury, Houston

Jim Daniels (diamond@eskimo.com)

1707 Jones Ave N.E.

Renton, Wa 98056

---------------------------  Change Log  --------------------------------

Version 0.91 - 4/26/94

1. Fixed problem with truncated filenames in the file browser. Will now

accept pathnames up to 80 characters in length.

2. Added configuration file option and modified command line option.

Now takes 0 or 1 command line parameter (<comport>).

3. Added Quiet Status configuration parameter for config file.

Eliminates the display of non-error status messages.

4. Added a Ymodem-128 option to be compatible with X-Ymodem 2.2 and

probably any other Ymodem program on the HP48. The Ymodem-128 option

only impacts PC->HP48 transfers as the Ymodem receive function auto

detects the block size.

Version 0.91 - 4/??/94

-------------------------------------------------------------------------

User Guide

Table of Contents

1.0  Installation and Startup

1.1  The XYSrvr Program

1.2  Your HP48

2.0  Operation

3.0  Pull Down Menu

3.1  File Menu

3.1.1  Receive

3.1.2  Transmit

3.1.3  Erase

3.1.4  File Mask

3.1.5  Directory

3.1.6  Change dir

3.1.7  About XYSrvr

3.2  Com Menu

3.3  Method Menu

3.4  Emulate Menu

3.5  Baud Menu

3.6  Length Menu

3.7  Parity Menu

3.8  Quit

4.0  Remote Commands

5.0  Configuration File

6.0  Compatibility

1.0  Installation and Startup

1.1  The XYSrvr Program

Copy XYSRVR.EXE to a directory of your choice. The program is totally self

contained so you can put it in a directory in your search path or whatever.

If XYSrvr is in your current directory or somewhere in your search path the

command line to execute XYSrvr would be:

XYSRVR [<ComPort>]

Otherwise:

[Drive:\Path\]XYSRVR [<ComPort>]

The available options for the one optional command line parameter are:

ComPort  ==> 1 or 2

XYSrvr will open with a credits screen. The Screen will time out in 5

seconds or just press any key to terminate it.

If the ComPort is not specified on the command line XYSrvr starts up with

the default settings referenced in section 2.0. A configuration file for

XYSrvr can also be created and used to set all the startup options to a

custom configuration. See section 5.0 for configuration commands. The

configuration file must be named XYSRVR.CFG. If you run XYSrvr from a

directory other than where XYSRVR.EXE is located then XYSRVR.CFG must be

somewhere in your search path. If XYSRVR.CFG is found the command line is

ignored.

1.2  Your HP48

Two variables in IOPAR can eliminate synchronization problems with XYSrvr

and other PC Xmodem programs. Go to your HOME directory and look for IOPAR.

If it is not in HOME then you will need to create it. After creating IOPAR

press the IOPAR menu key in the HOME directory. You should see something

like this:

{ 9600 0 0 0 3 3 }

^   ^ ^ ^ ^ ^

|   | | | | |

1. Baud Rate        --------------------|   | | | | |

2. Parity           ------------------------| | | | |

3. Receive-Pacing   --------------------------| | | |

4. Transmit-Pacing  ----------------------------| | |

5. Checksum         ------------------------------| |

6. Translate-code   --------------------------------|

The parameters of interest for Xmodem transfers are numbers 3 and 4. These

normally are defaulted to 0 which disables XON/XOFF handshaking. Edit the

IOPAR list and change both these parameters to a 1. Given the above example

you should end up with this:

{ 9600 0 1 1 3 3 }

STO this back into IOPAR. If you are experiencing sync, framing or timeout

errors during PC -> HP48 transfers then these values are probably not set

properly. I don't know of a way to set these without directly editing or

creating IOPAR. The second example of IOPAR should work with the default

XYSrvr settings if executed with the command line:

XYSRVR [<Comport>]

^

|----- This optional parameter could be a

1 or 2 depending on your hardware.

2.0  Operation

The XYSrvr screen is divided into two sections, the terminal window and the

status line. All Serial I/O communications are echoed to the terminal

window except for X/Ymodem file transfers. If one of the modes with Line

Feeds is selected then all <CR> will have a <LF> added when sent to the

terminal window. The actual serial data will be unaltered.

The status line (bottom line on the screen) is used to display XYSrvr

settings and to display X/Ymodem progress information. The status line will

look something like this:

------------------------------------------------------------------------------

Transfer Progress Message Area         COM2 X1C TL 9600 8 1 N  "ESC for Menu"

------------------------------------------------------------------------------

^  ^   ^^  ^   ^ ^ ^         ^

COM Port                -----------------|  |   ||  |   | | |         |

|   ||  |   | | |         |

File Transfer Protocol  --------------------|   ||  |   | | |         |

Screen Mode             ------------------------||  |   | | |         |

Line Feed Mode          -------------------------|  |   | | |         |

Baud Rate               ----------------------------|   | | |         |

Word Length             --------------------------------| | |         |

Number of Stop Bits     ----------------------------------| |         |

Parity                  ------------------------------------|         |

Menu Reminder           ----------------------------------------------|

The following is a list of all currently supported parameters:

| Parameter            | Options          | Display | Default |

|----------------------|------------------|---------|---------|

| COM Port             | 1                | COM1    |    X    |

|                      | 2                | COM2    |         |

| File Transfer Method | Xmodem           | X       |    X    |

|                      | Xmodem-1k        | X1      |         |

|                      | Ymodem-128       | Y C     |         |

|                      | Ymodem-1k        | Y1C     |         |

|                      | Xmodem CRC       | X C     |         |

|                      | Xmodem-1k CRC    | X1C     |         |

| Screen Mode          | Terminal with LF | TL      |         |

|                      | Server with LF   | SL      |    X    |

|                      | Terminal w/o LF  | T       |         |

|                      | Server w/o LF    | S       |         |

| Baud Rate            | 1200             | 1200    |         |

|                      | 2400             | 2400    |         |

|                      | 4800             | 4800    |         |

|                      | 9600             | 9600    |    X    |

| Word Length          | 5                | 5       |         |

|                      | 6                | 6       |         |

|                      | 7                | 7       |         |

|                      | 8                | 8       |    X    |

| Number of Stop Bits  | 1                | 1       |    X    |

|                      | 2                | 2       |         |

| Parity               | None             | N       |    X    |

|                      | Odd              | O       |         |

|                      | Even             | E       |         |

|                      | Space            | S       |         |

|                      | Mark             | M       |         |

| Menu Reminder        | No options       |         |         |

|------------------------------------------------------------|

3.0  Pull Down Menu

The pull down menu is accessed through the ESC key. The main pull down menu

will appear at the top of the terminal window. With the exception of the

File menu selection on the left side of the menu the menu choices are in the

same left to right order as the status line indicators which they control.

The menu is always available except when the Server is processing file

transfers.

3.1  File Menu

3.1.1  Receive

Receives a file in the current protocol and line settings. If Ymodem is the

current protocol XYSrvr will immediately enter the Ymodem batch receive

loop. This is a true Ymodem and the filename/size header blocks must be

provided by the sending device Ymodem protocol. None of the currently

available Ymodem HP48 programs that were available for testing supported

true Ymodem. The are in reality Xmodem-1k protocols. All Xmodem selections

will prompt you for a filename before executing the protocol. The filename

entry window allows you to browse your directories for a filename to

overwrite (Press enter without typing any characters in the filename

window). Press <F1> for more help when in the filename entry window.

3.1.2  Transmit

Send a file in the current protocol and line settings. If Ymodem is the

current protocol XYSrvr will not support batch mode. Only single file

transfers are allowed. This is a true Ymodem and the filename/size header

blocks are sent and must be accepted by the receiving device Ymodem

protocol. None of the currently available Ymodem HP48 programs that were

available for testing supported true Ymodem. They are in reality Xmodem-1k

protocols. All Xmodem selections will prompt you for a filename before

executing the protocol. The filename entry window allows you to browse your

directories for a filename to transmit (Press enter without typing any

characters in the filename window). Press <F1> for more help when in the

filename entry window.

3.1.3  Erase

Erase a file on your disk. Pops up a filename entry window. Enter a name or

browse with a blank entry (Press enter without typing any characters in the

filename window). Press <F1> for more help when in the filename entry

window. XYSrvr will ask you to verify your erase operation.

3.1.4  File Mask

Enter a file search mask for subsequent file browsing or directory list.

Defaults to "*.*".

3.1.5  Directory

Directory list browser. What you get when you browse a filename entry window.

3.1.6  Change dir

Pops up a filename entry window. Enter a new filepath or browse the current

drive (you should know how by now).

3.1.7  About XYSrvr

Redisplays the opening screen but with a 30 second timeout.

3.2  Com Menu

Select COM Port.

3.3  Method Menu

Select file transfer method option. The default for use with HP48

transfers is Xmodem. See Remote Operations section 4.0 for more

information. Ymodem protocols default to CRC error checking. Xmodem

protocols can be set to CRC or Checksum.

3.4  Emulate Menu

Select terminal mode, server mode,

Line Feed mode or clear the screen.

3.5  Baud Menu

Select baud rate.

3.6  Length Menu

Select word length and stop bit parameters.

3.7  Parity Menu

Select parity.

3.8  Quit

What more can I say!

4.0  Remote Commands

When Server mode is active XYSrvr acts like an X/Ymodem version of Kermit.

XYSrvr accepts command strings terminated in a <CR> ($0D or #13) or <LF>

($0A, #10 or the HP48G(X) rshift-period) but not both. If a command is sent

with CR/LF as a terminator the command will process fine but a status error

message will report a syntax error on the second terminator. A command is

composed of 2 or 3 words. A word is one or more characters separated by

space(s). The HP48 program to send an object named 'TEST' to the PC would

look like this:

Note : The ASCII carriage return character ($0D) is represented by the <CR>

symbol and can be replaced by <LF> (HP48G(X) rshift-period).            |

|

|-------------------------------------------------|

|

\<< "X R TEST<CR>" XMIT DROP 'TEST' XSEND \>> @ (  ->  )

A small program to simulate a non-list processing Xmodem version of the HP48

SEND command might look like this:

@ ( 'name' ->      )

\<< \-> name                       @ create ob id lam

\<< "X R "                       @ first two command words

name                        @ object id

#5BE9h SYSEVAL              @ convert to a string

"<CR>"                      @ ASCII #13 in a string

+ +                         @ concatenate 3 strings into 1

IF XMIT 0 > THEN            @ send the command

name XSEND                @ if no error then XSEND the object

ELSE                        @ else

"XYSsrvr Transmit Error"  @ leave an error message on the stack

END

\>>

\>>

I will be developing several HP48 programs to enhance the usability of

XYSrvr. I will include these in future releases XYSrvr and post them to the

most common archives and comp.sys.hp48.

The following is a list of all currently supported remote commands:

(Note: The commands are order specific. Word-1 then Word-2 then Word-3)

| Description                          | Word-1 | Word-2 | Word-3   |

|--------------------------------------|--------|--------|----------|

| Send file to PC Xmodem               | X      | R      | Filename |

| Send file to PC Xmodem-1k            | X1     | R      | Filename |

| Send file to PC Ymodem-128           | Y      | R      |          |

| Send file to PC Ymodem-1k            | Y1     | R      | Filename |

| Send file to PC Xmodem CRC           | XC     | R      | Filename |

| Send file to PC Xmodem-1k CRC        | X1C    | R      |          |

| Receive file from PC Xmodem          | X      | T      | Filename |

| Receive file from PC Xmodem-1k       | X1     | T      | Filename |

| Receive file from PC Ymodem-128      | Y      | T      | Filename |

| Receive file from PC Ymodem-1k       | Y1     | T      | Filename |

| Receive file from PC Xmodem CRC      | XC     | T      | Filename |

| Receive file from PC Xmodem-1k CRC   | X1C    | T      | Filename |

| Get short PC file list Xmodem        | X      | D      | S        |

| Get short PC file list Xmodem-1k     | X1     | D      | S        |

| Get short PC file list Ymodem-128    | Y      | D      | S        |

| Get short PC file list Ymodem-1k     | Y1     | D      | S        |

| Get short PC file list Xmodem CRC    | XC     | D      | S        |

| Get short PC file list Xmodem-1k CRC | X1C    | D      | S        |

| Get long PC file list Xmodem         | X      | D      | L        |

| Get long PC file list Xmodem-1k      | X1     | D      | L        |

| Get long PC file list Ymodem-128     | Y      | D      | L        |

| Get long PC file list Ymodem-1k      | Y1     | D      | L        |

| Get long PC file list Xmodem CRC     | XC     | D      | L        |

| Get long PC file list Xmodem-1k CRC  | X1C    | D      | L        |

|-------------------------------------------------------------------|

The Ymodem-128 option was added to support HP48 128 byte block transfers to

prevent buffer overflow. Ymodem-128 method selection only impacts PC->HP48

transfers. Both Ymodem-128 and Ymodem-1k auto-detect block sizes on

HP48->PC transfers.

The difference between a short and long file list is that the short list

only contains filenames and the long list contains filenames and all file

attributes.

5.0  Configuration File

A configuration file named XYSRVR.CFG can be used to set all your startup

options for XYSrvr. XYSrvr will search the current directory and all

directories in listed in the PATH environment variable. Any option not

specified or soecified incorrectly in XYSRVR.CFG will default to the values

listed in the table in section 2.0.

Configuration commands are composed of lines of command words, one option

per line and two or three command words seperated by spaces per command

line. A typical XYSRVR.CFG might look like this:

COM 2

METHOD X1C

EMULATE SERVER LF

BAUD 9600

LENGTH 8

STOP 1

PARITY NONE

QUIET STATUS

Most command words can be abbreviated to their first character. The above

example could also look like this:

C 2

M X1C

E S L

B 9

L 8

S 1

P N

Q S

The only exception in this release version is the METHOD qualifier. This

follows the same convention and syntax as for remote command Word-1 listed in

section 4.0.

The following is a list of all possible configuration commands:

| Description                | Word-1  |  Word-2  | Word-3

|----------------------------|---------|----------|--------|

| Com Port 1                 | COM     | 1        |        |

| Com Port 2                 | COM     | 2        |        |

| Xmodem                     | METHOD  | X        |        |

| Xmodem CRC                 | METHOD  | XC       |        |

| Xmodem-1k                  | METHOD  | X1       |        |

| Xmodem-1k CRC              | METHOD  | X1C      |        |

| Ymodem-128                 | METHOD  | Y        |        |

| Ymodem-1k                  | METHOD  | Y1       |        |

| Server w/o LF              | EMULATE | SERVER   |        |

| Server with LF             | EMULATE | SERVER   | LF     |

| Terminal w/o LF            | EMULATE | TERMINAL |        |

| Terminal with LF           | EMULATE | TERMINAL | LF     |

| 1200 Baud                  | BAUD    | 1200     |        |

| 2400 Baud                  | BAUD    | 2400     |        |

| 2800 Baud                  | BAUD    | 4800     |        |

| 9600 Baud                  | BAUD    | 9600     |        |

| 5 Data Bits                | LENGTH  | 5        |        |

| 6 data Bits                | LENGTH  | 6        |        |

| 7 Data Bits                | LENGTH  | 7        |        |

| 8 Dtat Bits                | LENGTH  | 8        |        |

| 1 Stop Bit                 | STOP    | 1        |        |

| 2 Stop Bits                | STOP    | 2        |        |

| No Parity                  | PARITY  | NONE     |        |

| Odd Parity                 | PARITY  | ODD      |        |

| Even Parity                | PARITY  | EVEN     |        |

| Space Parity               | PARITY  | SPACE    |        |

| Mark Parity                | PARITY  | MARK     |        |

| Disable non-error messages | QUIET   | STATUS   |        |

|----------------------------------------------------------|

The QUIET STATUS option can only be set from the configuration file. There

is no equivilent command from the XYSrvr menu and there is no status line

indication. It also does not disable the annoying registration reminder,

there is no command line or config file command to this. You should have

guessed it wouldn't be that easy.

6.0  Compatibility

To date the following list of hardware and software configurations have

been tested. If you have tested different hardware/software configurations

please e-mail me your results. Transfers at 9600 baud on the following

lists of hardware and software have worked without errors:

| Computer               | Calculator      | Software                        |

|------------------------|-----------------|---------------------------------|

| 66MHz 486DX2 PC        | HP48 GX Rev. R  | HP48 internal XSEND             |

|                        |                 | HP48 internal XRECV             |

|                        |                 | HP FXRECV                       |

|                        |                 | XMODEM2.GX Lib 995              |

|                        |                 | ( Use Xmodem Method )           |

| 25MHz 386SX  PC        | HP48 GX Rev. R  | HP48 internal XSEND             |

|                        |                 | HP48 internal XRECV             |

|                        |                 | HP FXRECV                       |

|                        |                 | XMODEM2.GX Lib 995              |

|                        |                 | ( Use Xmodem Method )           |

| 486 DX                 | HP48 SX Rev. E  | XYMmodem 2.04 Beta Lib 1275     |

|                        |                 | (Xmodem and HP->PC Ymodem only) |

|                        |                 | (Needs Ymodem-128 option for    |

|                        |                 |  PC->HP48 not yet tested)       |

|----------------------------------------------------------------------------|