Copyright (c) 1995 Bud Moss

All rights reserved

This file contains the article published in the March, 1995,

issue of The Computer Applications Journal.  The figures

mentioned would require excessive disk space and are not

included.  The graphs referenced in the fuzzy logic section are

the same as the program's tutorial (the names may be slightly

different).

If you do not have access to the magazine and want a copy of

all the figures, send $2.00 and a self addressed stamped envelope

to the address specified in the manual.  I'll also include my

other article and figures published in the November, 1994 issue

of MicroComputer Journal.

Fuzzy Logic - Evolution Or Hype?

Fuzzy logic is another step toward eliminating the need to

reduce our thoughts to the point that they become unrecognizable.

It allows us to design the core software of a control system or

similar task using a method much closer to the way we think.

It's no longer necessary to break down our ideas into line after

line of code or long and complex equations.  Instead, we use

graphs and shapes.  A few IF/THEN rules complete the system.

Fuzzy logic is not for everyone, just most of us.  If you

enjoy spending days generating and optimizing equations then days

more to implement and debug the system, fuzzy may not be for you.

For the rest of us, it's well worth the effort.

ABSOLUTES VERSUS GENERALIZATION

Have you ever been hiking and had to cross a stream but

there wasn't a bridge?  Instead of searching for a crossing, most

of us looked for another way.  Scanning the water, we saw the

rocks could be used as stepping stones.  The first thing - pick

the right ones.  If we happened to pack our protractor and ruler

we could jump into the water and document every characteristic of

every rock.  Next, we could generate a formula, plug the data

into a calculator, and hope.  Two or three days later, the stream

would be conquered and we'd be on our way.  Not very likely.

Instead, we based our decision on approximations.  First we

decided which characteristics should be considered and how

important they were compared to the others.  We knew that rocks

with a large flat top would be easier to stand on and the closer

together they were the better.  We also knew that the distance

apart was more important than the shape of the top.  After

finding how well each rock fit our categories, we chose the right

one.  That's what fuzzy logic does.  Allow us to be imprecise and

still arrive at the correct answer.  By setting up graphs (Size,

Top, Distance) adding shapes (Large, Flat, Close) and specifying

the rules (IF the rock is small AND pointed OR to far away THEN

1

eliminate it) we came up with the best rocks.  Unlike a system

that uses absolutes, once it was proven, we could find the

correct rock in any stream not just this one.

ITS BEGINNINGS

In 1965, Lotfi Zadeh found that, due to the contradictory

nature of control systems, they were not easily represented using

the traditional method of modeling them mathematically.  He

theorized that adding imprecision to system would allow it to

react more precisely when presented with conflicting input data.

He published an article, "Fuzzy Sets," creating the field of

fuzzy logic.

Western countries didn't exactly jump when fuzzy logic was

born.  In fact, it took almost 30 years for it to hit the

mainstream media.  There's probably a multitude of reasons but

most are related to the fact that the system could not be proven

mathematically.  Some reasoned that an unprovable system was the

same as no system at all.  Others were uncomfortable with the

word fuzzy.  It may be an excellent word for what it described

but, to those accustomed to precision, it may not have been the

best choice.

Unlike the West, the Japanese were interested.  The word

fuzzy was not translated but transferred phonetically allowing

acceptance of the concept at face value.  This led to the next

step, experimentation, quickly proving traditional methods of

proof unnecessary.

For years the Japanese have marketed fuzzy based products

but the West is catching up.  In the last few years many

companies have begun producing products based on fuzzy logic.

Most large IC manufactures are marketing micro-controllers and

co-processors optimized to perform in a fuzzy logic environment.

For example, did you know that fuzzy logic is used to control the

Saturn's transmission?  The designers found that they achieved

better performance and smoother operation as opposed to using the

traditional approach.  This trend will continue as more designers

see the benefits of this type of control.

THE HARDWARE

To gain an understanding of fuzzy logic we'll use the

simplest control problem, one input and one output.  Figure 1

shows the flow of the entire system.  What we'll do is vary the

frequency of a speaker as the room temperature varies.

The hardware was designed to show the ease of setting up a

control system based on fuzzy logic.  Most of us have an unused

parallel port or an A/B box so, instead of using another slot in

the PC, the hardware interface will be done via the PC's printer

port.

2

TEMPERATURE TO VOLTAGE

The LM335 converts the room temperature into a voltage

that's used to feed the A/D.  Basically, it's a zener diode whose

breakdown voltage varies proportionally with temperature.  For

each degree Kelvin the junction voltage changes 10mV.  (The

Kelvin temperature scale begins at absolute zero [ -459.67� F].

To convert to Fahrenheit, F� = 1.8 * Kelvin - 459.67�).  To hold

the input stable, R2 and C2 are configured as a low pass filter

with a one second time constant.

VOLTAGE TO BITS

The ADC0803 in Figure 2a is an 8 bit successive

approximation analog to digital converter.  Figure 2b breaks the

part into its major components.  To understand it, we'll go

through it pin by pin.

Pin 1 - Chip Select:  When low, the Start and OE inputs are

passed to the internal logic.

Pin 2 - Output Enable:  When high the output register is in

tristate.  A zero on this pin passes the data in the output

register to the output pins.

Pin 3 - Start:  A conversion begins one to eight clocks

after the rising edge of a signal on this pin.  An additional 64

clocks are needed to convert the data.  One clock later the data

is latched into the output register.  With a 640 kHz clock,

conversion time ranges from 103 to 114 usec.  If a second pulse

is received before the previous conversion is complete, the cycle

restarts.

Pin 4 - Clock In:  Sets the conversion time.  The pin can be

tied to an external clock source or a capacitor and resistor can

be used with the internal oscillator.  To find component values,

Frequency � 1 / (1.1 * R * C) with R � 10K.  The part is

guaranteed to meet specifications with a frequency of � 640KHz

and a low/high � 275 nsec.

Pin 5 - End of Conversion Signal:  Signals an external

device that the converted data is ready in the output register.

It is active low.

Pins 6 and 7 - Voltage In Positive/Negative:  Together these

pins determine the voltage that will be digitized.  Before

conversion VIn- is subtracted from VIn+ (differential input) and

the resultant used.  Pin 6 is the most positive, 7 the least.

Neither can exceed VCC + 50mV.

Pin 8 - Analog Ground:  This ground is used internally by

sections that handle the input signal.  For greatest accuracy,

the ground should be as noise free as possible.

Pin 9 - Voltage Reference:  The voltage on this pin is tied

one resistor below the top of the internal resistor ladder.  The

minimum resistance to ground of this input is 2.5K, typically,

8K.  The voltage can be viewed as doubled internally.  When used

3

with the VIn- pin, it sets the input voltage range that generates

the full scale output (0 - FF).  For example, if the minimum

input voltage is 0.5 volts and the maximum is 3.5, the reference

voltage would be, (3.5 - .5) / 2 = 1.5 volts.  An output of 0 is

read with both VIn+- at .5 volts, a FF with VIn- at 0.5 and VIn+

at 3.5 volts.  If the reference voltage is < one volt, additional

circuitry is required.

Pin 10 - Digital Ground:  The same as pin 8 but it's used

for digital.

Pin 11 - 18 - Output Port:  With OE low, the data in the

output register is passed to these pins.

Pin 19 - Clock Resistor:  If the internal oscillator is

used, a resistor from pin 4 to pin 19 in combination with a

capacitor sets the frequency of the clock.  The value of the

resistor should be close to 10K.

Pin 20 - VCC:  This voltage supplies all section of the A/D.

The absolute maximum is 6.5 volts.

PC INTERFACE

Figure 3 illustrates the major parts of the printer port's

hardware.  Table 1 has the addresses for each of the three ports.

Don't confuse the address of LPT1 with that of port 1.  They may

not be the same.  The operating system sets the address of LPT1

equal to the first parallel port found, which may or may not be

port 1.

Each parallel port can be broken into four sections: status,

control, input, and output.  The circuitry uses every free line

of the port except the interrupt to the processor that must

remain off (control port D4 = 0).  Depending on how your system

is configured, if you forget this you may end up with unexpected

results or a system crash.

The software used to read the A/D is shown in Listing 2.  To

start the conversion we drive D2 of the control port low then

high pulsing the Start pin.  After giving the A/D time to

digitize the voltage, we read the data.  The port's hardware

wasn't designed for this, so to get the data, we have to do a

little manipulation.  First, bits 0 - 2 are read from Status A

via data bits 0, 1, and 3.  There is logic behind using D3

instead of D2.  Notice that the clear input of the control port

is tied to the PC's reset line.  On power up, pins 1, 14, and 17

are pulled high, pin 16 is driven low.  The outputs are open

collector, thus, by using pin 17 we eliminate the possibility of

bus contention with data bit 2 of the A/D.  The other five bits,

D3 - D7, are read at Status B.  Notice bits D0 - D2 and D7 are

inverted by the port's hardware and must be reinverted to be

used.  A couple of lines of code later and we've back to the

original byte.

4

BITS TO VOLTAGE

After the value from the A/D is processed by the fuzzy

system, the DAC in Figure 4a converts it back to a value we can

use.  The DAC0832 is a double buffered multiplying D/A converter

with complementary current outputs that is easily interfaced to

many micro-processors.  A block diagram is shown if figure 4b.

Again we'll go through it pin by pin.

Pin 1, 2, 19 - Input Latch Control:  With CS and Wr1 low,

ILE high, the data at D0 - D7 is passed to the DAC register.  The

latch is equivalent to a 373, i.e., pass data when LE is high,

hold when low.  Driving CS/Wr1 high or ILE low, latches the data.

Pin 4 to 7, 13 to 16 - Input Data:  When the input latch

control is enabled data on these pins is transferred to the input

register.  This data determines the amount of total current that

flows through each of the output pins.

Pin 8 - Voltage Reference:  The voltage on this pin sets the

total current flowing through the output pins.  Internally, it's

returned to ground via a resistor.  The resistor's value can vary

between 10K and 20K.  The voltage can be a DC level, an AC

signal, or a combination with an  absolute maximum of �25 DC, �10

AC.  This is what gives the DAC it's multiplying ability.  The

current at the outputs can be changed by either changing the

digital input or adding/subtracting voltage at this pin.  To

illustrate, say the reference is 2.5 volts DC.  This generates

167 micro-amps (2.5/15k) of constant current at the outputs (pins

11 and 12).  Say we sum in a 0 to 5 volt sinewave.  The current

at the outputs is now the combination of the fixed reference +-

the additional 167 micro-amps of AC. In other words, without

changing the DAC's data, the current varies from the minimum of 0

(ref - 167) to a maximum of 334 (ref + 167).

Pin 9 - Feedback Resistor:  If the output current is to be

changed to a voltage via IOut1, this resistor should be used in

the feedback loop of the Op-Amp.  Its value and temperature

characteristic are identical to those of the resistor network

ensuring that after initial adjustments, the circuitry will

remain calibrated.

Pin 3, 10 - Ground:  Pin 3 is used in digital section, pin

10, the ladder network.

Pin 11 - Output Current 1:  The outputs, pins 11 and 12,

share the current set by VRef.  They are a complement of each

other, i.e., whatever current isn't flowing through one is

flowing through the other.

Pin 11 is called the true output.  The current through it is

minimum with an input of 0 maximum at FF hex.  The amount the

current changes per digital bit is approximately VRef/15K/256

(15K is typical).  In the circuit shown, an input of 0 produces 0

output current.  Each additional bit generates 652 nano-amps (167

micro/256) of current.  We can use the current per bit to find

the output for any input.  For example, 55 hex generates 56

micro-amps (85 * 652), FF hex � 166.  Keep in mind these are

calculated values using typical ranges.

5

Pin 12 - Output Current 2: This is an inverted pin 11.  The

current is maximum with an input of 0 (all current flowing

through pin 4), minimum at FF hex.  Instead of adding, each bit

subtracts from the output current.

Pin 17, 18 - Output Register Control:  Holding Wr2 and XFER

low passes the input register's data to the converter section.

Instead of passing the data directly from the input latch to the

D/A, the second register performs the double buffering.  It's

used in situations that require the DAC's data be written then

later passed to the convertor.  An example is where the output of

multiple DAC's must all change together.

Pin 20 - VCC:  Supplies the voltage for the IC.  The

absolute maximum is 17 volt DC.

The software is shown in listing 3.  The byte is written to

the output port and instantaneously (in our world) converted to a

DC level.

CURRENT TO SOUND

The remainder of the circuitry in Figure 4a converts the

current from the DAC to a pulse train that drives a buzzer.  To

stay with one supply, an ICL7660 (IC U4) is used to generate the

-5 volts.  Internally, an on board oscillator controls the on

time of four MOS switches.  The first half of the cycle is used

to charge C7 to VCC.  During the next half, the positive side of

C7 is tied to ground, the negative, to pin 5.  The charge is

transferred in the opposite polarity.  The part can deliver 100ma

without significant change in the output voltage.

First we integrate the current creating a sawtooth waveform

(U3a).  The frequency of the signal is dependant on the amount of

current flowing through pin 11.  At an input is 0 hex,

essentially no current flows through IOut1, thus, the frequency

is 0.  As the hex input increase, the current also increases

varying the charge/discharge rate of C6.

The signal is passed to an open collector comparator to

generate the squarewave (U3b).  Notice its output is tied to

VRef.  Each cycle, the charge across C6 must be reversed.  To do

this we drive U2-8 to +5 then to -5.  Assumes the comparator's

output just switched to +5.  C6 was charged to +1.667 volts

during the previous half cycle.  C6 discharges into IOut1

(electron flow), through the resistor ladder to VRef.  U3b's

output will switch to -5 volts when the voltage of C6 exceeds 1/3

the positive supply.  At this point all available supply voltage

is dropped across R5.  When U3b-3 is driven below U3b-2 the

output switches reversing the direction C6 charges.

To find component values, frequency = Digital input/(256 *

15k [typical] * C6) where, R5 = 3 * R4.  For the circuit shown,

Freq max = 255/(256 * 15k * 5nf) � 13kHz.

6

BUILDING AND CALIBRATION

Due to the simplicity and purpose of the circuitry, several

A/D and DAC characteristics were not mentioned.  All convertors

have a list of pitfalls.  If your application requires increased

accuracy, look into them before beginning.

When building the board, keep the converters' components as

close as possible to the ICs.  Keep the digital and analog

grounds separated except at the point of connection near where it

enters the board.

To overlook the problems associated with driving cables, the

board uses an on board connector.  If you use a cable, add the

necessary components (well, to follow the "rules").

To avoid the possibility of setting up a ground loop, float

the supply's ground (i.e., disconnect the jumper between the

supply and earth ground).

The converters are CMOSS.  Handle appropriately.

Use an ohm meter to verify the board's connections.  Ensure

all pins tied to the port are not tied to other pins.  Set all

pots to mid-range.  Hook the board into the printer port.  Run

the software in listing 1.  The step number and, if applicable,

data will be displayed.

Step 1 - The port's interrupt request is disabled.  Apply

power.  Verify U4-5 is approximately -5 volts.  Press return.

Step 2 - The A/D is read and the value is displayed.  Change

the sensor's temperature to the lowest that will be encountered.

Measure the voltage on pin 6.  Adjust TempMin (pin 7) for the

same value.  Change the sensor's temperature to maximum.  Again,

measure the voltage on pin 6.  Set the voltage on pin 9 to one-

half that of pin 6.  Verify the data displayed is FF hex.  Put

the sensor in minimum temperature.  Ensure the displayed value is

0.  Press return.

Step 3 - 0 to FF is written to the DAC in a continuous loop.

Verify all data pins of the DAC are pulsing.  Press return.

Step 4 - FF is sent to the DAC.  Check that the sawtooth

waveform and the pulse train are present.  Press return to exit.

ON TO FUZZY LOGIC

Now that the hardware is in place, there are several ways

the control system could be generated without using fuzzy logic.

One way might be to equate the temperature of the room with the

different frequencies produced by the speaker.  That's not too

hard.  All we need to do is specify, in absolute terms, the

difference between warm and cold.  We'll say that the room is

warm when the value read at the A/D is between 25 and 75.  That

means if the value is below 25 the room will is considered cold.

Is that correct?  Does the room suddenly change from warm to cold

at one specific temperature?  What's needed is another set.

We'll call it KindaWarm.  It is defined as the span of inputs

between  . . .  and on, and on.

The difficulty of the task quickly escalates into a

7

full-blown project.  What's needed is a way to tell a computer

that Warm, KindaWarm, and Cold, are not separate entities, but a

combination of all three.  Let's see how fuzzy does it.

START AT THE BEGINNING

The first thing to do is decide what the input versus the

output graph should look like.  We'll specify that as the room

temperature increases the frequency of the speaker will also

increase.  That means that our control system will be the

equivalent of reading the A/D and sending the value to the DAC.

No fun at all.  Instead, let's specify the frequency of the

speaker will be maximum when the room temperature is perfect,

minimum at either extreme.  We will further stipulate that the

graph will resemble a bell curve.

SET UP THE GRAPHS

Both the input and output require a separate graph. Figure 5

shows the graph for the input.  The values for the X axis are the

input values from the A/D.  They start at 0 (room coldest) and

end at 255 (room warmest).

The Y axis is called the degree of membership (fuzzy value).

Most documentation specifies its range as 0 to 1 and annotates it

as "mu".  Instead of adding to the complexity of this, we'll

refer to this axis as the percent of membership.  Our range will

start at 0 (no membership) and end at 100% (complete membership).

If we were using Boolean logic there would only be two values on

this axis, 0 (false) and 100 (true).  Fuzzy includes these and

all values between.  One could view Boolean as a pulse train

(True or False), fuzzy a sinewave (varying degrees of truth).

ADD THE SHAPES

Shapes replace all the drudgery of defining and redefining

values. The values contained in each set are still defined but,

instead of using numbers, a shape is drawn.  Each shape defines

what values are contained in the fuzzy set and the weight (%

membership) of each.  As an analogy, say you're in a boat and

heading toward the dock.  The input shape (set) contains the

distance between you and the dock.  The output shape, the range

of the gas lever.  When the dock is a mile away, the distance has

no membership in the input set so the lever is unchanged.  The

closer you get the higher the membership within the shape.  This

membership is transferred to the output's shape decreasing the

lever's position.

The shapes we can use are trapezoids and triangles.  It is

not readily apparent how we'll achieve a bell curve using these

shapes but, we will find that when combined with rules, any

output shape can be represented.

To select input shapes, first establish what values belong

in each set.  This defines the base of the shape.  The left

8

bottom is the minimum, the right, the maximum.  Next decide when

the fuzzy set becomes 100% true.  For example, referring to the

Warm shape in figure 8a, we're saying the room is completely warm

(membership is 100%) at only one point, the shape's peak.  If

desired, we could replace the triangle with a trapezoid, allowing

the set to be completely true for more than one input, or for

that matter, all inputs as in rectangles.  Now that the points

are defined, all that's left is to connect the dots.

Overlap shapes anytime values belong to more than one set.

When the input value for the RoomTemp graph (Fig 8a) is within

the KindaWarm shape three components, Cold, KindaWarm, and Warm

interact.  That's how we prevent the absolutes so common in

conventional logic.  Instead of specifying KindaWarm as a

singular set of values, we say it consists of a varying degree of

three.

Output shapes are a little different.  Instead of working

with the edges, centroids are used.  In laymen's terms, this is

the center of gravity, the point where the shape would balance on

the end of a pin.  Each centroid has an x and y coordinate of

which only the x value is used.

Shape selection is easier than inputs.  First, find the

number of shapes required.  This is dictated by the number of

dissimilar areas you're attempting to control.  In this system,

there are three areas, thus, three shapes.  FreqLow (Figure 9)

controls the leading and falling edges, FreqMiddle, the area

between this and the peak and, FreqHigh the peak.  Next,

determine what the output should do as fuzzy values vary.  For

example, the FreqLow shape is drawn so that as fuzzy values

decrease the output will increase.  FreqHigh is the opposite,

decreasing fuzzy values decreases the output.  An equilateral

triangle is used for FreqMiddle so the output for all fuzzy

values are the same, creating a line.  Position the shape by

visualizing the centroid with a fuzzy value of 100%.  Place it on

the graph so this point is at the desired output if this were the

only shape used.

A few notes about output shapes.

-    Overlapping shapes has no effect.  As long as the

centroid is in the same place, the base of the

FreqMiddle shape could be reduced without affecting the

output shape.

-    Unlike inputs, gaps between shapes have no effect.

-    As long as their bases are identical, a centroid's x

coordinate is the same for equilateral triangles,

trapezoids with equal slopes, and rectangles and is

always in the middle of the shape.

-    When using trapezoids, the length of the top determines

how close each centroid is to the previous.  The longer

the top, the less change in the x coordinate of the

centroids.

-    Of all the components, output shapes have the least

affect.

9

INPUT TO FUZZY VALUE

Fig 6a shows the completed graph for RoomTemp.  To see how

the fuzzy value for a given input is derived we'll use an input

of 205.

The first step is to draw a vertical line from 205 to the

top of the graph.  This line intersects two shapes, thus, the

value has membership in both fuzzy sets Hot and KindaHot.  To

find the fuzzy value, draw a horizontal line from the point of

intersection to the Y axis.  This tells us 205 has an 82%

membership in the KindaHot set and 21% in the Hot set.  We would

say that it's hot and getting hotter.

Now that we have the fuzzy value, we'll apply it to the

output.  Before doing that we need to specify how the conversion

will take place.

FORMULATING RULES

Rules define the relationship between the input and output

shapes. They are formulated much like the Basic statement, IF x

THEN y, with a few modifications.  Valid operators are And

(intersection), Or (union), and Not (complement).  Table 2

explain the operators and how each is evaluated.

As stated, rules tell the system how the inputs and outputs

are linked together.  To illustrate, we'll use the propeller of a

boat. If there's no propeller (rules), the motor (input) can run

all day but we ain't going no where (output).  If we install a

small propeller, the rule becomes, "If the motor is fast then the

output is slow".  Changing to a larger propeller, the rule

changes to, "If the motor is slow then the output is fast".

Fuzzy rules differ from their conventional counterparts.

Instead of a rule evaluating to one of two values, true or false,

they can have varying degrees of each.  For example, given this

statement, If X And Y Then Z, the statement evaluates to true

(the rule fires) if both X and Y have some value other than 0.

Each rule specifies what input conditions must be met for the

rule to evaluate to true, and once it does, which of the output

shapes to apply the value.

Once the operators are understood, rule development is

straight forward.  Write the rules as if you're explaining to

someone else how the system works.  First, visualize the input

graph superimposed over the final output graph (Figure 10a).  For

each input shape, define what the output should do when the input

is within the set.  For example, rule B in table 3a says if the

input falls in either the Cold or Hot shape set the frequency

low.  This rule controls the outside edges of the bell curve.

How many rules are required?  It depends on the number of

inputs the system has.  This system has one input with six

shapes.  To cover all possible input conditions a minimum of six

rules are required.  If the system had two inputs, each with

eight shapes, 8 * 8 or 64 rules would be needed to cover all

input combinations.  This doesn't include rules added to optimize

10

the output shape.  For example, this system uses ten rules

instead of the minimum of six (the OR operator is used as a

connector).  As you can see, there's a point when the number

becomes unmanageable.  Typically, for systems with more than one

input, not all combinations are possible, thus, not all rules are

required.  Also, the number of shapes can often be reduced and

still achieve a satisfactory output.

TRANSLATING FUZZY VALUES TO USABLE OUTPUTS

Now that we have the rules (table 3a), we'll determine which

fire.  As stated, an input of 205 has membership in both KindaHot

and Hot.  Checking the rules, we find that rules B and C use the

shapes.  Both rules use the OR operator so they evaluate to true

though there is no value for Cold or KindaWarm.

Next, we'll apply the fuzzy values to the output.  In fuzzy

terms, we'll defuzzify them.  The steps required from input to

final output are: crisp input -> fuzzy (% membership) -> rules ->

defuzzify -> crisp output.  The word crisp refers to the inputs

and outputs of the fuzzy system.

We found above that the % membership for rule B, with an

input of 205, was Hot at 21%.  We will defuzzify this value and

apply it to the output.  The process is more complex than

converting to fuzzy values.

Refer to figure 6b.  Draw a horizontal line from the 21%

mark on the Y axis.  The shape we'll use is specified in the rule

as FreqLow.  That's how the % membership is transferred.  It

defines the top of the area (dark outline) that's used to

calculate the centroid.  After finding the centroid, draw a

vertical line to the X axis.  This value, 29, is the output for

rule A.

If this were the only rule that fired, the speaker's

frequency would be close to minimum, not exactly what we want.

With an input of 205 the room is hot but has not reached maximum.

This is where fuzzy starts to work.  Due to the overlapping of

the Hot and KindaHot shapes, rule C also fires, increasing the

speaker's frequency.  Anytime more than one rule fires, the final

output is modified according to the weight of each that fired.

An analogy would be driving a car with pressure on both the brake

and the gas.  The final output, the speed of the car, is

dependant on the amount of pressure applied to each.

Before we can find the crisp output, we need to calculate

the centroid for the second rule (C).  This is done the same as

above by transferring the 82% membership to the FreqMiddle shape

(the shape is an equilateral triangle so, the centroid is in the

middle of the base whatever the fuzzy value).  Referring again to

Figure 6b, the centroid is 77.  No other rules fired, so we'll

calculate the final output.

11

Crisp Output   =  Crisp 1 * Weight 1 + Crisp 2 * Weight 2

(Weight 1 + Weight 2)

=  (29 * .21 + 77 * .82) / (.21 + .82)

=  67 is sent to the DAC

Notice the fuzzy value is used first in the calculation of the

centroid then again to find the final output.  There's a couple

of reasons for this.  First, whatever the type of shape,

centroids move very little.  Using the value to define the usable

area allows fine adjustments to be made on the final output

shape.  Next, it must be used in the final calculation so the

relative truth of each rule is considered.  For example, the

weights (relative truth) of rule B and C were 21% and 82%

respectively.  What we want is rule C to affect the output more

than rule B.  To do this, we first see how true the rule is

relative to itself then how true that is relative to total truth.

That's the basis of fuzzy logic.  It doesn't deal with absolute

truth, but instead, relative truth.  Still fuzzy? Let's try it

another way.  We'll break the equation into two parts and use the

first.

Crisp 1 * Truth 1 / (Truth 1 + Truth 2)

which says

Output for the rule relative to its truth / Total truth

Why is total truth used?  Because up front there's no way to know

what completely true is.  In the equation above, 103% (21% + 82%)

represents complete truth, not 100%.  If ten rules fire, each

with the weight of 100%, complete truth becomes 1000.  In

essence, whatever the value for total truth, it equals 100% true.

MAKING IT WORK

The output versus the input graph for the system is shown in

figure 10a.  Unfortunately, we didn't meet our objective.  There

are two problems, the top shouldn't be flat nor should the

corners be abrupt.

We need a way to modify the system to achieve the desired

output.  Let's break the problem into three parts, area between 0

and 62, 118 - 138, and 194 - 255.  Working with the first, we see

the Cold shape begins at 0 and ends at 62.  What we'll do is add

another rule that "pulls" the output toward 0 when the input is

within this shape.  Also, notice the Hot shape controls the same

portion but on the opposite end.  We'll fix both with one rule.

This is a good time to explain the two ways the final output

can be derived.  We'll call one the max method, the other

average.  Using the max method, only one centroid per output

shape is used in the final calculation.  The average does just

that, averages all centroids.  For example, to fix the problem,

we'll specify rule B twice.  With an input of 205, three

centroids would be produced, one for rule C, and two for B.  To

calculate the final output:

12

(MAX METHOD)

Crisp Output = (Cent A * Fuzzy A + Largest Centroid B * Fuzzy B)

(Fuzzy A + Fuzzy B)

(AVERAGE METHOD)

Crisp Output = (Centroid A * Fuzzy A + 2 * Centroid B * Fuzzy B)

(Fuzzy A + 2 * Fuzzy B)

At first glance it may appear that specifying a rule twice

doubles its effect.  Unfortunately, it's not that easy.  Due to

the interaction between all centroid/fuzzy pairs (it's that

relative truth thing), it may fix the problem, create another, or

have no effect.  It works here, so we'll use it.

How should the area between 118 and 138 be adjusted?  Should

we modify the rules or the shapes?  It's a toss up.  To explain,

say you toasted some bread but it's to light.  How do you make it

darker?  Toast it again (input), change the toaster's setting

(rules), or manually hold the lever down (output)?  They all

achieve the same thing, just in a different way.  Modifying

shapes is the easiest so let's start there.

Superimposing the input graph over the output, we see that

the area in question falls within the Perfect shape.  In fact,

the top goes flat as soon as the shape comes into existence.

When the input is within this shape, two rules fire, A and D.

There's our problem.  Both rules tell the system to increase the

frequency.  To fix it we need to add interaction, holding the

output low longer.  Checking the rules, we find C uses the

FreqMiddle shape.  If we could make this rule fire, it would

provide the fuzziness we need.  After modifying both the

KindaWarm and KindaHot shapes as shown in figure 8b, the problem

is solved.  At the points of overlap the rules "fight" each

other, smoothing the transition.

The modified output is shown in figure 10b.  Although it's

better, it still isn't right.  The transition with an input of 62

is still too abrupt.  To smooth it further, we'll add another

rule that will fire only when the input is within the area where

the Cold and KindaWarm shapes overlap.  The fuzzy value will be

applied to the FreqLow shape.

IF (RoomTemp Cold) AND (RoomTemp KindaWarm)

THEN (Speaker FreqLow)

Using the AND operator allows us to "zero in" on specific areas

of shape overlap.  At any other time, one or more of the fuzzy

values are 0, thus, the rule doesn't fire.

The same problem exists on the falling edge of the output.

Instead of writing a separate rule, we'll combine both using the

OR operator as shown in table 3b, rule F.

The final output is shown in figure 10c.  Table 4 shows

which rules fire as the input varies.  Although it took several

tries to get the optimum shape/rule mix, it's much easier than

modifying code or manipulating equations.

13

SOME USES

The applications are endless.  Some of them are:

-  Robotics.

-  Home/office burglar alarm and climate control.

-  AC/DC motor control.

-  Weather prediction through humidity, temperature, pressure,

etc.

-  Analyzing data for patterns/trends.

-  Controlling product flow in production.

Anything requiring control or generalization is a possible

application.

WHERE TO GO FROM HERE

Hopefully, you gained a basic understanding of fuzzy logic.

The next step is to apply the principles outlined in the article

to your own designs.

Addr Designator      PORT 1    PORT 2    PORT 3

1              3BC       378       278

2              3BD       379       279

3              3BE       37A       27A

Note:  All addresses in hex.

Table 1: Parallel Port Addresses

IF   -    Begins each rule.  Statements following are input

conditions.

THEN -    Ends the input conditions that must be met for a rule

to fire. Statements that follow are the output shapes

to which the fuzzy values will be applied.

AND  -         The AND operator specifies a rule will evaluate to

true (fire) only when the input is within the overlap

of two or more shapes.  For example, the graph in

figure 11a contains two shapes, S1 and S2.  The rule,

given below the graph, will fire only for inputs within

the shaded area.  The fuzzy values applied to the

output will follow the dark outline.

Technically, the AND operator designate that for a

rule to fire, the fuzzy value for both shapes must be

greater than 0.  The value used in the output

calculation is the minimum.  Given this statement, IF X

AND Y AND Z THEN  . . . , for the rule to fire X, Y,

and Z, must all be greater than 0.  The fuzzy value for

the set (the value applied to the output) is the

minimum of X, Y, and Z.

OR    -        "OR" works the opposite of "AND".  Instead of

using the overlap, a rule evaluates to true anytime the

14

input is within either shape.  This is shown in figure

11b. Again, the fuzzy values transferred to the output

follow the dark outline.

Rules can contain multiple ORs but only one fuzzy

value is used in the final calculation.  Given the

rule, IF A AND B AND C OR D AND E AND F OR G THEN H,

the fuzzy value applied to H is the maximum of each

combination separated by OR.  So, if A = 10%, B = 0%, C

= 80%, the outcome for the set of ANDs is the minimum,

0%.  Further, if D = 9%, E and F = 100%, the output

here, again the minimum, is 9%.  The fuzzy value

transferred to the output is dependent on G.  If G =

8%, the value applied H is 9%, if G is larger than 9%

its value is used.

In maximum mode (see text), the OR operator is

nothing more than a rule connector.  For example, the

rule in figure 11b could be separated into two, IF S1

THEN and IF S2 THEN, with no change in the output

shape.  The reason for this is that only one value for

each output shape is used for either the combined or

separate rules.

Things change for average mode.  Because all

centroids are used in the final calculation, the rule

using the OR would produce one centroid, the separate

rules, two.  Depending on if both rules fire

simultaneously, the final output may be significantly

different.  Essentially, using the OR operator in

average mode changes the rule to max mode (more

fuzziness).

NOT  -    The NOT operator inverts the shapes.  What happens is

the fuzzy value is found as outlined above but, before

being used, it's subtracted from 100%.  Thus, if the

fuzzy value is 77%, it's inverted and becomes 33%, if

33 it becomes 77.  Figure 11, c - e, illustrate how the

NOT modifier is evaluated for each operator.

NOTE

The operators work the same for multiple inputs.  To

use, visualize the inputs superimposed over each other.

Table 2:  Key Words

A -  IF (RoomTemp Perfect) THEN (Speaker FreqHigh)

B -  IF (RoomTemp Hot) OR (RoomTemp Cold) THEN (Speaker FreqLow)

C -  IF (RoomTemp KindaWarm) OR (RoomTemp KindaHot)

THEN (Speaker FreqLow)

D -  IF (RoomTemp Warm) THEN (Speaker FreqHigh)

Table 3a:  Initial Rules

15

E -  IF (RoomTemp Hot) OR (RoomTemp Cold) THEN (Speaker FreqLow)

F -  IF (RoomTemp KindaHot) AND (RoomTemp Hot) OR (RoomTemp

KindaWarm) AND (RoomTemp Cold) THEN (Speaker FreqLow)

Table 3b: Additional Rules

RoomTemp       Rules Fired

IN = 0          B,C,E,F

0 < IN < 62          C

IN = 62          A,C,D

62 < IN � 118        A,D

118 < IN < 124       A,C,D

124 � IN � 132        A,D

132 > IN < 138       A,C,D

138 � IN < 194        C,D

IN = 194            C

194 < IN < 255      B,C,E,F

IN = 255           B,E

Table 4: Rules Firing for Given Inputs

LM335 converts temperature to voltage.

|

ADC0803 converts voltage to bits.

|

System reads ADC's value at port and applies it to fuzzy system.

|

System finds output value using graphs, shapes, and rules.

|

Value is sent to the DAC.

|

DAC converts value to current.

|

Op Amp integrates current creating sawtooth waveform.

|

Op Amp changes sawtooth to pulse train.

|

Buzzer converts pulse train to sound.

Figure 1: System Outline

NOTE

The following is the minimum code required to use the hardware.

The only consideration was it be as simple as possible.

Equates are used in all three listings.

16

;;;  EQUATES  ;;;

Port equ 0378h         ;base address of port to use

;change this if different

StatusPort equ Port+1  ;addr of status port for rd

ControlPort equ Port+2 ;addr of ctrl port

Time equ 046ch         ;Dos timer tick address low byte

GetBit equ 080h        ;mask byte used to get bit

Dos equ 021h           ;Dos interrupt

Screen equ 010h        ;Bios interrupt for screen access

GetDisplay equ 0fh     ;get video mode

Cls equ 00             ;set video mode

Display equ 02         ;Dos display char function

PositionCursor equ 02  ;Bios set curser function

Exit equ 020h          ;program exit

GetKey equ 08          ;function to get char from keybd

KeyPressed equ 0ffh    ;key was pressed

CheckKey equ 0bh       ;function to check key pressed

LoopsToWait equ 0      ;takes 17 clocks for each loop - to find

the

; number required for your system

; Loops = 115 micro-sec/time period of

clock

;;;;;;;;;;;;;;;;;;;;;;;;;;;;  CODE ;;;;;;;;;;;;;;;;;;;;;;;;;;;;;

cal segment

main proc far

assume cs:cal

org 0100h

start:

mov dx,ControlPort

xor ax,ax

out dx,al         ;set IREQ to off

call ClearScreen

mov dl,'1'        ;indicate step 1

mov ah,Display

int Dos           ;send 1 to display

mov ah,GetKey

int Dos           ;apply power then press return

call ClearScreen

mov dl,'2'         ;indicate step two

mov ah,Display

int Dos            ;send 2 to the display

call ReadAD

call ClearScreen

mov dl,'3'         ;indicate step three

mov ah,Display

int Dos            ;send 3 to the display

call LoopWriteDac

call ClearScreen

mov dl,'4'         ;indicate step three

17

mov ah,Display

int Dos            ;send 4 to the display

mov dx,Port

mov al,0ffh

out dx,al          ;send an ffh to DAC

mov ah,GetKey

int Dos            ;get a key

call ClearScreen

int Exit           ;all done

main endp

;;;;;;;;;;;;;  ReadAD  ;;;;;;;;;;;;;

ReadAD proc near

mov ah,GetDisplay    ;read display mode

int Screen

mov ah,PositionCursor  ;going to set cursor to start

mov dh,1             ;line number

mov dl,0             ;column number

int Screen           ;bh has display page

call GetADValue      ;rets al with value

mov ah,al

mov cx,4             ;set up for rotates below

ror ah,cl            ;hi nibble to low

xchg al,ah           ;digits right to display

push ax              ;save for 2nd digit

call ToAscii

mov dl,al

mov ah,Display

int Dos              ;char in dl to screen

pop ax

xchg al,ah

call ToAscii

mov dl,al            ;next digit

mov ah,Display

int Dos

mov ah,CheckKey

int Dos              ;check for key

cmp al,KeyPressed

jne ReadAD

mov ah,GetKey

int Dos              ;clear keybd

ret

ReadAD endp

;*****  NOTE: Insert Listing 2 here  *****

18

;;;;;;;;;;;;;  LoopWriteDac  ;;;;;;;;;;;;;;;

LoopWriteDac proc near

inc al            ;next value

mov dx,Port

out dx,al

mov cx,08000h

WaitABit:

loop WaitABit     ;wait so change can be seen

push ax

mov ah,CheckKey

int Dos           ;check for key

cmp al,KeyPressed

pop ax

jne LoopWriteDac

mov ah,GetKey

int Dos           ;clear keybd

ret

LoopWriteDac endp

;;;;;;;;;;;;;;  ToAscii  ;;;;;;;;;;;;;;;;;

ToAscii proc near

and al,0fh           ;only low nibble

add al,030h

cmp al,03ah          ;> 9

jl ToAsciiEnds

add al,07h           ;get to A

ToAsciiEnds:

ret

ToAscii endp

;;;;;;;;;;;;; ClearScreen ;;;;;;;;;;;;;

ClearScreen proc near

mov ah,GetDisplay

int Screen           ;get display mode

mov ah,Cls

int Screen           ;clear screen

ret

ClearScreen endp

cal ends

end start

Listing 1: Calibration

GetADValue proc near

mov dx,ControlPort

mov ax,04

out dx,al         ;set start to 1 just in case

xchg ah,al

out dx,al         ;start to 0

xchg ah,al

out dx,al         ;then back to 1

mov cx,LoopsToWait  ;number of loops for 115 micro-secs

WaitOnAd:

loop WaitOnAd

19

in al,dx          ;get the 1st 3 bits

not al            ;complement all the bits

mov ah,al

and ah,07         ;unused bits to 0

shr al,1          ;D3 to D2 position

and al,04         ;mask all but D2

or ah,al          ;D0 - D2 in ah

mov dx,StatusPort

in al,dx          ;get D3 - D7

and al,0f8h       ;dump D0 - D2

or al,ah          ;or in D0 - D2

test al,080h      ;top bit a one?

jnz AndAZero      ;no

or al,080h        ;invert top bit

ret

AndAZero:

and al,07fh       ;invert top bit

ret

GetADValue endp

Listing 2: Read A/D

WriteDac proc near

mov dx,Port

out dx,al         ;Assumes value is passed in AL register

WriteDac endp

Listing 3: Write DAC

20