LIGHTON V1.0

(c)1999 Stefan Peichl, Heidelberg, Germany

http://home.t-online.de/home/stefan.peichl/

eMail: stefan.peichl@t-online.de

LIGHTON is a TSR allowing you to use a LED connected to the

serial port of the palmtop to illuminate the screen under bad

lightning conditions. LIGHTON assures, that the power of the

wired serial port is always switched on, so that the LED is

constantly shining, no matter what you do on the palmtop.

Normally the palmtop's serial port power supply is switched on

and off depending on your settings and the programs you use.

A LED, powered from the serial port, would go on and off as you

switch from program to program. LIGHTON watches the serial port

power supply and takes care, that it is always on.

Even during an infrared communication, the wired serial port

power supply is kept active by LIGHTON.

LIGHTON should work in any surrounding and has been tested with

DOS, System Manager, Software Carousel, Windows 3.0 and several

programs running on these platforms.

HOW TO USE LIGHTON

Add LIGHTON to your AUTOEXEC.BAT file. You may add the /S

command line parameter to suppress the activation sound.

LIGHTON uses about 500 bytes when loaded resident. To activate

it, press Fn and the 'L' key simultaneously. You will hear a

sliding tone, telling you that the program is active now. A LED

connected to the serial port should go on. If you press Fn+L

again, the LED should go out and a decreasing sound tells you,

that LIGHTON is deactivated.

To remove the program from memory, just enter LIGHTON /R from

the command line.

You may toggle the activation/deactivation sound with the /S

command line switch, even while LIGHTON is loaded resident.

LIGHTON ? displays a little help screen.

HOW TO CONNECT A LED TO THE SERIAL PORT

Connect the long leg (+) of the LED with the Ground pin and the

short leg (-) with the TD pin of the serial port. Depending on

if you do it at the end of the serial cable or with a custom

cable at the 10-pin palmtop serial port, you have to use

different pins:

Front view of palmtop pins and serial cable pins:

-------------------------------------------------

LED long leg at 5                LED long leg at 5

|                |

LED short leg at 3   |                |     LED short leg at 2

|   |                |     |

1     |   | 5            5 |     |   1

-----|---|-             --|-----|----

| . . . . . |            \ o o o o o /

| . . . . . |             \ o o o o /

-----------               ---------

6          10              9       6

Palmtop                Serial cable

If you use the serial cable end, you may just plug in the LED

in the corresponding holes of the cable as pointed out above.

If the legs of your LED are of equal length, try both ways,

find out which is the - leg and mark it by cutting away a

little piece.

TECHNICAL

You may change the LIGHTON hotkey to any other key you like.

The hotkey is stored at offset 10 and 11 dec of LIGHTON.COM

offset 10 dec: hotkey ascii code

offset 11 dec: hotkey scan code

Don't forget to add 100 hex if you do it under DEBUG.

LIGHTON chains into INT9 and INT1C. After having been loaded

resident, it checks for it's hotkey through INT9. If the key

has been pressed, LIGHTON switches the wired serial power on

at every timer tick. This is done at a very low Hornet index

register level and only needs about 10 bytes of code. So no

general slow down of overall performance should occur.

At the next hotkey detection, the serial power is switched off

only once, but no further actions are taken, that is, all your

settings concerning the serial power should work like before.

LIGHTON only cares about the wired serial port power supply.

The infrared power supply is not touched and may be used

independently, that is, you may use the infrared port while

the LED is powered by the wired port.

NOTE: This feature is not available through BIOS functions.

BIOS functions only allow to have either the wired or

the infrared channel powered, but not both!

This also means, that you should deactivate LIGHTON, if you

detach the LED, in order to save battery power.

DISCLAIMER AGREEMENT

LIGHTON is copyrighted freeware. Users of LIGHTON must accept

this disclaimer of warranty:

"LIGHTON is supplied as is. The author disclaims all warranties

expressed or implied, including, without limitation, the

warranties of merchantability and of fitness for any purpose.

The author assumes no liability for damages, direct or

consequential, which may result from the use of LIGHTON."