7    INSTALLING AND RUNNING

THE INBOARDTM 386/PC SOFTWARE

This chapter explains installing and using the

Inboard 386/PC software.  The Inboard 386/PC

software performs the following functions:

o  Dynamic speed switching.

o  Extended memory support -- not available until

now on the IBM PC or XT, COMPAQ PORTABLE or

PORTABLE PLUS, Tandy 1200 HD, or Leading Edge

Model D.

o  Extended memory diagnostics.

o  Turning extended memory into expanded memory.

o  Up to four times faster hard disk and screen

performance.

InboardTM 386/PC programs

The Inboard 386/PC software diskette contains

the following Inboard 386/PC programs:

o INBRDPC.SYS provides features to let you get the

most from the Inboard 386/PC, such as extended

memory, changing speed with control keys, and

faster system and screen performance.

o ISTATPC.EXE displays the status of the

INBRDPC.SYS settings.

o ICACHE.COM stores data from the computer's hard

disk in Inboard 386/PC memory so disk reads are

faster.

o ILIM386.SYS turns extended memory into expanded

memory.

o ILIM386.COM displays the status of all the

memory in the computer, including on Inboard

386/PC.

o ISPEEDPC.EXE changes speed from the DOS prompt

or from a batch file.

o CHKCOP.EXE verifies that the optional 80387 math

coprocessor is installed correctly.

INSTALLING AND RUNNING THE INBOARDTM 386/PC

SOFTWARE                                           7-1

o KEYBRDPC.EXE is for use with European keyboard

drivers and ill-designed resident programs that

steal keyboard interrupt vectors without

chaining correctly.

NOTE___________________________________________________

The Inboard 386/PC software works with DOS

versions 2.0 and greater, except for

ISPEEDPC.EXE, which works with DOS versions 3.0

and greater.

NOTE-----------------------------------------------------------------

If you are a Microsoft Windows user, delete the

HIMEM.SYS command from your CONFIG.SYS file

before installing the Inboard 386/PC software.

ILIM386.SYS will not work correctly with

HIMEM.SYS installed.

Installing the InboardTM 386/PC software

Running the Inboard 386/PC's SETUP program is

the easy way to install the Inboard 386/PC

software.  Just answer the questions SETUP asks

-- it will copy the Inboard 386/PC programs you

need to your root directory and add command

lines (at your request) to the CONFIG.SYS and

AUTOEXEC.BAT files.  All you do is follow the

instructions in this section, and SETUP does the

rest.

If you prefer to select the programs and

commands yourself, turn to Appendix D -- but be

forewarned that for most people figuring out the

commands and their parameters is confusing and

time-consuming.  Intel strongly recommends you

use SETUP.

This section contains instructions for running

SETUP.  Each step has a box beside it near the

outside edge of the page.  To keep track of

where you are, check off the box after you

complete each step.

NOTE___________________________________________________

SETUP restarts (warm boots) the computer when it

finishes.  If you have data that will be lost

when the computer is restarted (such as in a RAM

disk), save it before you run SETUP.

7-2        INSTALLING AND RUNNING THE INBOARDTM 386/PC

SOFTWARE

1    Make a backup copy of the Inboard 386/PC

diskette.  (Refer to the DOS manual that came

with your computer if you don't know how to copy

a diskette.)

2    With your screen displaying the DOS prompt,

insert the backup copy of the Inboard 386/PC

diskette into drive A.  (If you will be

installing the Inboard 386/PC software on a

system diskette rather than a hard disk, also

insert the system diskette in drive B.)

3    Select drive A as the default drive.  Type:

A:

and press Enter.

4    To invoke the software, type:

SETUP

and press Enter.

NOTE___________________________________________________

Programs that put the computer in virtual 8086

mode can't run with other programs that put the

computer in virtual 8086 mode.  (This is the

nature of these programs and has nothing to do

with the Inboard 386/PC.)  If you plan to use

such a program (for example, Windows 386), don't

tell SETUP to turn extended memory into expanded

memory.

5    Answer the questions SETUP asks.

6    If you told SETUP to add Inboard 386/PC commands

to the CONFIG.SYS and AUTOEXEC.BAT files, read

the note at the end of this step, and then go to

step 7.

If you want to edit these files yourself, add

the command lines (in order) from the CONFIG.SYS

file SETUP created to the top of your CONFIG.SYS

file, and add the command lines (in any order)

from the AUTOEXEC.BAT file SETUP created to your

AUTOEXEC.BAT file.  (Use any ASCII file editor,

such as EDLIN, to edit the CONFIG.SYS and

AUTOEXEC.BAT files.)

INSTALLING AND RUNNING THE INBOARDTM 386/PC

SOFTWARE                                           7-3

NOTE___________________________________________________

If you have an EGA board set to AUTOSWITCH mode,

remove the EGACACHE parameter from the

DEVICE=INBRDPC.SYS command (in the CONFIG.SYS

file).  Your computer may hang if you don't.

(Use any ASCII file editor, such as EDLIN, to

edit the CONFIG.SYS file.)

7    Restart the computer by pressing Ctrl/Alt/Del.

The commands in the CONFIG.SYS and AUTOEXEC.BAT

files can't take effect until you do.

Using the InboardTM 386/PC software

The following subsections describe using the

Inboard 386/PC software.

Changing the InboardTM 386/PC's speed

By default, the computer runs in very fast speed

when Inboard 386/PC is installed.  The Inboard

386/PC software lets you change the computer's

operating speed in two ways:  by typing the

ISPEEDPC command or by pressing a combination of

control keys.  Use the ISPEEDPC command from the

DOS prompt or in a batch file.  Press the

control keys any time, even while a program is

running.

The following two subsections explain how to

change speed.

If you're like most users, very fast speed and

very slow speed are all you'll ever need.  But

for the rare situation where neither of these

modes is right, Intel provides two additional

speeds (described in Appendix D).

The ISPEEDPC command

The ISPEEDPC command lets you switch between

very slow speed and very fast speed by entering

the ISPEEDPC command from the DOS prompt.  You

can enter the ISPEEDPC command from the DOS

prompt or from a batch file, not while an

application program is running.

The ISPEEDPC command has the following format:

ISPEEDPC  n

7-4        INSTALLING AND RUNNING THE INBOARDTM 386/PC

SOFTWARE

where n is 1 (very slow) or 4 (very fast).

To go to very fast speed, type:

ISPEEDPC  4

and press Enter.

INSTALLING AND RUNNING THE INBOARDTM 386/PC

SOFTWARE                                           7-5

To go to very slow speed, type:

ISPEEDPC  1

and press Enter.

Left-Shift/Ctrl/Alt

You can change speed anytime using Left-

Shift/Ctrl/Alt, whether you're at the DOS prompt

or in a program.

To go to very fast speed, simultaneously press

the Left-Shift key, the Ctrl key, the Alt key,

and the 4 key.  The computer will beep four

times.

To go to very slow speed, simultaneously press

the Left-Shift key, the Ctrl key, the Alt key,

and the 1 key.  The computer will beep once.

HINT___________________________________________________

All four keys must be held down together, but

you can't press four keys at exactly the same

time.  For best results, hold down the keys in

the following order:  Left-Shift first, then

Ctrl and Alt, and finally 1 or 4.  Don't release

the first three keys until you press 1 or 4.

7-6        INSTALLING AND RUNNING THE INBOARDTM 386/PC

SOFTWARE

Turning extended memory into expanded memory

If you already have an expanded memory board

(such as an Intel Above Board) in the computer,

you can use either the expanded memory board or

turn extended memory on the Inboard 386/PC into

expanded memory.  You can't do both.

Intel recommends that you continue to use the

Above Board.  Read Appendix C to see what you

must do to use Above Boards when the Inboard

386/PC is installed.

If you already have an expanded memory board in

the computer and choose to turn extended memory

on the Inboard 386/PC into expanded memory, you

must do the following:

o Remove the expanded memory board or reset it to

not provide expanded memory.

o Delete the EMM.SYS file in your root directory.

o Delete the DEVICE=EMM.SYS command in your

CONFIG.SYS file.

o Install the ILIM386.SYS program and edit the

CONFIG.SYS file.  (If you told the SETUP program

you wanted to reserve extended memory for

ILIM386.SYS, this has been done for you.  Refer

to Appendix D if you want to do it yourself.)

Displaying status

To display the status of all parameters for the

DEVICE=INBRDPC.SYS command (in the CONFIG.SYS

file), type:

ISTATPC

and press Enter.

The display provides information only -- you

can't change the settings from the display.

(Edit the DEVICE=INBRDPC.SYS command line in the

CONFIG.SYS file to change settings.  Appendix D

describes all the INBRDPC.SYS parameters.)

INSTALLING AND RUNNING THE INBOARDTM 386/PC

SOFTWARE                                           7-7

If you've installed ILIM386.SYS and want to

display the status of all the memory in the

computer, including memory on Inboard 386/PC,

type:

ILIM386

and press Enter.

Checking the 80387 math coprocessor

The CHKCOP.EXE software tells you whether the

optional 80387 math coprocessor is installed

correctly on the Inboard 386/PC.

To use the CHKCOP software, type:

CHKCOP

and press Enter.

Dynamic speed switching

European keyboards and ill-designed resident

programs that steal keyboard interrupt vectors

without chaining correctly won't let you change

speed by pressing Left-Shift/Ctrl/Alt/n.  The

KEYBRDPC.EXE software solves this problem by

reinstalling the INBRDPC.SYS dynamic speed

switching software.  To use KEYBRDPC.EXE, you

must invoke it every time you turn on or restart

the computer.  Type:

KEYBRDPC

and press Enter.

HINT___________________________________________________

Consider adding the KEYBRDPC command to the end

of your AUTOEXEC.BAT file.  If you add it, DOS

will automatically invoke KEYBRDPC.EXE whenever

you turn on or restart the computer using the

disk containing the AUTOEXEC.BAT file.  To add

KEYBRDPC.EXE to the AUTOEXEC.BAT file, use DOS's

EDLIN or any other editor that can modify ASCII

files.

Don't worry about running programs or using

keyboards that don't need KEYBRDPC.  It won't

interfere with programs or keyboards that don't

need it.

7-8        INSTALLING AND RUNNING THE INBOARDTM 386/PC

SOFTWARE

If you cached the hard disk

CAUTION________________________________________________

Some ill-designed extended memory programs can

write over the hard disk cache and the file

allocation table (if it's been cached),

resulting in corrupted data.  Intel has verified

that the programs listed in Table 7-1 won't

corrupt the hard disk cache.  Other programs may

not cause problems, but they haven't been

verified.  If you want to use an extended memory

program not listed in Table 7-1, be safe -- back

up your hard disk first.

Programs that use only conventional and expanded

memory can't corrupt the hard disk information

stored in the extended memory disk cache.

Table 7-1    Well-designed extended memory programs

__________________________________________________

Application program      Company

________________________ ____________________

AutoCAD                  AutoDesk

________________________ ____________________

Framework II                  Ashton Tate

________________________ ____________________

IBM Disk Cache*               IBM

________________________ ____________________

Software Carousel             Softlogic

Solutions

________________________ ____________________

Super PC-Kwik*           Multisoft

________________________ ____________________

Vcache*                  Golden Bow Systems

________________________ ____________________

VDISK                    IBM

_____________________________________________

*You can use only one disk caching program at a

time.  Although these disk

caching programs work with Inboard 386, Intel

recommends you use

ICACHE.COM (which was written especially for

Inboard 386/PC) instead.

INSTALLING AND RUNNING THE INBOARDTM 386/PC

SOFTWARE                                           7-9

If you have problems

Almost all programs run fine with Inboard

386/PC.  An occasional program, however, may

need special treatment.  If you have trouble

running a program, try the following solutions

in order.

o Almost all application programs run fine in very

fast speed.  A few copy-protected programs,

though, can't run in very fast speed.  If you

get a message such as "Disk Error" or "Illegal

Copy" when your programs run in very fast speed,

load the program in very slow speed and then

switch to very fast speed (by pressing Left-

Shift/Ctrl/Alt/4) to run it.  If you still get

errors, load and run the program in very slow

speed.

o Reset any time-dependent parameters to

compensate for the computer's increased speed

when Inboard 386/PC is installed.  For example,

turn off the MultiMate Advantage keyboard

accelerator.  For CROSSTALK XVI, increase the

time-out for the dial tone and redial tone.

If these solutions don't fix the problem or you

have problems with your computer immediately

after installing the Inboard 386/PC software,

turn to Appendix B for troubleshooting advice.

Where to go from here

Now that you've installed the software, run your

programs like always.  The only difference

you'll notice is the increased speed.  Enjoy!

7-10       INSTALLING AND RUNNING THE INBOARDTM 386/PC

SOFTWARE