How to use the Marvel II encoder board under Windows 95

1. 	To switch from PAL to NTSC  resolution (or vice versa) for the encoded out 	signal, edit your config.sys file.

The line to be modified is :

DEVICE=C:\...\MRV2DRV.SYS NTSC

Just change the parameter to NTSC or PAL, according to the desired resolution. 	You can edit your config.sys file through the Sysedit application available under 	Windows 95  (To make the changes effective, you have to reboot).

2.	If you have chosen to use the encoder in NTSC resolution, you have to adjust the 	refresh rate of your VGA screen to 60 Hz.  To edit your Win95 registries, go 	into Regedit.  Under the following path :

HKEY_LOCAL_MACHINE\System\CurrentControlSet\Services\Class\Display

you will find the Marvel II driver, i.e. "Tseng Labs Matrox ET4000/W32 PCI".

Go into Default for that driver and doubleclick on RefreshRate.  Then, type in 60.

3.	In NTSC resolution, in the Display Properties Settings, you have to set the 	resolution to 640x480.  In PAL, 768x576.

4.	Finally, execute the Encon.exe program which works in both resolutions.  To turn 	off the encoder, use Encstop.exe.  You can find those two programs in 	encoder.zip, which can download directly from the BBS, from the marvel2 	folder.