July 13, 1987

Sparta PCBoard

Hard Drive Conference

(201) 729-7056

Sparta  PCBoard recently began using and selling an advanced RLL  Hard

Disk Controller made by a company caller Perstor Data Systems, Inc. We

are offering this controller at substantial savings over the suggested

retail  price,  along with telephone support,  and our usual 24 hour a

day support through the Sparta PCBoard Hard Drive Conference. Below is

a  description of the controllers,  along with a list of  drives  that

have  been certified by the manufacturer to work with the  controller,

and,  lastly,  a  list  of  those drives known not to  work  with  the

controller.

Perstor   Data   Systems,   Inc.,   currently  offers  one   line   of

controllers...  the  Perstor 200 Series.  In this line of  controllers

there  are two models,  the PS180 and PS200.  Both models utilize  the

Advanced Run Length Limited 2,7 encoding method.

The  Perstor  PS180 Controller has a data transfer rate of 9  megabits

per second to increase hard disk capacity by at least 90% and provides

format  of  31  sectors per track.  The PS200 Controller  has  a  data

transfer rate of 10 megabits per second to increase hard disk capacity

by at least 100% and provides 34 sectors per track.  Both  controllers

are  ROM residence selectable and port address switchable to allow you

to have more than one BIOS ROM controller.  In addition, the PS180 and

PS200  are multi-layer controllers,  reducing electronic noise in  the

circuitry.

The  Perstor 200 Series Controllers will work with virtually any  high

quality Winchester disk drive compatible with the IEEE  specifications

for   the  ST506/412  interface.   Perstor  has  conducted  successful

qualification  tests on most of the popular fixed disk drives  in  the

market today,  including Miniscribe,  Seagate,  Maxstor, Newbury Data,

CDC,  LaPine,  Rodime,  and  other  models (listed at the end of  this

article).

The  PS180 Controller works with MFM and RLL approved  drives,  plated

and oxide media, and the PS200 Controller works best with RLL approved

plated media drives,  whether installed in a PC,  XT,  or AT or any of

their  compatibles.   Both controllers use a IBM compatible  BIOS,  so

they are compatible with MSDOS,  PCDOS,  UNIX, XENIX, and CPM, as well

as Novell Netware.

Both  the PS180 and PS200 Controller are capable of supporting  drives

with  15 heads or less and 2048 cylinders or less.  They come with  an

Advanced Error Checking and Correction capability using a 56 bit  code

to  allow  twice  the bit dropout factor  of  normal  controllers.  In

addition  to  this  56 bit ECC,  both models have a  25  megabit  data

recovery  circuit,  and on both controllers is a sector buffering  RAM

chip   to  overcome  speed  differences  between  the  drive  and  the

controller.

On  both  models,  you have your choice of mounting hard  disk  drives

internally or externally,  and using the controller as a primary,  co-

resident, or multiple controller. Plus, you have the option of setting

up the controller for BIOS or non-BIOS use, and can have either Direct

Memory Access or programmed input/output.

DiskExpander,  a compaction software program packaged with the Perstor

Controllers  as  a no-cost option,  provides another 50%  increase  in

capacity above the doubling provided by the  controller.  DiskExpander

is  enhanced  by a cache access method that lowers the average  access

time  to  about 15ms when  used.  However,  this  compaction  software

program needs a DOS 3.1 or higher environment.

Approved Drive List

The following drives have been successfully tested by the manufacturer

with the Perstor 200 series controllers.

Miniscribe    Seagate      Maxstor   Tandon    Rodime

MS3212	      ST225        XT1065    TM362     3055

MS3425        ST225-003B   XT1085              352

MS3438        ST238        XT1140

MS6032        ST251

MS6053        ST4038       Newbury   CDC       Lapine

MS6085        ST4096

MS8425                     1140      WREN 2    Titan 30

MS8435

Additionally,  I  have personally tested and helped format the Fujitsu

72MB drive,  the Maxstor XT2150,  and several CDC Wren I  drives.  The

Maxstor XT2190 Drive has also been successfully formatted and operated

on a network under heavy usage and has performed flawlessly.

Some of the experiences so far have been:

1)  The  Seagage  4096  and Maxstor XT1085 are being  used  on  Sparta

PCBoard  24  hours a day.  MACE and Norton's DT have been run  on  the

drives every single day for the past month and neither have picked  up

a  single  soft  sector or hard sector error that was  not  picked  up

initially by Disk Manager or Vfeature.

2)  Chuck  Ammann  is  running  a Maxstor  XT2140  that  is  currently

formatted on a PS200 10mbits per second controller and he was able  to

get  196MB on on 1024 cylinders out of a possible 1224 cylinders  that

the  drive has.  We had a chore formatting his drive until we got  the

right ROM BIOS table burned and installed.

3) Other Sysops running the Perstor Controller

Paul Kopit - Software Society

Neil Leder - Doctor's Office BBS

Paul Waldinger - Sound of Music BBS

Gary Thurlow - Wilmington Delaware BBS

Lee Windsor - Hounds Haven BBS

Eric Green - ELCO Base BBS

Miles Lester - Xignals PCBoard - Auburn Alabama

4) Our experiences so far have been from almost flawless  installation

to  hair-pulling teeth-knashing sessions that took several hours.  Why

the  differences?  Beats the heck out of me.  We have the  controllers

installed  in a mixed bag of machines such as  12mhz  Clone,  Olivetti

M28,  AST Research Premium 286,  Multitech Accel 900,  IBM PC-XT and a

stock IBM AT machine.

5) Problems reported so far..

1.. Does not work real great with SpeedStor prior to 4.02 since

Speedstor up to this version would not support 34 sectors per

track needed by the PS200. If you do use the PS200, you cannot

use an even interleave such as 2, 4, 6, or 17, though the

controller supports interleave all the way up to 30.

2..  Appears to hangs for unexplainable reason during a low level

format using Vfeature Deluxe. Though you can then start the

process over and usually will go completely through the second

time.

3..  Will not work with some Seagate ST4038. Will find out what

revision.

4..  Will not work at all with a CMI Drive. But then what does work

well with this drive?

5..  Will not work with older Priam Drives. Though seems to work AOK

with current production run of the Priam. Priam does a test to

track 0 after receiving a positive indication that it has done

a seek to track 0. Current software does not support the extra

test. Will do so in the future.

6..  Limited size BIOS Tables. Though there are only four drive type

types in the current BIOS, with the next production run, the

BIOS will support 16 drive types in each table and the two

drives on a card can be of two different types.

7..  Does not currently have any way of hooking up the activity

indicator light on AT type machines. I have drawn up a circuit

design that will allow your controller to be modified if you

need an activity indicator light on the front of your AT.

What  is  the ideal interleave to use?  That is a  tough  question  to

answer and depends a lot on your mix of drive, speed of machine, speed

of RAM,  etc. I have devised a very good method of only formatting the

first  100  tracks  of  any drive until I have tested  it  at  several

different  interleaves.  Only after I have found the right  interleave

for  my  hardware  combination do I format and  partition  the  entire

drive.

How does this controller co-exist with the floppy/hd controller in  an

AT? Good question. It is simply a matter of running your setup program

and telling the setup that you do not have any HD installed.  Plug the

Perstor in, hook up the hard drive and let er rip.

Perstor  plans  in  the near future to have  a  combination  floppy/HD

controller  that will give the same blazing fast performance as  their

current 8 bit line of controllers. Should you wait til then? That is a

decision  only you can make.  Are you running out of room?  Need  some

cheap  additional storage in a hurry at a very reasonable  price.  The

Perstor Series 200 is the way to do it.

What  32MB barrier software has been tested on it.  We have used Speed

Stor,  Vfeature,  and Disk Manager.  Favorites at this point? Vfeature

seems  to the the favorite.  Disk Manager version 3.01 works  superbly

with the disk controller also.  That is what I am currently running at

Sparta  PCBoard.  Though I own and have tried all three of  the  above

mentioned packages.

Pricing!

Suggested Retail   My price

Perstor 180 - 9 Mbits Per second         $395.00            $295.00

Perstor 200 - 10 Mbits Per second        $495.00            $395.00

Disk Manager 3.01 (or current version)    $99.00             $60.00

Telephone Support (corporate)             $50.00 Per Hour    $50.00

Telephone Support (individual)            $50.00 Per Hour    $Free

Bulletin Board Support                    $Free              $Free

You pay the phone bill is all that is required. Shipping will be at

your direction, surface UPS or overnight. Fedex Saturday Delivery if

you have an account with Federal Express. You pay the shipping.

Need Special ROM burned to fit your particular drive combination?

Let me know. I can burn an eprom with whatever table you need in

the BIOS ROM.

Method of payment?

Since  I don't do this for a living and am just trying to help  people

get some more storage at a cheap price,  I am only looking at a modest

profit on this deal.  Therefore,  I need the money up front to keep my

cash  flow  going.  Money Order or certified funds preferred unless  I

know you and have done business in the past.  Hey,  at these prices, I

cannot afford funny money, right?

Address where I can be reached

Richard Driggers

47 Conestoga Trail

Sparta, NJ 07871

Phone Number where I can be reached

(201) 729-9409 Home

(201) 729-7056 BBS

Thanks in advance.#######################################################################