Tropez Plus Multimedia Tests / Troubleshooting

This document contains several embedded multimedia files as well as links to common Windows 95 utilities that will help you test the output of the Tropez Plus.  Double-click on an item to activate it.  You will need to have your speakers turned on and plugged into the output jack of the Tropez Plus to hear the demonstrations.

____________________________________________________________________

Digital Audio

Double-Click on the icon below to test the Tropez Plus' Digital Audio playback:

Wavetable Synthesis

Double-Click on the icon below to test the Tropez Plus' MIDI playback:

_____________________________________________________________________

Didn't Hear Anything?

In case you didn't hear one or both of the two sounds or you received an error message, you can follow the troubleshooting steps below to enable the Tropez Plus.  More in-depth troubleshooting is available in the Tropez Plus manual.

Check here first to see if your mixer levels are set properely.  The Wave fader controls Digital Audio volume, the Synth fader controls the Wavetable Synthesizer volume, and the Master fader controls the overall volume.  If you seem to be "missing" certain faders, click on 'Options' then 'Properties' in the mixer and place a check mark next to all of the boxes in the lower portion of the Window:

Still nothing?

Check here to see if you have the Tropez Plus chosen as the default Audio and MIDI device.

The Tropez Plus should be chosen for both Playback and Recording under the Audio tab of Multimedia Properties.  The Tropez Plus Sample Player should be selected under the MIDI tab if you want to hear the onboard wavetable synth.  If you heard the FM Synthesizer when you played the MIDI file, Tropez Plus FM Synthesis was probably selected. Make sure you click on OK to save any changes.

Couldn't open the mixer or nothing listed under Multimedia Properties?

Check here to see if Windows 95 is reporting any errors with the Tropez Plus.

Look under the 'Sound, video and game controllers' key (you may need to double-click on it).  You should have at least five devices installed.

- Gameport Joystick

- MPU-401 Compatible

- Tropez Plus PnP Audio Hardware

- Tropez Plus PnP Control Registers

- Tropez Plus Wavetable MIDI Synthesizer

If you do not have all five devices installed, the card will not function properly.  Follow the steps at the end of this document for uninstalling the Tropez Plus, and try the installation again.  If you do have all of the devices installed and one or the other has an exclamation point next to the device, you have a system conflict of some kind.

Common Conflicts

The two most common conflicts for the Tropez Plus are a Port Address or an interrupt conflict.

Port Address / Interrupt Conflict:

Click on the 'System Properties' icon below to open up the Device Manager again.   Go to 'Sound, video and game controllers' and identify the device that is not working properly.  Non-functioning devices will have a small, yellow circle with an exclamation point next to them.

The five devices and their default settings are listed below.  Since Plug-and-Play has configured your card, it has chosen settings that are not supposed to be conflicting with other devices in the system.  Obviously, Plug-and-Play is not always perfect and sometimes needs some extra coaxing.  If you need to change a Port Address, IRQ, or DMA that PnP has allocated to the card, you may need to remove the check next to 'Use automatic settings' and change the 'Settings based on' box to Basic configuration 0001 or 0002  depending on the device.

TROPEZ PLUS PNP AUDIO HARDWARE:

Direct Memory Access	1			*Playback DMA

Direct Memory Access	0			*Record DMA

Interrupt Request		5			*WSS/SB compatible IRQ

Input/Output Range		0534-0537		*WSS Port Address

Input/Output Range		0388-038B		*FM Synth Address (Don't Change!)

Input/Output Range		0220-022F		*SB-compatible Address

TROPEZ PLUS WAVETABLE MIDI SYNTHESIZER:

Interrupt Request		9			*Wavetable/MIDI IRQ

Input/Output Range		0330-033F		*Wavetable/MIDI Port Address

MPU-401 COMPATIBLE:

Input/Output Range		0350-0351		*Secondary MPU-401 Port Address

(Interrupt Request		5			*Secondary MPU-401 IRQ)

Note:	The Interrupt request is disabled for this device by default.  You should

only enable it if you plan to attach a second MIDI adapter to the Tropez

Plus via an optional 15-pin connector.   You can attach MIDI instruments

to the Tropez Plus without this connector by using the Primary MIDI port

which is controlled through the Wavetable MIDI Synthesizer.

TROPEZ PLUS PnP CONTROL REGISTERS:

Input/Output Range		0100-0107		*PnP Control Address

GAMEPORT JOYSTICK:

Input/Output Range		0200-0207		*Gameport Port Address

Hey!  There are too many settings...which one do I change?!?

Relax...since the most common resource conflicts are caused by an interrupt that is being used by another device that Windows 95 is not aware of, that is generally the best place to start.  When changing any resource values (IRQ, Port, or DMA) in a device, it is generally a good idea to change only one resource at a time.  When you find the right combination of resources for a device,  you may not even have to restart the computer.  Windows 95 can usually allocate the resources "on the fly" and your device will become active.  To fully test the changes, however, a complete shut down is recommended.

Also, when changing one of the values, try to keep the rest of the resources as close to the defaults as possible.

Un-installing / Re-installing the Tropez Plus

If you have tried several of the suggestions above, and the Tropez Plus is still not working,  you will need to remove the current Tropez Plus drivers.  To assist you with this, we have provided an un-install procedure that will remove all Tropez Plus drivers and support files from your system.

Warning:  Although this procedure will specifically remove all Tropez Plus drivers, it may remove all or part of other soundcard drivers installed in Windows 95.  If you have multiple soundcards in Windows 95, you may need to manually remove the Tropez Plus drivers from the Device Manager and not use this procedure.

Before double-clicking on either of the icons below, insert your disk labeled "Tropez Plus Windows 95 Drivers" into your Floppy drive.

You will be taken directly to the CLEANTBS directory on your Windows 95 Drivers disk.  Using your mouse, RIGHT click on the file, CLEANTBS.INF.  A menu will appear with several choices, select "Install" to begin the un-installation.  The floppy drive will be accessed for several seconds while the Tropez Plus drivers are removed.  You can then close all programs and shut down the system.  When you reboot the system, the Tropez Plus will be re-dected by Windows 95 PnP.

Back to the top and try again!